3GPP TSG-WG SA2 Meeting #157	S2-2307890

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-2307843)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0808 | rev | 4 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Procedures for Federated Learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are a few ENs about Nnwdaf_MLModelTrainingInfo to be resolved. To resolve the ENs, definition of Nnwdaf_MLModelTrainingInfo services should be added, and related procedures should be updated accordingly. As described in step 4 of clause 6.2C.2.3, the FL Server NWDAF sends termination request by invoking Nnwdaf_MLModelTrainingInfo_Request service operation, and optionally indicating the reason, e.g. FL Client NWDAF is unselected by the FL Server NWDAF for the FL process, or the FL process is suspended, etc. Therefore, the “Correlation Termination Flag” is changed to “Termination Request”, and the reason is added as the optional information to the input of the Nnwdaf_MLModelTrainingInfo_Request service operation. The Nnwdaf_MLModelTrainingInfo_Response may include the local ML model metric and Training Input Data Information as described in step 4 of clause 6.2C.2.2, so the Status report of FL training that includes these two parameters should be added to the output of the Nnwdaf_MLModelTrainingInfo_Request service operation. It’s unclear what parameter should be included in the Nnwdaf_MLModelTrainingInfo_Response when the FL Client NWDAF is not able to complete the training of the interim local ML model within the maximum response time provided by the FL Server NWDAF. It proposes that the FL Client NWDAF shall send the Delay Event Notification to the FL Server NWDAF. The Delay Event Notification includes a delay event indication that indicates the FL Client NWDAF is not able to complete the training of the interim local ML model within the maximum response time, an optional cause code (e.g. local ML model training failure, more time necessary for local ML model training), and the expected time to complete the training if available. As described in step 6a of clause 6.2C.2.2, the FL Server NWDAF may send a Nnwdaf_MLModelTrainingInfo_Response message to update the global ML model metric to the consumer. Therefore, the global ML model metric should be added to the output of the Nnwdaf_MLModelTrainingInfo_Request service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add procedure of Nnwdaf_MLModelTrainingInfo service. - Add definition of Nnwdaf_MLModelTrainingInfo service. Change the “Correlation Termination Flag” to “Termination Request”, and add the reason for the Termination Request to the input of the Nnwdaf_MLModelTrainingInfo_Request service operation. Add the Status report of FL training that includes the local ML model metric and Training Input Data Information to the output of the Nnwdaf_MLModelTrainingInfo_Request service operation. Add the Delay Event Notification that includes a delay event indication, an optional cause code and the optional expected time to complete the training to the output of the Nnwdaf_MLModelTrainingInfo_Request service operation. Add the global ML model metric to the output of the Nnwdaf_MLModelTrainingInfo_Request service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are unresolved ENs about Nnwdaf_MLModelTrainingInfo. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2F.x(new), 7.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2F.x	ML Model Training Information Request

The procedure in Figure 6.2F.x-1 is used by an NWDAF service consumer, i.e., an NWDAF containing MTLF to request another NWDAF, i.e., an NWDAF containing MTLF, for the information about ML model training based on the ML model provided by the service consumer.  The service may be used by an NWDAF containing MTLF to enable e.g. Federated Learning.

NWDAF

Service Consumer

NWDAF

containing MTLF

2. Get the information about the model training base on a model provided by the service consumer

1. Nnwdaf_MLModelTrainingInfo_Request

3. Nnwdaf_MLModelTrainingInfo_Request response

Figure 6.2F.x-1: Procedure for ML Model Training Information Request

1.	The NWDAF service consumer may request the NWDAF containing MTLF to get the information about the ML model training based on the ML model provided by the service consumer by invoking the Nnwdaf_MLModelTrainingInfo_Request service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2F.2.

In order to enable Federated Learning, NWDAF Service consumer acting as FL Server NWDAF requests to get ML Model Training Information from an NWDAF containing MTLF acting as FL Client NWDAF, which is selected by the FL Server NWDAF. The details are specified in clause 6.2C.

The NWDAF service consumer may use the request to check if an NWDAF can meet the ML model training requirements (e.g. Interoperability information, Analytics ID, Service Area/DNAI, and/or availability of data and time). In such cases, the NWDAF service consumer includes an ML Preparation Flag.

The NWDAF service consumer may use the request to get the Model Accuracy of the ML Model provided by the service consumer using local training data in the NWDAF containing MTLF as the testing dataset. In such cases, the service consumer NWDAF includes a Model Accuracy Check Flag.

2.	When the ML Preparation Flag is present in the request, the NWDAF containing MTLF only checks whether it can meet the ML model training requirement and/or can successfully download the model if the model information is provided. Based on the check result, the NWDAF containing MTLF gets a successful return code or failure cause code (e.g. NWDAF does not meet the ML training requirements) as the information about the ML model training.

When the Model Accuracy Check Flag is present in the request, the NWDAF containing MTLF uses the local training data as the testing dataset to calculate the Model Accuracy of the ML model provided by the service consumer NWDAF. The NWDAF containing MTLF includes the Model Accuracy into the information about the ML model training.

When the NWDAF containing MTLF is ongoing ML model training based on the ML model provided by the service consumer, the NWDAF containing MTLF gets a failure cause code (e.g. ML training is not complete) as the information about the ML model training.

When the NWDAF containing MTLF completes ML model training based on the ML model provided by the service consumer, the NWDAF containing MTLF gets a successful return code and the ML Model Information of the trained ML model as the information about the ML model training.

3.	The NWDAF containing MTLF replies to the NWDAF service consumer with the information about the ML model training by invoking the Nnwdaf_MLModelTrainingInfo_Request response service operation.

# * * * * Second change * * * *

7.X	Nnwdaf_MLModelTrainingInfo Service

7.X.1	General

Service Description: This service enables the consumer to request for the information about ML model training based on the ML Model provided by the consumer.

NOTE:	In this release of the specification, the service provider and consumer are limited to NWDAF containing MTLF.

When used for Federated Learning, this service enables FL server NWDAF to enable Federated Learning while providing global ML model information to FL Client NWDAF and getting local ML model information from the FL Client NWDAF.

7.X.2	Nnwdaf_MLModelTrainingInfo_Request service operation

Service operation name: Nnwdaf_MLModelTrainingInfo_Request.

Description: Request information about NWDAF ML model training with specific parameters.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	Interoperability information;

-	ML Model ID: identifies the provided ML model.

Inputs, Optional:

-	ML Model Information (i.e, file address (e.g., URL or FQDN) of ML Model that needs to update);

-	ML Training Information (i.e. data availability requirement, time availability requirement).

-	ML Preparation Flag;

-	ML Model Accuracy Check Flag;

-	ML Correlation ID;

-	Termination Request, when terminating the Federated Learning identified by the ML Correlation ID, and optionally indicating the reason, e.g. FL Client NWDAF is unselected by the FL Server NWDAF for the FL process, or the FL process is suspended, etc;

-	Training Filter Information;

-	Use case context.

Outputs Required: When the request is accepted: Operation execution result indication. When the request is not accepted, an error response with cause code (e.g., NWDAF does not meet the ML training requirements, ML training is not complete, NWDAF overload, not available for the FL process anymore, etc.).

NOTE: The detail reasons in the cause code are up to stage 3.

Outputs, Optional:

-	ML Model ID.

- Set of the tuple (Analytics ID, ML model Information (i.e., file address (e.g., URL or FQDN) of updated ML Model).

- ML Correlation ID, when for Federated Learning;

-	Corresponding Use case context;

-	Status report of FL training: local ML model metric and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension of data, etc.), which are generated by the FL Client NWDAF during FL procedure;

-	Delay Event Notification with the following parameters:

-	delay event indication: this parameter indicates that the FL Client NWDAF is not able to complete the training of the interim local ML model within the maximum response time provided by the FL Server NWDAF.

-	[OPTIONAL] cause code (e.g. local ML model training failure, more time necessary for local ML model training, etc.).

-	[OPTIONAL] the expected time to complete the training.

-	global ML model metric.

# * * * * End of changes * * * *