SA WG2 Meeting #157 	S2-2308040

# Berlin, Germany, May 22nd – 26th, 2023		(revision of S2-2306674)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0146 | rev | 1 | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common EAS/DNAI determination for a set of UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson, NTT DOCOMO, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Edge_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - There are several FFS in 6.2.3.2.7 related to Coordination among SMFs and creating/updating/deleting set of UEs data determined by 5GC i.e. Common EAS and common DNAI determination. This CR resolves those FFS - The procedure in 6.2.3.2.7 related to Synchronization of Common EAS/DNAI for SMFs has concerns, for example it requires that SMFs updates to UDR via respective PCFs, however, this may not work for HR-SBO cases where SMF does not interface directly with PCF and to receive AF traffic influence data subscribes directly to NEF using TrafficInfluenceData _Subscribe. - Current procedure in 6.2.3.2.7 does not work well e.g. due to UDR locking the data, unclear which SMF allowed to updating data, etc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates 6.2.3.2.7 to provide details on determination of common EAS and/or DNAI, coordination among SMFs serving the set of UEs and storing information for a set of UEs in UDR together with AF traffic influence data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No clear solution to common EAS/DNAI determination and storing information for a set of UEs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.6, 6.2.3.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 23.502, CR 4160 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.2.3.2.6	EAS discovery corresponding to Common DNAI for a set of UEs

The common DNAI for the set of UEs can be provided either by AF or determined by 5GC. When the AF determines the common DNAI, the AF provides the common DNAI for the set of UEs via AF Traffic influence procedure as defined in clause 4.3.6.2 of TS 23.502 [3]. The AF may determine the common DNAI based on candidate DNAI(s) reported by SMF as described in clause 4.3.6.3 of TS 23.502 [3]. 

When the 5GC determines  the common DNAI, the common DNAI is determined by NEF and the NEF stores the common DNAI in UDR as part of AF traffic influence request information, as described in clause 6.2.3.2.7.

The following is the procedure for discovery EAS corresponding to a Common DNAI for set of UEs accessing the same application.

Figure 6.2.3.2.6-1: EAS discovery corresponding to Common DNAI for a set of UEs

1.	The AF request in step 1 of figure 4.3.6.2-1 in TS 23.502 [3] is used to request selecting the common DNAI for a set of UEs accessing the application as identified in the AF Request.

AF may use External/Internal Group ID(s) or a list of UEs or any UE as Target UE Identifier(s) and additionally Spatial Validity Condition to identify the set of UEs for correlated selection of common DNAI.

The following information may be included in AF request as defined in clause 5.6.7.1 in TS 23.501 [2]:

-	An indication of traffic correlation may be provided for indication of selecting the same DNAI (i.e. selecting EAS corresponding to the same DNAI) for the set of UEs accessing the same application.

-	A Traffic Correlation ID may be provided for identification of the set of UEs accessing the application identified by the Traffic Description in AF request.

-	A Common DNAI to be accessed by the set of UEs can be included in AF request, if it is determined by AF.

-	FQDN(s) may be included which is corresponding to the application identified by Traffic Description in AF request.

-	Spatial Validity Condition could be provided for limiting the location of the UEs, and also "any UE" or a UE list or group ID can be provided for defining set of UEs accessing the same DNAI.

In step 3a of figure 4.3.6.2-1 in TS 23.502 [3], NEF updates the AF influence data related to the traffic correlation ID in the UDR with a Notification Endpoint to subscribe to be notified with information related to SMF’s involvement for UE members of the set of UEs.

In step 5 of figure 4.3.6.2-1 in TS 23.502 [3], PCF determines the UEs influenced by the AF Request, and for each UE, based on AF request, PCF creates PCC rule with Traffic Correlation ID and indication of traffic correlation, Common DNAI, FQDN(s) and Notification endpoint of NEF subscription received to step 3a and sends the PCC rule to the SMF.

2.	Based on steps 1~19 in figure 6.2.3.2.2-1, with the following changes:

In step 9:

If FQDN in Neasdf_DNSContext_Notify Request is corresponding to the application indicated in PCC rule, e.g. the FQDN is included in the FQDN(s) in the PCC rule and if indication of traffic correlation is set, SMF determines the UE belongs to set of UEs identified by Traffic Correlation ID and accessing the application and determines the UE connects to EAS corresponding to the common DNAI for the set of UEs. If FQDN(s) is included in PCC rule, the SMF can use the FQDN(s) in PCC rule and the FQDN in Neasdf_DNSContext_Notify Request to match the FQDN with the PCC rule, i.e. the matched PCC rule includes the FQDN(s) containing the FQDN.

If the common DNAI is not available in the PCC Rule received in step 1, SMF invokes the NEF to determine the common DNAI as described in clause 6.2.3.2.7.
	Once the common DNAI is available, for Option A, SMF provisions EASDF with the information to build EDNS Client Subnet option that refers to a location that is topologically close to the common DNAI; for Option B, SMF provisions EASDF with Local DNS server related to the common DNAI.

# * * * * First change * * * *

##### 6.2.3.2.7	Coordination among SMFs for Common EAS/DNAI determination

Figure 6.2.3.2.7-1: Handling of Common EAS, CommonDNAI for set of UEs

1. SMF sends Nsmf_TrafficCorrelation_Notify to the NEF with Notification Endpoint received in the PCC rule as described in clause 6.2.3.2.5 and 6.2.3.2.6, and provides: EAS IP address based on EASDF procedure, list of candidate DNAI(s), SMF ID, number of PDU sessions it is serving for the set of UEs.

2. If the NEF determines that there is currently no common EAS IP address and/or common DNAI available for this set of UEs identified by Traffic Correlation ID, it selects a common DNAI and/or common EAS using the list of DNAI(s) and EAS IP address(es) received in step 1. Then the NEF updates traffic influence data with the 5GC determined common EAS/DNAI for the set of UEs.

The updated traffic influence data triggers notifications to PCF(s) that in turn trigger associated PCC rule updates to the concerned SMF(s), if any, with PDU Session(s) associated with the traffic correlation ID.

The NEF maintains list of SMFs serving the set of UEs and the associated data received in step 1.

3. NEF responds by acknowledging to the SMF.

4. The update in UDR triggers notification to the PCFs that have subscribed for notification. The PCF(s) sends PCC rule(s) with common EAS, Common DNAI and Traffic correlation ID, as part of traffic influence data to the SMF(s) having the PDU Session(s) associated with the Traffic Correlation ID.

SMF(s) may select other candidate DNAI(s) for the PDU session(s) or a new EAS IP address via the EASDF procedure e.g. due to UE(s) mobility. In this case, the SMF notifies to the NEF as in the above step 1, with the list of candidate DNAI(s) and/or EAS IP address. This may trigger NEF to re-select common DNAI or common EAS. NEF determines common EAS and/or common DNAI based on received EAS IP, list of candidate DNAI(s), number of PDU sessions SMF(s) is serving for the set of UEs.

If another DNAI/EAS IP address is selected by the NEF, it updates the common DNAI or common EAS in the UDR in the Traffic Influence data.

NOTE: How NEF determines a common EAS/DNAI is implementation.

# * * * * End of changes * * * *