3GPP TSG-WG SA2 Meeting #157	S2-2308076

# Berlin, Germany, May 22 – 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0826 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

X

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PFD determination Analytics – Confidence level |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 12-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The NWDAF collects input data from UPF using UserDataUsageMeasures that reports per Application Id, the volume and some Application data to help NWDAF to derive PFDs for known Applications, i.e. Application related Information: URL/s and/or Domain name/s detected in the PDU Session for the target traffic as defined in 23.502 clause 5.2.26.2.1 on Nupf_EventExposure service. To derive new PFDs, the NWDAF needs both to collect input data that is matching the Application ID, to apply the algorithms to derive new PFDs for known Application ID. The Application ID (e.g. video) may have a wider scope in the UPF so that it allows NWDAF to retrieve some input data classified at the UPF that can be refined at the NWDAF (e.g. a specific video application). This newly derived PFDs have different confidence level, depending on the amount of input data retrieved from UPF, or the volume reported or other algorithms that NWDAF uses, The NWDAF provides a level of accuracy to the PFDs that are reported to the consumer so that the consumer can decide where to use them or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The NWDAF may assign a diferent level of accuracy to the PFDs. The NWDAF reports to the consumer PFD determination analytics that reaches the preferred level of accuracy. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consumer may use PFD determination Analytics with a low level of accuracy, without knowing that. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.16.1, 6.16.2, 6.16.3, 6.16.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# ********************* First Change *********************

## 6.16	PFD Determination Analytics

### 6.16.1	General

This clause specifies the procedure on how NWDAF can provide NWDAF-assisted PFD Determination analytics, in the form of statistics.

To assist determination of PFDs for known application identifiers, if the related Service Level Agreement does not preclude, an NWDAF may perform data analytics on existing PFD information and user plane traffic and provide analytics results in the form of new or updated PFDs, e.g. IP 3-tuple list in PFD is new or updated, to an analytics consumer in the 5GC. The NEF(PFDF) as the consumer may forward new or updated PFD information provided by the NWDAF to the UPF via the SMF to detect a known application, as defined in TS 23.502 [3].

The service consumer may be a NEF(PFDF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = " PFD Determination";

-	Target of Analytics Reporting: "any UE";

-	Application identifier;

-	Analytics Filter Information optionally containing:

-	S-NSSAI;

-	DNN;

-	An Analytics target period indicates the time period over which the statistics are requested;

-	Optionally, the preferred level of accuracy on the PFD statistics;

-	Optionally, maximum number of objects. This refers to the analytic output (i.e. maximum number of new or updated PFDs);

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

# ********************* Next Changes *********************

### 6.16.2	Input Data

The NWDAF collects information on user data traffic from NF(s) for a specific S-NSSAI, DNN, Application ID, and retrieves the existing PFDs from the NEF(PFDF). The detailed input data are described in Table 6.16.2-1.

Table 6.16.2-1: Input data to detect known application from NFs

| Information | Source | Description |
| --- | --- | --- |
| Application ID (NOTE x) | UPF | Identifies the application |
| IP Flow Information (1..max) | UPF | Per IP flow related information for the application |
| > IP 5-tuple | UPF | Identifies a service flow of the UE that uses the application. |
| > Start time | UPF | Start time of traffic detection for the flow. |
| > End time | UPF | End time of traffic detection for the flow. |
| > UL Data volume | UPF | Measured UL data traffic volume for the flow. |
| > DL Data volume | UPF | Measured DL data traffic volume for the flow. |
| > UL Data Rate | UPF | Measured UL data rate for the flow. |
| > DL Data Rate | UPF | Measured DL data rate for the flow. |
| > URL list | UPF | List of URLs extracted from the inspected user plane packets in the flow. |
| > Domain Name list | UPF | List of domain names extracted from the inspected user plane packets in the flow. |
| PFD Information | NEF(PFDF) | PFD Information stored in the UDR (as Application Data) and retrieved by NEF (PFDF), as defined in clause 6.1.2.3.2 of TS 23.503 [4]. |
| > Application ID | NEF(PFDF) | Identification of the application that refers to one or more application detection filters. |
| > IP 3-tuple list | NEF(PFDF) | Including protocol, server side IP address and port number. |
| > URL list | NEF(PFDF) | The significant parts of the URL to be matched, e.g. host name. |
| > Domain Name list | NEF(PFDF) | A Domain Name matching criteria and information about applicable protocol(s). |
| NOTE x: The Application ID maps in the UPF to a detection algorithm that has a wider scope than the Application ID provided by the consumer. |  |  |

NOTE:	Extensive reporting of all traffic flows may conflict with the requirement to avoid extra UPF load. An NWDAF may subscribe only for reporting for some UEs to limit the load.

# ********************* Next Changes *********************

### 6.16.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.16.3-1. The output analysis may be used to provision new or updated PFDs information for known applications. The NWDAF may assign a confidence to the PFD Determination statistics based on the input data provided by UPF.

Table 6.16.3-1: PFD Determination statistics

| Information | Description |
| --- | --- |
| Application ID | Application ID of the application that refers to the application detection filter (Flow descriptor, URL or Domain name information) stored in the UDR. |
| List of application traffic description |  |
| > S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| > DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| > Flow descriptor | Flow descriptor containing 3-tuple server side IP address, server port number, and protocol of uplink/downlink application traffic as described in TS 23.503 [4]). |
| > URL list | the significant parts of the URL to be matched, e.g. host name. |
| > Domain name information | A Domain name matching criteria and information about applicable protocol(s). |
| Confidence | Confidence on the provided application traffic description for the Application ID. |

# ********************* Next Changes *********************

### 6.16.4	Procedures

Figure 6.16.4-1 shows the procedure that a consumer can use to request PFD Determination analytics.

Figure 6.16.4-1: A procedure for NWDAF-assisted PFD Determination

1.	The Consumer NF (NEF(PFDF)) requests or subscribes to the NWDAF to request PFD Determination analytics for a known application identifier. The Target of Analytics Reporting is set to Any UE.

The Analytics Filter Information may optionally include the S-NSSAI and/or DNN.

2.	The NWDAF fetches currently stored PFD information in use from UDR via NEF(PFDF) for the Application ID provided in step 1, as listed in Table 6.16.2-1.

3.	The NWDAF collects information from the UPF (event "UserDataUsageMeasures") as listed in Table 6.16.2-1 either via SMF (Option 1) or directly from UPF (Option 2) as defined in clause 5.8.2.17 of TS 23.501 [2]. NWDAF discovers the SMF (Option 1) or UPF (Option 2) through NRF as defined in clause 4.15.4.5.3 of TS 23.502 [3]. The event "UserDataUsageMeasures" is defined in clause 5.2.26.2 of TS 23.502 [3], the subscription to the event may include an Application ID set to a value that allows the NWDAF to retrieve input information to derive PFD Determination Analytics for the Application ID that is provided by the consumer.

NOTE: The Application ID provided by the consumer and the Application ID provided to the UPF to retrieve input data for PFD Determination Analytics can be different ones.

4.	UPF reports the data directly to NWDAF.

5.	The NWDAF derives PFD Determination analytics, e.g. new or updated PFD information for the existing Application ID. When the consumer provides the preferred level of accuracy, the NWDAF provides PFD Determination Analytics that reaches this level to the consumer.

6.	In the case that PFD information for the existing Application ID is new or to be updated, the NWDAF notifies PFD Determination analytics to the consumer NF (i.e. NEF(PFDF)) with PFD Information.

********************* End of Changes *********************