3GPP TSG-WG SA2 Meeting #158	S2-2308676

# Gothenburg, Sweden, 21-25 August 2023		(revision of S2-2304xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4767 | rev | - | Current version: | 18.2.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Meaning of availability for 3GPP access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The (UE generated) PMF reports are used by the UPF in ATSSS to understand whether a certain access of the MA PDU Session is available or unvailable. It is also described that the UE reports availability/unavailability of accesses based on implementation. While for a non-3GPP access the meaning of available is clear (since there is no concept of UE in CM-idle mode), for a 3GPP acces it is unclear if it is available only when the UE is in CM-connected mode or also when the UE is in CM-idle mode. This could lead to interoperability issues and/or unnecessary signaling (due to paging) in scenarios in which the UE is in CM-idle mode over the 3GPP access but still reports that the 3GPP access is available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that for a 3GPP access the PMF reports it is available if the UE is in CM-connected mode over that access, while it reports the access is unavailable if the UE is in CM-idle mode over that access. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification which could lead to interoperability issues and unnecessary signaling. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.32.6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 5.32.5.3	Access Availability/Unavailability Report

If required by the network in the Measurement Assistance Information, the UE shall provide access availability/unavailability reports to the network. How the UE detects the unavailability and the availability of an access is based on implementation. A UE in CM-IDLE over 3GPP shall report that the 3GPP access is unavailable. When the UE detects the unavailability/availability of an access, it shall:

-	build a PMF-Access Report containing the access type and an indication of availability/unavailability of this access;

-	send the PMF-Access Report to the UPF via the user plane.

The UPF shall acknowledge the PMF-Access Report received from the UE.

# >>>>END OF CHANGES<<<<