3GPP TSG-WG SA2 Meeting #158	S2-2309463

# Goteborg, Sweden, August 21 – 25, 2023		(revision of S2-2308620)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4303 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling UE number with at least one PDU Session under Hierarchical NSAC architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNSAC |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | When hierarchical NSAC architecture is deployed at the network, how to handle the UE number with at least one PDU Session is missing. - For option 1, it is clear that two counting are executed independently. - For optoin 2, two counting is exectued at the same NSACF. So for hierarchical NSAC architecture, it follows the same principle. Also due to the UE already registered indication will not be provided by the SMF+PGW-C, the UE entry will not be created at the Primary NSACF. Hence in this case only the additioanl quota is returned to the NSACF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the related procedure on how to handle the UE number with at least one PDU Session is missing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear how to handle the NSAC for the UE number with at least one PDU Session when hierarchical NSAC architecture is deployed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.5.9a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.11.5.9a	Network Slice Admission Control in 5GS for maximum number of UE with at least one PDU session and one PDN connection

Support of NSAC in conjunction with interworking with EPC for maximum number of UE with at least one PDU session/PDN connection is described in clause 5.15.11.5a of TS 23.501 [2].

For deployment option 1 of two separate NSACF, the same mechanisms in clause 4.2.11.2 is used with the following additions:

-	The SMF+PGW-C invokes this procedure during PDU session establishment and PDU connection release. In this case, the AMF in figure 4.2.11.2-1 is replaced with SMF+PGW-C.

-	Step 2: The SMF+PGW-C includes in the message the S-NSSAI, identity of SMF+PGW-C, UE ID and update flag. The update flag may include either 'increase', 'decrease' or 'update' values.

-	Step 2: The AMF is replaced by the SMF+PGW-C ID.

-	Step 3: The 'update' value indicates that for existing UE registration the Access Type is to be replaced with a new Access Type during inter access mobility. If the update flag parameter from the SMF+PGW-C indicates update value, the NSACF locates the existing entry with UE ID and NF ID and replaces the Access Type in the existing entry. If there was no UE entry in the new Access Type then NSACF will create an UE entry and increase the number of UE registered. If there was one UE entry in the new Access Type with same NF ID then NSACF will not increase the count again. If there was one UE entry in the new Access Type with different NF ID then NSACF will add this new NF ID but does not increase the count. If there was one entry with same NF ID in the old Access Type then NSACF will remove the UE entry and decrease the count.

-	The NSACF determines whether or not to accept the request as described in clause 5.15.11.5a of TS 23.501 [2]

NOTE 1:	EAC mode is not applicable here.

For deployment option 1, if Hierarchical NSAC architecture is deployed at the network, besides the above impact:

- For NSAC for number of UEs, when the local maximum number or local threshold is reached, the NSACF may interact with the Primary NSACF before it returns the response back to the SMF+PGW-C. For more details on handling between the NSACF and Primary NSACF see clause 4.2.11.2a.

- For NSAC for number of PDU Sessions, when the local maximum number is reached, the NSACF may interact with the Primary NSACF before it returns the response back to the SMF+PGW-C. For more details on handling between the NSACF and Primary NSACF see clause 4.2.11.4a.

For deployment option 2 of single NSACF, same mechanisms in clause 4.2.11.4 are used with the following additions:

-	The NSACF also counts the maximum number UE with at least one PDU session/PDN connection based on the update value received from SMF+PGW-C.

-	If the update flag parameter from the SMF+PGW-C anchoring the PDU session/PDN connections indicates increase value and the maximum number of UE with at least one PDU session/PDN connection has already been reached, then the NSACF returns a result parameter indicating that the maximum number of PDU Sessions per network slice has been reached. If the maximum number of UE with at least one PDU session/PDN connection has not been reached, the NSACF checks the UE ID. If the UE ID is located, the NSACF stores the NF ID and the Access Type and does not increase the number of UE with at least one PDU session/PDN connection for that S-NSSAI. If the NSACF did not locate the UE ID, it creates an entry for the UE ID, stores the NF ID and Access Type and increases the number of PDU Sessions for that S-NSSAI.

-	If the update flag parameter from the SMF+PGW-C anchoring the PDU session/PDN connections indicates decrease and if there is only one entry associated with the UE ID, the NSACF removes the UE ID from the list of UEs registered with the network slice for each of the S-NSSAI(s) indicated in the request from the SMF+PGW-C and also the NSACF decreases the number of UEs per network slice that is maintained by the NSACF for each of these network slices. If there are multiple entries associated with the UE ID, the NSACF removes the entry associated with the NF ID but the UE ID is kept in the list of UEs registered with the S-NSSAI. NSACF decrease the count for number of UEs with at least one PDU session/PDN connection and removes the entry only when NSACF find the last entry.

-	If the update flag parameter from the SMF/SMF+PGW-C anchoring the PDU session/PDN connections indicates update, the NSACF locates the existing entry with UE ID and NF ID and replaces the Access Type in the existing entry. If there was no UE entry in the new Access Type then NSACF will create an UE entry and increase the number of UE registered. If there was one UE entry in the new Access Type with same NF ID then NSACF will not increase the count again. If there was one UE entry in the new Access Type with different NF ID then NSACF will add this new NF ID but does not increase the count. If there was one entry with same NF ID in the old Access Type then NSACF will remove the UE entry and decrease the count.

-	The NSACF determines whether to accept the request as described in clause 5.15.11.5a of TS 23.501 [2]

NOTE 2:	EAC mode is not applicable here.

For deployment option 2, if Hierarchical NSAC architecture is deployed at the network, besides the above impact:

- The NSAC for number of UEs and NSAC for number of UEs need be handled within the same NSACF or Primary NSACF.

- For NSAC for number of UEs, when the local maximum number or local threshold is reached, the NSACF may interact with the Primary NSACF before it returns the response back to the SMF+PGW-C. For more details on handling between the NSACF and Primary NSACF see clause 4.2.11.2a.

NOTE 3: 	The SMF+PGW-C does not provide the UE already registered indication to NSACF. Hence when the NSACF interacts with Primary NSACF, the Primary NSACF does not manage the UE entry by itself.

- For NSAC for number of PDU Sessions, when the local maximum number is reached, the NSACF may interact with the Primary NSACF before it returns the response back to the SMF+PGW-C. For more details on handling between the NSACF and Primary NSACF see clause 4.2.11.4a.

# * * * * End of changes * * * *