SA WG2 Meeting #S2-159	S2-2310108

09 - 13 October, 2023, Xiamen, P.R. China

3GPP TSG-CT WG4 Meeting #117	C4-233573

# Göteborg, Sweden, 21st– 25th August 2023

# Title:	LS on Data rate monitoring

# Release:	Rel-18

# Work Item:	XRM

Source:	CT4

To:	SA2

Cc:	CT3

Contact Person:

Name:	Bruno Landais

Tel. Number:

E-mail Address:	bruno.landais@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

Clause 5.45.4 of TS 23.501 specifies:

The QoS Monitoring for data rate allows the measurement of the UL and/or DL data rate per QoS flow at UPF and it can be applied to a Non-GBR or GBR QoS flow. The data rate is measured over a monitoring averaging window with a standardized value.

According to the QoS Monitoring request for UL and/or DL data rate from SMF, UPF is required to initiate data rate measurement for a QoS Flow and to report the measured data rate as instructed.

CT4 kindly asks SA2 to clarify:

- the data rate information that the UPF shall expose. 

For instance, if the (PFCP) Measurement Period IE (indicating the period to generate periodic measurement reports) is set to 10 seconds and the monitoring averaging window is 2 seconds, should the UPF measure the data rate during 2 seconds only once within the 10 seconds and report this measurement, or should the UPF measure the data rate up to 5 times within the 10 seconds and report the minimum, maximum and average data rate over the measurement period?

- what is the standardized value of the monitoring averaging window for non-GBR QoS flows?

2. Actions:

To SA2 group.

ACTION: 	CT4 kindly asks SA2 to provide the requested clarifications.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm