3GPP TSG-SA WG2 Meeting #159	S2-2310236

# Xiamen, China, October 09 – 13, 2023								          (revision of S2-23xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0360 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of Candidate 5G ProSe UE-to-UE Relay Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the incoming LS (S2-23xxxxx/C1-236448), CT1 asked the following questions about Candidate 5G ProSe UE-to-UE Relay Discovery: According to clause 6.3.2.4.4 of TS 23.304, for candidate 5G ProSe UE-to-UE Relay Discovery to support the negotiated Relay reselection, the discoverer End UE discovers a candidate 5G ProSe UE-to-UE Relay without sending the RSC and the User Info ID of the discoveree 5G ProSe End UE. Then: Question 2: How can the 5G ProSe UE-to-UE relay UE tell that the 5G ProSe UE-to-UE Relay Discovery Solicitation message is for the relay reselection so that it can skip finding the target end UE and respond to the source end UE directly? Question 3: How can the 5G ProSe UE-to-UE relay UE know which service the discoverer end UE wants to use before accepting the 5G ProSe UE-to-UE Relay Discovery Solicitation message? For Question 3, the issue is that if “In the 5G ProSe UE-to-UE Relay Discovery Solicitation message the RSC is not included and the User Info ID of the discoveree 5G ProSe End UE is replaced with the User Info ID of a candidate 5G ProSe UE-to-UE Relay (as described in Step 1 of clause 6.3.2.4.4)”, then the “RSC” becomes optional in the UE-to-UE Relay Discovery Solicitation message and the UE-to-UE Relay determines whether to send Solicitation message to target End UEs (as described in step 2 of clause 6.3.2.4.3) or to send to a Response message to the source End UE directly (as described in Step 4 of clause 6.3.2.4.4) based on the presence/absence of RSC in the received Solicitation message. This complicates the UE-to-UE Relay operation and makes the Model B UE-to-UE Relay discovery procedure ambiguous. Furthermore, the Candidate 5G ProSe UE-to-UE Relay Discovery can be seen as an optimization of Model B UE-to-UE Relay discovery which is not necessary but is introducing complexity and ambiguity to CT1 protocol design. The Model B UE-to-UE Relay discovery can be used by the target End UE to discover the selected UE-to-UE Relay. For Question 2, the issue is that if “In the 5G ProSe UE-to-UE Relay Discovery Solicitation message the RSC is not included and the User Info ID of the discoveree 5G ProSe End UE is replaced with the User Info ID of a candidate 5G ProSe UE-to-UE Relay (as described in Step 1 of clause 6.3.2.4.4)”, it introduces more ambiguity to CT1 protocol design. In the Model B UE-to-UE Relay discovery, User Info ID of target 5G ProSe End UE is mandatory, but now it becomes optional in the Candidate 5G ProSe UE-to-UE Relay Discovery as in fact the User Info ID of a candidate 5G ProSe UE-to-UE Relay is included instead. On the other hand, User Info ID of 5G ProSe UE-to-UE Relay is optional in the Model B UE-to-UE Relay discovery but it becomes mandatory in the Candidate 5G ProSe UE-to-UE Relay Discovery. Again, the Model B UE-to-UE Relay discovery can be used by the target End UE to discover the selected UE-to-UE Relay. For the Candidate 5G ProSe UE-to-UE Relay Discovery procedure (as described in clause 6.3.2.4.4), it is not clear whether RSC needs to be included in the UE-to-UE Relay Discovery Response message and if it is needed how does the UE-to-UE Relay decide which RSC should be included as no RSC is included in the UE-to-UE Relay Discovery Solicitation message. Based on the above discussion, it can be seen that Candidate 5G ProSe UE-to-UE Relay Discovery is not necessary but is introducing complexity and ambiguity to CT1 protocol design. It is proposed to use the Model B UE-to-UE Relay discovery for target End UE to discover the selected UE-to-UE Relay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to remove the Candidate 5G ProSe UE-to-UE Relay Discovery and to use the Model B UE-to-UE Relay discovery for target End UE to discover the selected UE-to-UE Relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary procedure introduces complexity and ambiguity to CT1 protocol design. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.4, 6.7.4.2, 6.7.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * 1st Change * * * *

6.3.2.4.4	Void

* * * * 2nd Change * * * *

6.7.4.2	Negotiated 5G ProSe Layer-2 UE-to-UE Relay reselection

Depicted in Figure 6.7.4.2-1 is the procedure for the negotiated 5G ProSe Layer-2 UE-to-UE Relay reselection.

Figure 6.7.4.2-1: Negotiated 5G ProSe Layer-2 UE-to-UE Relay reselection

1.	A PC5 unicast link is established between the 5G ProSe End UEs via a 5G ProSe UE-to-UE Relay, based on the procedure defined in clause 6.7.2.

2.	The initiating 5G ProSe End UE determines, e.g. based on PC5 signal strength, to perform UE-to-UE Relay reselection and obtains a list of candidate UE-to-UE Relays per RSC. The initiating 5G ProSe End UE may receive UE-to-UE Relay Discovery Announcement messages from 5G ProSe UE-to-UE Relays or initiate the 5G ProSe UE-to-UE Relay discovery procedures to find the candidate 5G ProSe UE-to-UE Relays. The initiating 5G ProSe End UE determines the candidate 5G ProSe UE-to-UE Relays e.g., based on the PC5 signal strength of the received UE-to-UE Relay Discovery Announcement message, RSC within the UE-to-UE Relay Discovery Announcement message. The candidate 5G ProSe UE-to-UE Relays support the same RSC which is associated with the PC5 unicast link between the initiating 5G ProSe End UE and the 5G ProSe UE-to-UE Relay.

3.	The initiating 5G ProSe End UE sends a Link Modification Request message to the responding 5G ProSe End UE which includes a Relay re-selection indication, the User Info ID(s) of the candidate 5G ProSe UE-to-UE Relay(s) and optionally the Layer-2 ID(s) of the candidate 5G ProSe UE-to-UE Relay(s) and security information.

4.	The responding 5G ProSe End UE selects a new 5G ProSe UE-to-UE Relay from the candidate 5G ProSe UE-to-UE Relays per RSC, based on the Relay re-selection indication in the Link Modification Request message. If the responding 5G ProSe End UE has not received a UE-to-UE Relay Discovery Announcement message from a candidate 5G ProSe UE-to-UE Relay (e.g. during a previous 5G ProSe UE-to-UE Relay Discovery procedure) or does not have a PC5 connection with the candidate 5G ProSe UE-to-UE Relay associated with the same RSC, then the responding 5G ProSe End UE may perform the 5G ProSe UE-to-UE Relay discovery procedure with the User Info ID of a candidate 5G ProSe UE-to-UE Relay in discovery message and may set the Layer-2 ID of the candidate 5G ProSe UE-to-UE Relay, if received at step 3, as the Destination Layer-2 ID to carry the discovery message, by using the 5G ProSe UE-to-UE Relay Discovery procedure defined in clause 6.3.2.4.3. The PC5 signal strength of the UE-to-UE Relay Discovery Announcement message or UE-to-UE Relay Discovery Response message may be used to select the new 5G ProSe UE-to-UE Relay.

5.	The responding 5G ProSe End UE sends a Link Modification Accept message to the initating 5G ProSe End UE, including the User Info ID of the new 5G ProSe UE-to-UE Relay and security information.

6.	5G ProSe End UEs set up PC5 unicast links, if not already set up, with the new 5G ProSe UE-to-UE Relay, by reusing the procedure defined in clause 6.7.2 and the PC5 unicast is link established between 5G ProSe End UEs via the new 5G ProSe UE-to-UE Relay. The 5G ProSe End UEs switch the data traffic via the new 5G ProSe UE-to-UE Relay. The security information is used to verify that the new link has been set up successfully.

NOTE 1:	The security information contents and usage will be defined by SA WG3.

NOTE 2:	Whether a 5G ProSe End UE releases a PC5 unicast link with the original 5G ProSe UE-to-UE Relay after reselection depends on whether the PC5 unicast link is still required and UE implementation.

* * * * 3rd Change * * * *

6.7.4.3	Negotiated 5G ProSe Layer-3 UE-to-UE Relay reselection

Depicted in Figure 6.7.4.3-1 is the procedure for the negotiated 5G ProSe Layer-3 UE-to-UE Relay reselection.

Figure 6.7.4.3-1: Negotiated 5G ProSe Layer-3 UE-to-UE Relay reselection

1.	5G ProSe End UEs have set up PC5 unicast links with a 5G ProSe UE-to-UE Relay, based on the procedure defined in clause 6.7.1.

2.	The 5G ProSe End UEs have additionally set up PC5 unicast links with a 5G ProSe UE-to-UE Relay, based on the procedure defined in clause 6.7.1.

3.	The 5G ProSe End UEs are transferring traffic via the 5G ProSe UE-to-UE Relay.

4.	The initiating 5G ProSe End UE determines, e.g. based on PC5 signal strength, to perform UE-to-UE Relay reselection and obtains a list of candidate UE-to-UE Relays per RSC. The initiating 5G ProSe End UE may receive UE-to-UE Relay Discovery Announcement messages from 5G ProSe UE-to-UE Relays or initiate the 5G ProSe UE-to-UE Relay discovery procedures to find the candidate 5G ProSe UE-to-UE Relays. The initiating 5G ProSe End UE determines the candidate 5G ProSe UE-to-UE Relays e.g., based on the PC5 signal strength of the received UE-to-UE Relay Discovery Announcement message, RSC within the UE-to-UE Relay Discovery Announcement message. The candidate 5G ProSe UE-to-UE Relays support the same RSC which is associated with the PC5 unicast link between the initiating 5G ProSe End UE and the 5G ProSe UE-to-UE Relay.

5.	The initiating 5G ProSe End UE sends a Link Modification Request message to the responding 5G ProSe UE-to-UE Relay, which includes a Relay re-selection indication, the User Info ID(s) of the candidate 5G ProSe UE-to-UE Relay(s), the IP addresses of the responding 5G ProSe End UEs and optionally the Layer-2 ID(s) of the candidate 5G ProSe UE-to-UE Relay(s).

Multiple 5G ProSe End UEs IP addresses may be included when the initiating 5G ProSe End UE is communicating with multiple 5G ProSe End UEs via the 5G ProSe UE-to-UE Relay.

6.	5G ProSe UE-to-UE Relay determines the responding 5G ProSe End UE based on the IP address received from the initiating 5G ProSe End UE and sends a Link Modification Request message to the responding 5G ProSe End UE. The Link Modification Request message includes a Relay re-selection indication, User Info ID(s) of the candidate 5G ProSe UE-to-UE Relay(s), IP address of the initiating 5G ProSe End UE and optionally the Layer-2 ID(s) of the candidate 5G ProSe UE-to-UE Relay(s).

If multiple 5G ProSe End UEs are specified in the Link Modification Request message received from the initiating 5G ProSe End UE, the 5G ProSe UE-to-UE Relay sends a PC5 Link Modification Request to each of the 5G ProSe End UEs.

7.	The responding 5G ProSe End UE selects a new 5G ProSe UE-to-UE Relay from the candidate 5G ProSe UE-to-UE Relays per RSC, based on the Relay re-selection indication in the Link Modification Request message. If the responding 5G ProSe End UE has not received a UE-to-UE Relay Discovery Announcement message from a candidate 5G ProSe UE-to-UE Relay (e.g. during a previous 5G ProSe UE-to-UE Relay Discovery procedure) or does not have a PC5 connection with the candidate 5G ProSe UE-to-UE Relay associated with the same RSC, then the responding 5G ProSe End UE may perform the 5G ProSe UE-to-UE Relay discovery procedure with the User Info ID of a candidate 5G ProSe UE-to-UE Relay in discovery message and may set the Layer-2 ID of the candidate 5G ProSe UE-to-UE Relay, if received at step 6, as the Destination Layer-2 ID to carry the discovery message, by using the 5G ProSe UE-to-UE Relay Discovery procedure defined in clause 6.3.2.4.3 and initiates the procedure to set up PC5 unicast links with the new 5G ProSe UE-to-UE Relay, by reusing the procedure defined in clause 6.7.1. The PC5 signal strength of the UE-to-UE Relay Discovery Announcement message or UE-to-UE Relay Discovery Response message may be used to select the new 5G ProSe UE-to-UE Relay.

8.	The responding 5G ProSe End UE sends a Link Modification Accept message to the 5G ProSe UE-to-UE Relay, including the User Info ID of the new 5G ProSe UE-to-UE Relay, IP address of the initiating 5G ProSe End UE, IP address of the responding 5G ProSe End UE for communication via the newly selected 5G ProSe UE-to-UE Relay and Relay re-selection indication.

9.	5G ProSe UE-to-UE Relay sends a Link Modification Accept message to the initating 5G ProSe End UE, including the User Info ID of the new 5G ProSe UE-to-UE Relay, IP address of the responding 5G ProSe End UE, IP address of the responding 5G ProSe End UE for communication via the newly selected 5G ProSe UE-to-UE Relay and Relay re-selection indication.

10-11.	Link Modification Ack is sent from the initating 5G ProSe End UE to the responding the 5G ProSe End UE via the 5G ProSE UE-to-UE Relay, including the IP address of the initiating 5G ProSe End UE for communication via the newly selected 5G ProSe UE-to-UE Relay, the IP address of the responding 5G ProSe End UE and Relay re-selection indication.

12.	The 5G ProSe End UEs transfer traffic via the newly selected 5G ProSe UE-to-UE Relay.

NOTE:	Whether a 5G ProSe End UE releases a PC5 unicast link with the original 5G ProSe UE-to-UE Relay after reselection depends on whether the PC5 unicast link is still required and UE implementation.

* * * * End of Changes * * * *