SA WG2 Meeting #S2-159	S2-2310256

Xiamen, P. R. China, October 9-13, 2023

Title:	Reply LS on Issues on Data rate monitoring

Release:	Rel-18

Work Item:	XRM

Source:	SA2

To:	CT4

Cc:

Contact person:	Dan Wang

wangdanyjy@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	No

- Overall description

SA2 thanks to CT4 for the LS. SA2 discussed the questions and reaches the following consensus:

Question 1:

- the data rate information that the UPF shall expose.

For instance, if the (PFCP) Measurement Period IE (indicating the period to generate periodic measurement reports) is set to 10 seconds and the monitoring averaging window is 2 seconds, should the UPF measure the data rate during 2 seconds only once within the 10 seconds and report this measurement, or should the UPF measure the data rate up to 5 times within the 10 seconds and report the minimum, maximum and average data rate over the measurement period?

Answer 1: SA2 thinks that the data rate is a statistic average value measured and calculated over the monitoring window. Therefore one-time measurement within the 10 seconds may be fortuitous and result in unexpected or abnormal data rate value. Therefore, it is expected the UPF can measure the data rate up to 5 times within the 10 seconds, and report the minimum,maximum and average data rate over the measurement period.

Question 2:	2) what is the standardized value of the monitoring averaging window for non-GBR QoS flows?

A2: One typical standardized value of the averaging window is 2000ms.

- Actions

To CT4:

ACTION: 	SA2 kindly asks CT4 to take the above reply into account.

3	Dates of next TSG SA2 meetings

3GPP TSG SA2#160	13th – 17th November, 2023	Chicago, USA

3GPP TSG SA2#Ad hoc  22th-26th January, 2024             Online