SA WG2 Meeting #S2-159	S2-2310273

9 - 13 October 2023, Xiamen, China	(revision of S2-230xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4464 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to information flow for UPF event exposure service for any UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2023-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently the Step 1 in figure Figure 4.15.4.5.3-1, leads to ambiguity whether the consumer we are talking about is UPF event consumer or some entity which consumer NWDAF service, and NWDAAF acts as the UPF event consumer. Further there are incorrect clause number for the references to other procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated Step-1 in the figure, firstly to imply that step is conditional to when event consumer is NWDAF; secondly changed consumer to analytics consumer, so that consumer here is not mistaken with UPF events consumer - Corrected the reference in Step 6. (from Figure 4.15.4.6.2-1 to Figure 4.15.4.5.2-1) - Corrected the reference in Step. 7; and changed wording from “same as” -> “as described in” - Other editorial and minor wording modifications |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect/ambiguous specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 4.15.4.5.3	Information flow for UPF event exposure service for any UE

Figure 4.15.4.5.3-1: UPF Information Exposure to the UPF event consumer (e.g. NWDAF) of any UE scenario

1.	(For the case when the UPF event consumer is NWDAF) The analytics consumer sends a request to the NWDAF for analytics on any UE. The analytics consumer provides the value “any UE” in the Target of Analytics Reporting. Analytics Filter Information optionally contains DNN, S-NSSAI, Area of Interest, Application server IP address/FQDN, APP ID, DNAI, etc.

2.	(Optional and only when the UPF event consumer is NWDAF) If the Analytic Filter Information does not contain DNN/S-NSSAI, but only includes application server IP address/FQDN, the NWDAF should first obtain the DNAI from NEF. The NWDAF invokes Nnef_DNAIMapping_Subscribe service to request the DNAI information. The request includes EAS IP/IP range and/or FQDN.

3.	(Conditional, if step 2 took place) The NEF determines the suitable DNAI(s) and provides them to NWDAF.

4.	The UPF event consumer triggers the SMF(s)/UPF(s) discovery to NRF by Nnrf_NFDiscovery_Request providing the DNN, S-NSSAI, DNAI etc. This procedure is to discover the related SMF(s)/UPF(s) associated with any UE and support the indicated DNAI. Either SMF(s) or UPF(s) are discovered depending on whether the subscription request to UPF events meets the criteria for direct subscription to UPF as defined in clause 5.8.2 of TS 23.501 [2]).

5.	The NRF provides Nnrf_NFDiscovery_Response that may refer to several SMFs/UPFs.

6.	(Option 1) If the subscribed UPF events needs the SMF(s) to do a third-party subscription onto UPF (as defined in clause 5.8.2 of TS 23.501 [2]), the same procedure as Indirect subscription via several SMFs (steps 3 - 5 in Figure 4.15.4.5.2-1(for single UE)) takes place via each discovered SMF.

7.	(Option 2) If the subscribed UPF events allows to directly subscribe to UPF (as defined in clause 5.8.2 of TS 23.501 [2]), the UPF event consumer (e.g. NWDAF) triggers the Nupf_EventExposure_Subscribe to all discovered UPFs. The information included in the subscription is as described in step 3 in clause 4.15.4.5.2.

8.	Each of the UPFs invokes Nupf_EventExposure_Notify service operation directly to the UPF event consumer (e.g. NWDAF).

END OF CHANGES