SA WG2 Meeting #159	S2-2310302

# Xiamen, PRC, October 9-13th, 2023            	       		                 (revision of S2-231)

Title:	[draft] LS on Support of multiple UEs in Northbound APIs

Response to:	C3-232510/ S2-2310077

Release:	Rel-18

Work Item:	NBI18

Source:	Nokia, Will be SA2

To:	CT3

Cc:	SA6

Contact Person:

#### Name:	 Laurent Thiebaut

E-mail Address:	 Laurent.thiebaut@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall Description:

SA2 thanks CT3 for their LS. SA2 have discussed it and would like to answer the following:

- Many NEF API(s) already support an External Group Identifier as a target of the API thus most of the cases where a collection of UE(s) is targeted by the AF should already be covered.

- An optimization to handle the same AF request that would target many UE(s) may require to impact the southbound interfaces of the NEF (downstream APIs, e.g. 5GC APIs), interfaces that may not be in the scope of CT3.

- Current NEF API where currently a list of UE(s) may be provided often assume some specific input parameters when a list of UE(s) is provided, e.g. the common EAS requirement for Nnef_TrafficInfluence_Create, a Consolidated Data Rate Threshold for Nnef_AFsessionWithQoS. SA2 should assess how to support multiple UEs on a case-by-case basis.

The addition of the option to provide a list of UE(s) (as the API target) to any NEF API may thus be more than a simple protocol optimization while many NEF API(s) already support a Group Identifier as a target. As Rel18 is now functionally frozen at SA2 level, SA2 believes that such optimizations should not be pursued in this release.

2. Actions:

To CT3:

ACTION: 	SA2 kindly asks CT3 to take the above information into account.

3. Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#160 | November 13th – 17th, 2023 | Chicago | USA |
| SA2#160 AH | January 22-26th, 2024 | TBD | TBD |
| SA2#161 | February 26th – March 1st, 2024 | EU | EU |

Please see https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/