3GPP TSG-WG SA2 Meeting #159	S2-2310310

Xiamen, China, Oct 9 – 13, 2023

Title:	[DRAFT] LS Reply on support including of data source information in the analytics request

Response to:	LS (C3-2333756) on support including of data source information in the analytics request from CT3

Release:	Release 18

Work Item:	eNA_Ph3, eNetAE

Source:	[Huawei to be] SA2

To:	CT3

Cc:

Contact Person:

Name:	Haiyang Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	 TS 23.288 CR 0931

1. Overall Description:

SA2 thanks CT3 for the LS on support including of data source information in the analytics request. Please find SA2 Answer for the following question:

Question 1: May the Data Source NF information be optionally provided also in Nnwdaf_AnalyticsSubscription_Subscribe and Nnwdaf_AnalyticsInfo_Request service operations?

SA2 Answer:  The NWDAF_DataManagement_Subscribe or Ndccf_DataManagement_Subscribe is used for Enhanced Procedures for Data Collection as described in clause 6.2.6 of TS 23.288 and NWDAF could be the consumer, i.e. NWDAF can provide the Data Source information, e.g. NF Instance (or NF Set) ID from which the data needs to be collected in the subscription.  There are two cases:

- For data collection to retrieve behavior data which is not related to specific UEs. The NWDAF can decide the source NF Instance (or NF Set) ID by configuration. It seems there is no benefit to provide Data Source NF information in Nnwdaf_AnalyticsSubscription_Subscribe and Nnwdaf_AnalyticsInfo_Request service operations to notify NWDAF about it.

- For data collection to retrieve behavior data for specific UE(s), currently the NWDAF needs to shall first determine which NF instances are serving the UE(s) as stated in table 6.2.2.1-2 of TS 23.288 unless the NWDAF has already obtained this information due to recent operations related to this UE. Thus SA2 agrees to provide Data Source NF information in Nnwdaf_AnalyticsSubscription_Subscribe and Nnwdaf_AnalyticsInfo_Request service operations to notify NWDAF about it to avoid redundant operation listed in table 6.2.2.1-2 of TS 23.288.

In conclusion, SA2 agreed the Data Source NF information be optionally provided also in Nnwdaf_AnalyticsSubscription_Subscribe and Nnwdaf_AnalyticsInfo_Request service operations as in the attachment.

2. Actions:

To CT3 group.

ACTION: 	SA2 asks CT3 group to kindly take the above into account and update their specifications accordingly, if needed.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #159		November 13 – 17, 2023			Chicago, USA

TSG-SA2 Meeting #160 Ad Hoc	January 22 – 26, 2024			Online