3GPP TSG-SA2 Meeting #159	S2-2310421

09 - 13 October, 2023, Xiamen, P.R. China	(revision of S2-230xxxx)

Title:	[draft] Reply LS on Issues on QoS Monitoring parameters for XR service

Response to:	LS on Issues on QoS Monitoring parameters for XR service (S2-2310102/C3-233649)

Release:	Rel-18

Work Item:	XRM

Source:	SA2

To:	CT3

Cc:	-

Contact person:

#### Name:	Hyunsook Kim

#### E-mail Address:	hyuns.kim at lge dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.501 CR#xxxx

1	Overall description

SA2 would like to thank CT3 regarding the LS on Issues on QoS Monitoring parameters for XR service.

SA2 has discussed the matter in SA2#159 meeting and approved the attached CR. Also, SA2 provides following answer.

Question 1:	If NG-RAN provides congestion measurements based on NG-RAN implementation, do the reporting period, and periodic and event triggered reporting frequencies apply to the monitoring and exposure of congestion information?

SA2 Answer:	From the NG-RAN’s perspective, the monitoring reporting period and/or frequencies on the congestion information are up to NG-RAN implementation. The UPF can report the monitoring results as specified in clause 5.8.2.18 of TS 23.501.

Question 2:	If the answer to question 1 is yes, for event triggered reporting, does it mean that the minimum and the maximum congestion reports provided by NG-RAN also need to be reported, if received within the minimum waiting time?

SA2 Answer:	The congestion reports provided by NG-RAN are up to NG-RAN implementation. If received, the UPF may report the monitoring results as specified in clause 5.8.2.18 of TS 23.501, and it depends on the reporting instruction configured by SMF based on the authorized QoS Monitoring policy received from the PCF or local configuration or both.

Question 3:	For data rate monitoring, if the averaging window is shorter than the waiting time, does the UPF report the minimum and maximum monitored data rate or should the UPF report the average?

SA2 Answer:	As specified in clause 5.8.2.18 of TS 23.501, if the measurement results are received during the Minimum waiting time, the UPF shall report the minimum and the maximum measurement result when the Minimum waiting time is over.

Question 4:	Do the minimum and the maximum PDV measurement results also need to be reported by the PCF?

SA2 Answer:	From the PCF’s perspective, the reporting period and/or frequencies on Packet Delay Variation are up to PCF implementation.

Question 5:	For the Congestion information, Data Rate and PDV, does it mean that the measurement failure also needs to be reported if no measurement result is available?

SA2 Answer:	As specified in clause 5.8.2.18 of TS 23.501, the UPF may report a measurement failure, and it depends on the reporting instruction configured by SMF based on the authorized QoS Monitoring policy received from the PCF or local configuration or both.

Question 6:	If the answer to question 5 is yes, how does the UPF detect that no Congestion or Data Rate measurement result is available?

SA2 Answer:	The UPF considers that the congestion monitoring result is not provided by NG-RAN during the reporting period, to be “no available congestion measurement result”. The data rate measurement is based on average window, therefore, the UPF considers that the reporting period is shorter than the average window, then the measurement result is not calculated yet, to be “no available data rate measurement result”.

Question 7:	Whether all TSC QoS requirements are applicable to multi-modal/XRM communications or not?

SA2 Answer:	Generally, it depends on the traffic characters for XR Services. In addition, it is noteworthy that as specified in clause 5.37.8.1 of TS 23.501, the UL and/or DL Periodicity and N6 Jitter Information associated with the DL Periodicity can be provided by the CN to NG RAN via TSCAI.

2	Actions

To CT3

ACTION: 	SA2 kindly asks CT3 to take above information into consideration.

3	Dates of next TSG SA WG2 meetings

TSG-SA2#160		November 13 – 17, 2023		Chicago, USA

TSG-SA2 Ad-hoc	January 22 – 26, 2024		e-meeting