SA WG2 Meeting #159	S2-2310443

October 09 – 13, 2023, Xiamen, China	(revision of S2-231xxxx)

Source:	LG Electronics

Title:	New KI: Enhancement for subscription and policy control to support energy related information as service criteria

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This paper proposes a new key issue for FS_EnergySys regarding WT#2.

Discussion

FS_EnergySys SID (SP-231192) includes the following WT:

- WT #2. Study enhancement for subscription and policy control to enable network energy savings as service criteria.

This paper proposes new KI related to WT#2 based on clause 6.15a "Energy Efficiency as a Service Criteria", in particular clause 6.15a.2 "Energy related information as a service criteria" of TS 22.261v19.4.0.

Proposal

It is proposed to agree the following changes into TR 23.700-66.

* * * * Start of 1st Change * * * *

!!! All New Texts !!!

5.X	Key Issue #X: Enhancement for subscription and policy control to support energy related information as service criteria

5.X.1	General description

For best-effort traffic for services without QoS criteria, policies can be defined to limit energy use for the services. Additionally, policies can be defined with a maximum energy credit limit, e.g. for best-effort services to limit the total amount of energy consumption. These policies expand the options of subscription policies to control energy consumption in the 5G system.

This key issue addresses how to enhance subscription and policy control to support energy related information as service criteria. The energy related information can be e.g. a maximum energy credit limit, a maximum energy consumption (i.e. quantity of energy for a specified period of time). In particular, the following aspects are required to be studied:

-	Whether and how the 5G system supports subscription policies that define the energy related information for services without QoS criteria.

-	Whether and how the 5G system supports a means to enforce the policy for the energy related information for services without QoS criteria to constrain energy consumption.

NOTE:	The granularity of the subscription policies can either apply to the subscriber (all services), or to particular services.

* * * * End of Changes * * * *