SA WG2 Meeting #159	S2-2310459

# October 9th – 13th, 2023; Xiamen, CN               		(revision of S2-220)

Source: 	CATT, Thales

Title: 	FS_5GSAT_Ph3_ARCH_Key issue for WT#1

Document for: 	Approval

Agenda Item: 	19.1

Work Item / Release:	FS_5GSAT_ARCH_Ph3 / Rel-19

Abstract of the contribution: this paper proposes a key issue for the WT#1 of SID FS_5GSAT_ARCH_Ph3.

1	Discussion

The Work Task#1 of the SID is:

WT1: Regenerative payload generic architecture study

WT-1.1: Study and identify any impacts on 5GS and EPS for the scenario with gNB/eNB embedded on the satellite.

In fact, a key issue for this WT was already identified in Rel-16 TR 23.737, but was not studied:

“5.6	Key Issue #6: RAN mobility with NGSO regenerative-based satellite access

5.6.1	General description

Embarking RANs on-board moving NGSO satellites implies frequent handovers of these RANs for any given 5CN, as well as their interconnection through an Xn interface.

As NGSO satellites coverage spans a large coverage, and can include a significant number of UEs, a large number of UE's could be simultaneously handed over from one RAN to another one, leading to a group handover of the anchor point be the RAN and the CN.

Under these circumstances, this key issue shall address the following points:

-	What are the conditions for a 5CN to handover from a RAN to another RAN?

-	What are the requirements on the Xn and on the NG interfaces to ensure the mobility of the RAN with respect to the 5CN?

-	What are the functional requirements on the 5CN and RAN to ensure the mobility of the RAN with respect to the CN?

Existing / proposed procedures from other topics should also be reviewed.

”

Therefore, it is proposed to use this key issue as the baseline in this study.

As RAN mobility will cause much frequent disconnection and re-connection of TNL over N2/S1-AP interface of an AMF/MME, it is also proposed to study how to efficient manage N2/S1-AP connection.

RAN mobility will also cause the change of RAN node serving a TA(s), thus for paging a UE in a TA(s), the AMF need to know which RAN node should page the UE over the air interface.

2 Proposal

It is proposed to update TR 23.700-29 for FS_5GSAT_ARCH_Ph3 as followed.

5	Key Issues

5.X	Key Issue #X: RAN mobility with NGSO regenerative-based satellite access

5.X.1	Description

Embarking RANs on-board moving NGSO satellites implies:

-	frequent handovers of these RANs for any given 5GC/EPC, as well as their interconnection through an Xn/X2 interface;

-	frequent disconnection and re-connection of TNL over N2/S1-AP interface of an AMF/MME;

-	frequent change of RAN node serving a TA.

As NGSO satellites coverage spans a large coverage, and can include a significant number of UEs, a large number of UE's could be simultaneously handed over from one RAN to another one, leading to a group handover of the anchor point be the RAN and the CN.

Under these circumstances, this key issue shall address the following points:

-	What are the conditions for a 5GC/EPC to handover from a RAN to another RAN?

-	What are the requirements on the Xn/X2 and on the NG/S1 interfaces to ensure the mobility of the RAN with respect to the 5GC/EPC?

-	What are the functional requirements on the 5GC/EPC and RAN to ensure the mobility of the RAN with respect to the CN?

-	How to manage N2/S1-AP connection efficiently, i.e., minimize the impact of frequent disconnection and re-connection of TNL over N2/S1-AP interface.

-	The impact to paging procedure when the RAN node(s) serving a registration area of a UE dynamically changes

-     The impact of dynamically changed latency over N2/S1-MME interface to mobility management procedure and session management procedure.