3GPP TSG-WG SA2 Meeting #159	S2-2310523

# Xiamen, China, October 09 – 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0944 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on providing data source information in the analytics request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT3 sends a LS (C3-233756) to SA2 asking whether the information (i.e. NF Instance ID(s) or NF Set ID(s)) of data source NF can be optionally provided in Nnwdaf_AnalyticsSubscription_Subscribe and Nnwdaf_AnalyticsInfo_Request service operations. Considering that: - Analytics consumer NF may have the information (i.e. NF Instance ID(s) or NF Set ID(s)) of data source NF(s) from which the input data for the requested analytics shall/should be collected, e.g. the PCF as analytics consumer has the information of the SMF serving the UE or PDU session and/or information of the AMF serving the UE when requesting for network analytics related to "Service Experience", "UE Mobility", "UE Communication", "Abnormal behaviour", "Data Dispersion", "Session Management Congestion Control Experience", etc as specified in TS 23.503; - Analytics consumer NF may have preferences on which NF instance to provide the input data, e.g. the PCF may have preferred AF(s) for providing input data for network analytics related to "User Data Congestion", "Dispersion Analytics", "DN Performance", etc as specified in TS 23.503, so as to obtain network analytics for (specific applications or areas served by) the specific AF; it should be allowed that the analytics consumer NF provides the information of data source NF(s), if known, to the NWDAF to facilitate data collection for network analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the analytics consumer NF can provide data source information when subscribing to or requesting for analytics information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NWDAF cannot know the data source NF information from the analytics consumer NF even if the consumer has valid information of data source NF(s) for data collection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3, 6.3.1, 6.4.1, 6.6.1, 6.7.2.1, 6.7.3.1, 6.7.5.1, 6.8.1, 6.9.1, 6.10.1, 6.11.1, 6.12.1, 6.13.1, 6.14.1, 6.16.1, 6.17.1, 6.18.1, 6.19.1, 6.20.1, 6.21.1, 7.2.2, 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

6.1.3	Contents of Analytics Exposure

The consumers of the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7 provide the following input parameters listed below.

-	A list of Analytics ID(s): identifies the requested analytics.

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. Analytics Filter Information is defined in the analytics related clauses.

-	Target of Analytics Reporting: indicates the object(s) for which Analytics information is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) A Notification Target Address (+ Notification Correlation ID) as defined in clause 4.15.1 of TS 23.502 [3], allowing to correlate notifications received from NWDAF with this subscription.

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Subscription Correlation ID: identifies an existing analytics subscription that is to be modified.

-	Related to analytic consumers that aggregate analytics from multiple NWDAF subscriptions:

-	[OPTIONAL] (Set of) NWDAF identifiers of NWDAF instances used by the NWDAF service consumer when aggregating multiple analytics subscriptions. See clause 6.1A.

-	Analytics Reporting Information with the following parameters:

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Analytics Reporting Parameters as per Event Reporting parameters defined in Table 4.15.1-1 of TS 23.502 [3].

NOTE 1:	When the Analytics Reporting Parameters indicates a periodic reporting mode and the periodicity of the report is equal to or greater than the Supported Analytics Delay associated with the Analytics ID (if available) defined in clause 6.2.6.2 of TS 23.501 [2], it is expected that the periodic reporting can be provided by the NWDAF as requested.

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Reporting Thresholds, which indicate conditions on the level of each requested analytics that when reached shall be notified by the NWDAF.

-	[OPTIONAL] Matching direction: A matching direction may be provided such as below, above, or crossed. If no matching direction is provided, the default direction is crossed.

-	[OPTIONAL] Acceptable deviation: An acceptable deviation from the threshold level in the non-critical direction (i.e. in which the QoS is improving) may be set to limit the amount of signalling.

-	Analytics target period: time interval [start..end], either in the past (both start time and end time in the past) or in the future (both start time and end time in the future). An Analytics target period in the past is a request or subscription for statistics. An Analytics target period in the future is a request or subscription for predictions. The time interval is expressed with actual start time and actual end time (e.g. via UTC time). When the Analytics Reporting Parameters indicate a periodic reporting mode, the time interval can also be expressed as positive or negative offsets to the reporting time, which indicates a subscription for predictions or statistics respectively. By setting start time and end time to the same value, the consumer of the analytics can request analytics or subscribe to analytics for a specific time rather than for a time interval.

-	Time window for historical analytics: time interval [start..end]. The time window for historical analytics indicates the time interval during which the historical analytics was generated. If the time window for historical analytics is included, the NWDAF only needs to provide the existing analytics, and does not need to generate new analytics.

-	[OPTIONAL] Data time window: if specified, only events that have been created in the specified time interval are considered for the analytics generation.

-	[OPTIONAL] Preferred level of accuracy of the analytics ("Low", "Medium", "High" or "Highest").

-	[OPTIONAL] Preferred level of accuracy per analytics subset ("Low", "Medium", "High" or "Highest"). When a preferred level of accuracy is expressed for a given analytics subset, it takes precedence for this subset over the above preferred level of accuracy of the analytics. Analytics subsets are defined in the "Output Analytics" clause of applicable analytics.

-	[OPTIONAL] Dataset Statistical Properties: information in order to influence the data selection mechanisms to be used for the generation of an Analytics ID, assuring that the generated Analytics ID reflects the statistical characteristics of the data that are relevant for the NWDAF consumer. The following dataset statistical properties are allowed:

-	Uniformly distributed datasets, which indicates the use of data samples that are uniformly distributed according to the different aspects of the requested analytics (e.g. equivalent data samples for each UE listed as a Target of Analytics Reporting or for S-NSSAIs included in the Analytics Filter Information).

-	Datasets with or without outliers, which indicates that the data samples shall consider or disregard data samples that are at the extreme boundaries of the value range.

-	Time when analytics information is needed (if applicable): indicates to the NWDAF the latest time the analytics consumer expects to receive analytics data provided by the NWDAF. It should not be set to a value less than the Supported Analytics Delay of the selected NWDAF if applicable. If the time is reached the consumer does not need to wait for the analytics information any longer, yet the NWDAF may send an error response or error notification to the consumer. "Time when analytics information is needed" is a relative time interval as the gap with respect to analytics request /subscription (e.g. "in 10 minutes").

NOTE 2:	If the analytics request contains the parameter "Time when analytics information is needed" for Analytics ID(s), this parameter takes precedence over the requested periodicity, if a periodic reporting mode is requested.

NOTE 3:	If the Time when analytics information is needed is provided and it is less than the Supported Analytics Delay per Analytics ID (if available) defined in clause 6.2.6.2 of TS 23.501 [2], it is expected that the NWDAF might not be able to treat the Analytics ID on time.

-	[OPTIONAL] Maximum number of objects requested by the consumer (max) to limit the number of objects in a list of analytics per Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response.

-	[OPTIONAL] Preferred granularity of location information: "TA level", "cell level" or "longitude and latitude level".

NOTE 4:	As defined in clause 4 of TS 23.032 [34], longitude and latitude level means the location information is expressed as longitude and latitude in geographical coordinate instead of TA ID or cell ID that is only known in 3GPP system. It also stands for the location information that is expressed as a reference point in local co-ordinate.

-	[OPTIONAL] Spatial granularity size: maximum number of TA or cells used to define an area for which analytics are provided. When this parameter is provided, the NWDAF should provide analytics per group of TA of cells accordingly.

-	[OPTIONAL] Temporal granularity size: minimum duration of each time slot for which analytics are provide. When this parameter is provided, the NWDAF should provide analytics per elementary time slot accordingly.

NOTE 5:	It is up to NWDAF implementation to determine whether the data is taken into account that the UE locates in an area for a shorter time than the Temporal granularity size.

-	[OPTIONAL] Preferred orientation of location information: ("horizontal", "vertical", "both").

-	[OPTIONAL] Preferred order of results when a list of analytics is returned, possibly with a criterion for identifying the property of the results to which the preferred ordering is applied.

-	[OPTIONAL] Maximum number of SUPIs (SUPImax) requested by the consumer to limit the number of SUPIs in an object. When SUPImax is not provided, the NWDAF shall return all SUPIs concerned by the analytics object. When SUPImax is set to 0, the NWDAF shall not provide any SUPI.

-	[OPTIONAL] Output strategy: indicates the relevant factors for determining when the analytics reported. The following values are allowed:

-	Binary output strategy: indicates that the analytics shall only be reported when the preferred level of accuracy is reached within a cycle of periodic notification as defined in the Analytics Reporting Parameters.

NOTE 6:	If preferred level of accuracy is more important than providing an output, then the binary strategy is used so that all analytics outputs have equivalent confidence in the prediction.

-	Gradient output strategy: indicates that the analytics shall be reported according to the periodicity defined in the Analytics Reporting Parameters irrespective if the preferred level of accuracy has been reached.

NOTE 7:	If having an analytics output is more important than reaching the preferred level of accuracy, then the gradient output strategy is used so that each NWDAF will timely provide the output indicating the confidence of the prediction at the moment of the output generation.

NOTE 8:	When no output strategy is included in the subscription, the analytics output will be generated based on the gradient strategy and includes the confidence of the prediction for the reporting period.

-	[OPTIONAL] Analytics metadata request: indicates a request from one NWDAF to another NWDAF to provide the "analytics metadata information" related to the produced output analytics. This input parameter indicates which parameters in "analytics metadata information" are required to aggregate the output analytics for the requested Analytics ID(s).

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Consumer NF's serving area or NF ID. During a pending analytics subscription transfer, this information can be used by the NWDAF to find out if the analytics consumers may change as described in clause 6.1B.2.

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Information of previous analytics subscription. When setting up the analytics generation, this information may be used to retrieve analytics context from the previous NWDAF in order to build upon the context that is already related to this subscription as described in clause 6.1B.2.1.

-	[OPTIONAL] Use case context: indicates the context of use of the analytics to select the most relevant ML model.

NOTE 9:	The NWDAF can use the parameter "Use case context" to select the most relevant ML model, when several ML models are available for the requested Analytics ID(s). NWDAF containing AnLF can additionally provide the parameter "Use case context" when requesting an ML model from an NWDAF containing MTLF. The values of this parameter are not standardized. For example, the AMF can use a given value of "Use case context" when requesting UE Mobility analytics for optimizing the definition of a Registration Area, and a different value of "Use case context" when requesting UE Mobility analytics for determines a paging strategy.

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) [OPTIONAL] Analytics feedback information: indicates that the consumer NF has taken an action(s) influenced by the previously provided analytics, which may or may not affect the ground truth data corresponding to analytic ID requested at the time which the prediction refers to, and consequently affect the ML Model Accuracy monitoring by the subscription with following parameter(s):

-	Corresponding Analytics ID(s) which has been used for taking an action(s);

-	Indication whether the action will affect on ground truth data (if available);

-	Time stamp(s) of the action(s) taken.

NOTE 10:	The consumer NF cannot include Analytics feedback information in initial subscription request. Analytics feedback information can be included in modification request for the existing analytics subscription.

-	[OPTIONAL] Analytics Accuracy Request information with the following parameters:

-	Analytics accuracy request: indicates NWDAF to provide accuracy information to the analytics consumer.

-	[OPTIONAL] Analytics accuracy information time window: time interval [start. end], which indicates that analytics consumers only consider the accuracy information which is generated within this time interval.

-	[OPTIONAL] Analytics accuracy information periodicity: time period, which indicates periodic reporting of accuracy information for the corresponding Analytics ID(s).

-	[OPTIONAL] Analytics Accuracy threshold(s): a reporting threshold of accuracy value, which is indicated that:

-	The NWDAF can provide analytics output and optionally analytics accuracy value to the analytics consumer(s) when the accuracy value is above this Analytics Accuracy threshold;

-	The NWDAF can provide "Stop Analytics Output Consumption indication", "Updated Analytics" or the analytics accuracy information to the analytics consumer(s) when the accuracy value is under this threshold which indicates the deviation of the predictions from the actual network data does not meet analytics accuracy requirement.

-	[OPTIONAL] Minimal number of analytics output occurrences: determines the minimal number of analytics output provided by NWDAF that have to be considered in the determination of the accuracy information.

-	[OPTIONAL] Updated Analytics flag: indicates that the NWDAF can provide updated analytics for provided Analytics ID(s), if updated analytics can be generated within analytics accuracy information time window.

-	[OPTIONAL] Correction time period: a relative time interval as the gap with respect to analytics is provided, which is indicated the time interval during which the updated analytics can be accepted by the analytics consumer.

-	[OPTIONAL] Pause analytics consumption flag: is a flag indicating to NWDAF to stop sending the notifications of analytics outputs for a subscribed analytics ID, without unsubscribing to such analytics ID.

-	[OPTIONAL] Resume Analytics Subscription request: is a flag indicating to NWDAF to resume the notification of analytics outputs for an existing analytics ID(s) subscription(s) that have been previously paused.

-	[OPTIONAL] NF Instance ID(s) or NF Set ID(s) of data source NF(s) from which the data can be collected for generation of the requested analytics.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7, the output information listed below:

-	(Only for Nnwdaf_AnalyticsSubscription_Notify) The Notification Correlation Information.

-	For each Analytics ID, the analytics information in the requested Analytics target period. If the analytics subset is subscribed or requested, then the corresponding analytics information shall be provided.

-	Timestamp of analytics generation: allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period: defines the time period for which the analytics information is valid.

NOTE 11:		Validity period is determined by NWDAF internal logic and it is a subset of Analytics target period.

-	Confidence: probability assertion, i.e. confidence in the prediction.

-	[OPTIONAL] For each Analytics ID the Termination Request, which notifies the consumer that the subscription is requested to be cancelled as the NWDAF can no longer serve this subscription, e.g. due to user consent revoked, NWDAF overload, UE moved out of NWDAF serving area, etc.

-	[OPTIONAL] Analytics metadata information: additional information required to aggregate the output analytics for the requested Analytics ID(s). This parameter shall be provided if the "Analytics metadata request" parameter was provided in the corresponding Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation.

-	Number of data samples used for the generation of the output analytics;

-	Data time window of the data samples;

-	Dataset Statistical Properties of the analytics output used for the generation of the analytics;

-	[OPTIONAL] Data source(s) of the data used for the generation of the output analytics;

-	[OPTIONAL] Data Formatting and Processing applied on the data;

-	Output strategy used for the reporting of the analytics.

-	(Only for error response or error notification) Revised waiting time: indicates to the consumer a revised waiting value for "Time when analytics information is needed". Each NWDAF may include this as part of error response or error notification to "Time when analytics information is needed" as described in clause 6.2.5. Revised waiting time is the minimum time interval recommended by NWDAF to use as "Time when analytics information is needed" for similar future analytics requests/subscriptions.

-	[OPTIONAL] Analytics accuracy information generated for each analytics ID, including:

-	Analytics accuracy value for requested Analytics ID(s): a value shall be provided if "Analytics accuracy request" parameter was provided in the corresponding Nnwdaf_AnalyticsSubscription_Subscribe service operation. This parameter may be provided if the value crosses the analytics accuracy threshold(s) which is indicated in the subscribe request or locally configured, or the analytics accuracy information periodicity indicated in the subscribe request is reached.

-	[OPTIONAL] An indication that the determined accuracy value for the analytics ID does not meet the analytics accuracy threshold requested for the analytics ID.

-	[OPTIONAL] Updated Analytics: NWDAF provides updated Analytics, which is generated within analytics accuracy information time window, for provided Analytics ID(s), if "Updated Analytics flag" parameter was indicated in the corresponding Nnwdaf_AnalyticsSubscription_Subscribe service operation.

-	[OPTIONAL] Stop Analytics Output Consumption indication: NWDAF provides to the consumer an indication to stop the consumption of the Analytics ID(s) related to the subscription ID based on NWDAF internal logic or specified analytics accuracy threshold.

-	[OPTIONAL] Stop Analytics Output Consumption time window: NWDAF provides to the consumer a time window to stop the consumption of the Analytics ID(s) related to the subscription ID based on NWDAF internal logic or specified analytics accuracy threshold.

-	[OPTIONAL] Resume Analytics Output Consumption indication: NWDAF provides to consumer an indication to resume the consumption of analytics output for existing subscription to the analytics ID(s) that was previously paused.

-	[OPTIONAL] Accuracy Information Termination: NWDAF notifies the consumer that the subscription to the accuracy information for an analytics ID has been cancelled as the NWDAF does not support the accuracy checking capability, e.g. as an indication that a new target serving NWDAF supporting accuracy checking capability could not have been selected during the analytics transfer procedures.

NOTE 12:	It is left to Stage 3 to decide whether the Accuracy Information Termination is a cause related to the Termination Request or not.

* * * Next Change * * * *

6.3.1	General

The NWDAF provides slice load level information to a consumer NF on a Network Slice level or a Network Slice instance level or both. The NWDAF is not required to be aware of the current subscribers using the slice. The NWDAF notifies slice specific network status analytics information to the consumer NF that is subscribed to it. A consumer NF may collect directly slice specific network status analytics information from NWDAF. This information is not subscriber specific.

The NWDAF services as defined in the clause 7.2 and clause 7.3 are used to expose slice load level analytics from the NWDAF to the consumer NF (e.g. PCF, NSSF or AMF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "Load level information";

-	Analytics Filter Information:

-	S-NSSAI and NSI ID;

NOTE 1:	The use of NSI ID in the network is optional and depends on the deployment choices of the operator. If used, the NSI ID is associated with S-NSSAI. NSI ID is only applicable when the consumer of analytics is NSSF or AMF.

-	optionally, the list of analytics subsets that are requested among those specified in clause 6.3.3A;

-	optionally, for analytics exposure in roaming case (see clause 6.1.5), the PLMN ID identifying the target PLMN (i.e. PLMN of which the roaming analytics is requested); and

-	optionally, for analytics exposure in roaming case (see clause 6.1.5), mapped S-NSSAI of the HPLMN if the consumer NF is in the VPLMN.

NOTE 2:	The terms "HPLMN" and "VPLMN" here refer to a roaming case in which at least one UE served by the NWDAF analytics consumer is involved.

-	an optional Area of Interest;

-	an optional list of NF types;

-	optionally, Load Level Threshold value;

-	optionally, "maximum number of objects" indicating the maximum number of Network Slice instances expected in output, when the Analytics Filter Information does not indicate an NSI ID; and

-	an Analytics target period indicating the time period over which the statistics or predictions are requested.

-	optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

* * * Next Change * * * *

6.4.1	General

This clause specifies how NWDAF can provide Observed Service Experience (i.e. average of observed Service MoS and/or variance of observed Service MoS indicating service MOS distribution for services such as audio-visual streaming as well as services that are not audio-visual streaming such as V2X and Web Browsing services) analytics, in the form of statistics or predictions, to a service consumer.

The Observed Service Experience analytics may provide one or more of the following outputs:

-	Service Experience for a Network Slice: Service Experience for a UE or a group of UEs or any UE in a Network Slice;

-	Service Experience for an Application: Service Experience for a UE or a group of UEs or any UE in an Application or a set of Applications;

-	Service Experience for an Edge Application over a UP path: Service experience for a UE or a group UEs or any UE in an Application or a set of Applications over a specific UP path (UPF, DNAI and EC server);

-	Service Experience for an Application over a RAT Type or Frequency or both: Service experience for a UE or a group of UEs in an Application or a set of Applications over a RAT Type or over a Frequency or both as defined in Table 6.4.1-1.

-	Service Experience for an Application transferring data over a PDU Session: Service experience for a UE or a group of UEs or any UEs in an Application or a set of Applications transferring data over a PDU Session with PDU Session parameters i.e. S-NSSAI, DNN, PDU Session Type , SSC mode and optionally an Access Type or with combination of PDU Session parameters such as a list of the tuple (PDU Session Type, SSC mode) optionally per Access Type.

Therefore, Observed Service experience may be provided as defined in clause 6.4.3. For example, individually per UE or group of UEs, or globally, averaged per Application or averaged across a set of Applications on a Network Slice.

The service consumer may be an NF (e.g. PCF, NSSF, AMF, NEF), AF, or the OAM.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "Service Experience";

-	Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information as defined in Table 6.4.1-1 and optionally a list of analytics subsets that are requested (see clause 6.4.3);

-	optionally, maximum number of objects and maximum number of SUPIs;

-	optionally, preferred level of accuracy of the analytics;

-	optionally, preferred level of accuracy per analytics subset;

-	optionally, preferred order of results for the list of Application Service Experiences and/or Slice instance service experiences: "ascending" or "descending";

-	optionally, preferred granularity of location information: TA level or cell level or "below cell level";

-	Analytics target period that indicates the time window for which the statistics or predictions are requested;

-	in a subscription, the Notification Correlation Id and the Notification Target Address; and

-	optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the Service Experience to be reached in order to be notified by the NWDAF (see Table 6.4.3-1 and Table 6.4.3-2).

NOTE:	Definition of "below cell level" is described in clause 6.1.3.

-	optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

Table 6.4.1-1: Analytics Filter Information related to the observed service experience

| Information | Description | Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Application | Network Slice | Edge Applications over a UP path | Application over RAT Type and frequency | Application transfering data over a PDU Session |
| Application ID (0...max) | The identification of the application(s) for which the analytics information is subscribed or requested. | Y | N | Y | Y | Y |
| S-NSSAI (NOTE 3) | When requesting Service Experience for a Network Slice: identifies the Network Slice for which analytics information is subscribed or requested. When requesting Service Experience for an Application: identifies the S-NSSAI used to access the application together with the DNN listed below. | N | Y | N | N | C |
| NSI ID(s) | Identifies the Network Slice instance(s) for which analytics information is subscribed or requested. | N | N | N | N | N |
| Area of Interest (NOTE 6) | Identifies the Area (i.e. set of TAIs), as defined in TS 23.501 [2] for which the analytics information is subscribed or requested. | N | N | N | N | N |
| DNN (NOTE 3) | When requesting Service Experience for an Application, this is the DNN to access the application. | N | N | N | N | C |
| DNAI (NOTE 1) | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. | N | N | Y | N | N |
| RAT Type (NOTE 2) | Identifies the RAT type. | N | N | N | Y | N |
| Frequency (NOTE 2) | Identifies the Frequency value(s) (e.g. high, low). | N | N | N | Y | N |
| Application Server Address(es) (NOTE 1) | List of IP address(s)/FQDN(s) of the Application Server(s) that a UE, group of UEs, or 'any UE' has a communication session for which Service Experience Analytic information is requested. | N | N | Y | N | N |
| UPF anchor ID (NOTE 1) (NOTE 4) | Identifies the UPF where a UE has an associated PDU session | N | N | N | N | N |
| PDU Session type (NOTE 3) | Identifies the type of the associated PDU Session | N | N | N | N | C |
| SSC Mode (NOTE 3) | Identifies the SSC Mode selected for the associated PDU Session | N | N | N | N | C |
| Access Type (NOTE 3) | Identifies the Access type of the associated PDU Session | N | N | N | N | C |
| Mapped NSSAI (NOTE 5) | Identifies the mapped NSSAI in the HPLMN. May be used in VPLMN for analytics exposure in roaming case (see clause 6.1.5). | N | N | Y | N | N |
| PLMN ID | Identifies the target PLMN (i.e. PLMN from which the analytics are requested, for analytics exposure in roaming case (see clause 6.1.5). | N | N | Y | N | N |
| NOTE 1: These parameters can be provided when a consumer requires analytics for an edge application over a UP path. NOTE 2: A service consumer can provide either a RAT Type or a Frequency or a specific combination of RAT Type and Frequency. A service consumer can also provide multiple instances of RAT Type or multiple instances of Frequency or multiple combinations of RAT type and Frequency. A service consumer can also provide "any" RAT type indication "any" Frequency value indication or "any" indication for all the RAT type and Frequency value the NWDAF has received for the application. NOTE 3: One or more of these parameters can be provided by the consumer when requesting analytics for an application running over a PDU Session(s). NOTE 4: UPF ID is only needed when the target of NWDAF analytics on Service Experience is a specific UPF. NOTE 5: The terms "HPLMN" and "VPLMN" here refer to a roaming case in which at least one UE served by the NWDAF analytics consumer is involved. NOTE 6: If the request is for fine granularity location information (i.e. with a finer granularity than cell), the AOI may be described as shown in clause 5.5 of TS 23.273 [39]. |  |  |  |  |  |  |

NOTE 1:	A service consumer can use the Area of Interest in order to reduce the amount of signalling that the analytics subscription or request generates.

The NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.4.3.

NWDAF collects the network data from AF (directly or via NEF) and from other 5GC NF(s) in order to calculate and provide statistics and predictions on the observed service experience to a consumer NF or to OAM. When the AF provides Service Experience Information and the Target of Analytics Reporting is one or more UE ID(s), the AF may also provide a Service Experience Contribution Weight with each UE's Service Experience value. The Service Experience Contribution Weight is determined by the AF and indicates the relative importance of each UE's Service Experience. The NWDAF may use the Service Experience Contribution Weight(s) to calculate and provide statistics, confidence values and predictions on the observed service experience to a consumer NF or to OAM.

NOTE 2:	The relative importance of that is conveyed in the Service Experience Contribution Weights is used to indicate the relative importance of the Service Experience value (i.e. MOS). For example, it might be that one the Service Experience of one UE is not very important because the UE is an infrequent user of the service, or the UE does not use all features that are associated with the service. Whereas another UE might be considered important because the UE is a frequent user of the service or a user who uses many features that are associated with the service.

Based on the Analytics Filter information in Table 6.4.1-1 and the Target of Analytics Reporting provided by the service consumer in the analytics subscription or request, NWDAF determines whether service experience analytics should be delivered for:

i)	Application(s);

ii)	Network Slice;

iii)	both Application(s) and Network Slice;

iv)	Edge Applications over a UP path;

v)	Application(s) over RAT Type(s) and/or Frequency value(s);

vi)	Application(s) running over a PDU Session using the following PDU Session parameters or combination of them, i.e. S-NSSAI, DNN, PDU Session type, SSC Mode and optionally per Access Type.

If NWDAF is unable to differentiate based on the analytics subscription or request, it provides service experience analytics for both Application(s) and Network Slice.

If service experience for both Application(s) and Network Slice is desired but the Target of Analytics Reporting or Analytics Filter information values (e.g. Area of Interest) need to be different, separate subscriptions/requests may be provided by the service consumer.

* * * Next Change * * * *

6.6.1	General

With Network Performance Analytics, NWDAF provides either statistics or predictions on the gNB status information, gNB resource usage, communication performance and mobility performance in an Area of Interest; in addition, NWDAF it may provide statistics or predictions on the number of UEs that are located in that Area of Interest.

The service consumer may be an NF (e.g. PCF, NEF, AF), or the OAM.

The consumer of these analytics may indicate in the request:

-	Analytics ID = "Network Performance";

-	Target of Analytics Reporting: either a single UE (SUPI ), or a group of UEs (an Internal Group ID that refers to the group for which the analytics on the number of UEs that are located in the Area of Interest at the time indicated in the Analytics target period is requested) or any UE;

-	Analytics Filter Information:

-	Area of Interest (list of TA or Cells) which restricts the area in focus (mandatory if Target of Analytics Reporting is set to "any UE", optional otherwise);

-	Optionally, Traffic type of interest (overall traffic, GBR traffic or Delay-critical GBR traffic);

NOTE:	If Traffic type of interest is not provided, overall traffic is considered.

-	Optionally, a list of analytics subsets that are requested among those specified in clause 6.6.3;

-	Optionally, a preferred level of accuracy of the analytics;

-	Optionally, preferred level of accuracy per analytics subset (see clause 6.6.3);

-	Optionally, preferred order of results for the list of Network Performance information:

-	ordering criterion: "number of UEs", "communication performance" or "mobility performance";

-	order: ascending or descending;

-	Optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for respective analytics information (see clause 6.6.3) in order to be notified by the NWDAF;

-	An Analytics target period indicates the time period over which the statistics or prediction are requested; and

-	Optionally, maximum number of objects.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, Spatial granularity size (if an Area of Interest is provided) and Temporal granularity size.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.6.3.

* * * Next Change * * * *

6.7.2.1	General

NWDAF supporting UE mobility statistics or predictions shall be able to collect UE mobility related information from NF, OAM and to perform data analytics to provide UE mobility statistics or predictions.

The service consumer may be a NF (e.g. AMF, SMF or AF).

The consumer of these analytics may indicate in the request:

-	Analytics ID = "UE Mobility".

-	Target of Analytics Reporting: a single UE (SUPI) or a group of UEs (an Internal Group ID);

-	Analytics Filter Information optionally containing:

-	Area of Interest (AOI): restricts the scope of the UE mobility analytics to the provided area. If the request is for fine granularity location information (i.e. with a finer granularity than cell), the AOI may be described as shown in clause 5.5 of TS 23.273 [39];

NOTE 1:	For LADN service, the consumer (e.g. SMF) provides the LADN DNN to refer the LADN service area as the AOI.

-	Visited Area(s) of Interest (visited AOI(s)): additional filter to only consider UEs that are currently (i.e. now) in the "AOI" and had previously (i.e. in the "Analytics target period") been in at least one of the Visited AOI(s). If this parameter is provided, the Analytics target period shall be in the past (i.e. supported for statistics only);

-	Linear distance threshold: An event where the UE moves by more than some predefined straight line distance from a previous location as per TS 23.273 [39]. The consumer can provide more than one value of Linear distance threshold.

-	an optional list of analytics subsets that are requested (see clause 6.7.2.3);

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

NOTE 2:	For regular analytics scenarios, the Analytics target period is associated with the Analytics Filter Information = AOI, while for the scenario that Analytics ID=UE Mobility and Analytics Filter Information = (AOI and visited AOI(s)), as described in this clause, the Analytics target period is associated with the visited AOI(s) and to obtain the statistics for those UEs that currently reside in the AOI and had previously (i.e. in the "Analytics target period") been in at least one of the Visited AOI(s).

-	Optionally, maximum number of objects;

-	Preferred level of accuracy of the analytics;

-	Optionally, Preferred level of accuracy per analytics subset (see clause 6.7.2.3);

-	Preferred order of results for the time slot entries: ascending or descending time slot start;

-	Optionally, preferred granularity of location information: TA level or cell level "below cell level";

NOTE 3:	Definition of "below cell level" is described in clause 6.1.3.

-	Optionally, Preferred orientation of location information: ("horizontal", "vertical", "both");

-	Optionally, Spatial granularity size and Temporal granularity size;

-	UE Location order indicator: indicates the NWDAF should derives and provides the UE Mobility analytics for UE Location in time order; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

* * * Next Change * * * *

6.7.3.1	General

In order to support some optimized operations, e.g. customized mobility management, traffic routing handling, RFSP Index Management, QoS improvement or Inactivity Timer optimization, in 5GS, an NWDAF may perform data analytics on UE communication pattern and user plane traffic and provide the analytics results (i.e. UE communication statistics or prediction) to NFs in the 5GC or an AF.

An NWDAF supporting UE Communication Analytics collects per-application communication description from AFs. If consumer NF provides an Application ID, the NWDAF only considers the data from AF, SMF and UPF that corresponds to this application ID. NWDAF may also collect data from AMF.

The consumer of these analytics may indicate in the request:

-	Analytics ID = "UE Communication".

-	Target of Analytics Reporting: a single UE (SUPI) or a group of UEs (an Internal Group ID).

-	Analytics Filter Information optionally including:

-	S-NSSAI;

-	DNN;

-	Application ID;

-	Area of Interest.

-	an optional list of analytics subsets that are requested (see clause 6.7.3.3);

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics.

-	Optional Preferred level of accuracy per analytics subset (see clause 6.7.3.3);

-	Optional preferred order of results for the list of UE Communications:

-	ordering criterion: "start time" or "duration",

-	order: ascending or descending;

-	Optionally, maximum number of objects;

-	Optionally, Spatial granularity size (if an Area of Interest is provided); and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

* * * Next Change * * * *

6.7.5.1	General

This clause defines how to identify a group of UEs or a specific UE with abnormal behaviour, e.g. being misused or hijacked, with the help of NWDAF.

NOTE 1:	The misused or hijacked UEs are UEs in which there are malicious applications running or UEs which have been stolen.

The consumer of this analytics could be a 5GC NF. The 5GC NF subscribes analytics on abnormal behaviour from a NWDAF based on the UE subscription, network configuration or application layer request.

The NWDAF performs data analytics on abnormal behaviour if there is a related subscription and returns exception reports that result from the analysis of the correlations between behavioural variables. The exception reports contain an Exception Level expressed in the form of a scalar value, possibly supplemented by additional measurements.

The consumer of this analytics shall indicate in the request:

-	Analytics ID = "Abnormal behaviour";

-	Target of Analytics Reporting: a single UE, any UE or an Internal Group Identifier;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Analytics Filter Information optionally including:

-	expected UE behaviour parameters;

-	expected analytics type or list of Exception IDs with associated thresholds for the Exception Level, where the expected analytics type can be mobility related, communication related or both;

-	Area of interest;

-	Application ID;

-	DNN;

-	S-NSSAI.

NOTE 2:	The expected analytics type generally indicates whether mobility or communication related abnormal behaviour analytics or both are expected by the consumer and the list of exception IDs indicates what specific analytics are expected by the consumer. Either the expected analytics type or the list of Exception IDs needs to be indicated, but they are not presented simultaneously. When the expected analytics type is indicated, the NWDAF performs corresponding abnormal behaviour analytics which are supported by the NWDAF. The relation between the expected analytics type and Exception IDs is defined in Table 6.7.5.1-1.

-	Optionally, maximum number of objects and maximum number of SUPIs;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

Table 6.7.5.1-1: Relation between expected analytics type and Exception IDs

| Expected analytics type | Exception IDs matching the expected analytics type |
| --- | --- |
| mobility related | Unexpected UE location, Ping-ponging across neighbouring cells, Unexpected wakeup, Unexpected radio link failures. |
| communication related | Unexpected long-live/large rate flows, Unexpected wakeup, Suspicion of DDoS attack, Wrong destination address, Too frequent Service Access. |

If the Target of Analytics Reporting is any UE, then the Analytics Filter should at least include:

-	Area of Interest or S-NSSAI, if the expected analytics type or the list of Exception IDs is mobility related.

-	Area of Interest, application ID, DNN or S-NSSAI, if the expected analytics type or the list of Exception IDs is communication related.

If the Target of Analytics Reporting is any UE, the consumer of this analytics shall request either mobility related only or communication related only abnormal behaviour analytics, but not both at the same time.

The expected UE behaviour parameters that the consumer can indicate in the request when known depend on the Exception ID that the consumer expects. They may encompass UE behaviour parameters as defined in clause 4.15.6.3 of TS 23.502 [3] and other parameters. Table 6.7.5.1-2 shows the mapping between each Exception ID and UE behaviour parameters.

Table 6.7.5.1-2: Description of Expected UE Behaviour parameters per Exception ID

| Exception ID | UE behaviour parameters to provide |
| --- | --- |
| Unexpected UE location | Expected UE Moving Trajectory Stationary Indication |
| Unexpected long-live/large rate flows | Periodic Time Scheduled Communication Time Communication Duration Time |
| Unexpected wakeup | Periodic Time Communication Duration Time Scheduled Communication Time |
| Suspicion of DDoS attack | Periodic Time Communication Duration Time Scheduled Communication Time Scheduled Communication Type Traffic Profile Expected transaction Dispersion |
| Too frequent Service Access | Periodic Time |
| Unexpected radio link failures | Expected UE Moving Trajectory |
| Ping-ponging across neighbouring cells | Expected UE Moving Trajectory Stationary Indication |

When the NWDAF detects those UEs that deviate from the expected UE behaviour, e.g. unexpected UE location, abnormal traffic pattern, unexpected transaction dispersion amount, wrong destination address, etc. the NWDAF shall notify the result of the analytics to the consumer as specified in clause 6.7.5.3.

* * * Next Change * * * *

6.8.1	General

The NWDAF can provide user data congestion related analytics, by one-time reporting or continuous reporting, in the form of statistics or predictions or both, to another NF. User Data Congestion related analytics can relate to congestion experienced while transferring user data over the control plane or user plane or both. A request for user data congestion analytics relates to a specific area or to a specific user. If the consumer of these analytics provides a UE ID, the NWDAF determines the area where the UE is located. The NWDAF then collects measurements per cell and uses the measurements to determine user data congestion analytics.

The request for user data congestion related analytics indicates the location area information where congestion related analytics is desired or indicates a UE Identity that can be used by the NWDAF to determine the location area information where congestion related analytics is desired. When requesting user data congestion, the consumer may request the identifiers of the applications that contribute the most to the traffic in the area. The consumer may indicate how many applications should be reported by providing the maximum number of applications in the request or subscription.

When the consumer of user data congestion related analytics subscribes to user data congestion related analytics, it may indicate a threshold and the NWDAF will provide analytics to the consumer when the congestion level crosses the threshold. The consumer can indicate an S-NSSAI in the request when congestion analytics are needed on a per slice level.

The service consumer may be an NF (e.g. NEF, AF, PCF).

The consumer of these analytics may indicate in the request or subscription the following parameters, its content is described in the clause 6.1.3:

-	Analytics ID = "User Data Congestion";

-	Target of Analytics Reporting: either a single UE (SUPI), or "any UE";

NOTE:	The Target of Analytics Reporting set to "any UE" applies when user data congestion analytics relates to a specific Area of Interest.

-	Analytics Filter Information:

-	Area of Interest (i.e. list of TAIs or Cell IDs) which restricts the area in focus (mandatory if Target of Analytics Reporting is set to "any UE", optional otherwise);

-	an optional list of analytics subsets that are requested, (see clause 6.8.3);

-	Optional S-NSSAI, in order to obtain congestion analytics only on a given slice;

-	Optional Reporting Threshold, which applies only for subscriptions and indicates conditions on the congestion level (Network Status Indication, see clause 6.8.3) to be reached in order to be notified by the NWDAF.

-	Preferred level of accuracy of the analytics;

-	Preferred order of results for the list of User Data Congestion statistics or predictions:

-	ordering by Applicable Time Window, chronological or reverse chronological order; or

-	ordering by Network Status Indication, ascending or descending;

-	Optional maximum number of objects;

-	An Analytics target period indicates the time period over which the statistics or prediction are requested, either in the past or in the future;

-	Optionally, Temporal granularity size; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

The NWDAF notifies the result of the analytics to the consumer as indicated in clause 6.8.3.

* * * Next Change * * * *

6.9.1	General

The consumer of QoS Sustainability analytics may request the NWDAF analytics information regarding the QoS change statistics for an Analytics target period in the past in a certain area or the likelihood of a QoS change for an Analytics target period in the future in a certain area. The consumer can request either to subscribe to notifications (i.e. a Subscribe-Notify model) or to a single notification (i.e. a Request-Response model).

The service consumer may be a NF (e.g. AF).

The request includes the following parameters:

-	Analytics ID = "QoS Sustainability";

-	Target of Analytics Reporting: any UE;

-	Analytics Filter Information:

-	QoS requirements (mandatory):

-	5QI (standardized or pre-configured) and applicable additional QoS parameters and the corresponding values (conditional, i.e. it is needed for GBR 5QIs to know the GFBR); or

-	the QoS Characteristics attributes including Resource Type, PDB, PER and their values;

-	Location information (mandatory): an Area Of Interest or a path of interest. The location information could reflect a list of waypoints:

-	if the location information is an Area Of Interest, the area can be either described in a coarse granularity as list of TAIs or Cell IDs, or in a fine granularity as geographical area (that can be smaller than a cell), or both (coarse and fine granularity); if both granularities are provided, the NWDAF understands that the area of interest is the intersection between the fine granularity location and the list of TAIs or Cell IDs.

-	if the location information is a path of interest, the area can be either described in a coarse granularity as list of TAIs or Cell IDs, or in a fine granularity as a list of waypoints (expressed as longitude and latitude in geographical coordinates) and combined with a radius value, or both (coarse and fine granularity); if both granularities are provided, the NWDAF understands that the path of interest is the intersection between the fine granularity location and the list of TAIs or Cell IDs.

-	Threshold linear distance: The distance travelled by the UE before reporting subsequent location as described in TS 23.273 [39].

NOTE 1:	Threshold linear distance is used by the NWDAF when requesting location of the UE from the GMLC.

NOTE 2:	In this Release, the consumer of the "QoS Sustainability" Analytics ID will provide location information in the area of interest format (TAIs or Cell IDs or geographical area) which is understandable by NWDAF.

NOTE 3:	When location information is described as an area of interest in a fine granularity (i.e. as geographical area that can be smaller than a cell) the Cell ID(s) can already be determined by the NEF based on its local configuration and provided to the NWDAF in addition. This addresses the scenario when Cell ID(s) cannot be determined by the NWDAF based on its local configuration. Alternatively, the NWDAF can also be configured with the Cell ID(s) corresponding to an area of interest in a fine granularity.

-	S-NSSAI (optional);

-	Optional maximum number of objects;

-	Optional UE Device and Context Information: which may contain one or more of the following:

-	Speed range, which is a range of UE speeds for which analytics is requested, where the speed range is indicated as a range of Velocity Estimate, as in clause 6.1.6.2.17 of TS 29.572 [37];

-	Device information, which may contain one of the following:

-	List of equipment types, according to clause 8.0 of GSMA TS.06 [38].

-	Analytics target period: relative time interval, either in the past or in the future, that indicates the time period for which the QoS Sustainability analytics is requested;

-	Optionally, Spatial granularity size and Temporal granularity size;

-	Reporting Threshold(s), which apply only for subscriptions and indicate conditions on the level to be reached for the reporting of the analytics, i.e. to discretize the output analytics and to trigger the notification when the threshold(s) provided in the analytics subscription are crossed by the expected QoS KPIs.

-	A matching direction may be provided such as crossed (default value), below, or above.

-	An acceptable deviation from the threshold level in the non-critical direction (i.e. in which the QoS is improving) may be set to limit the amount of signalling.

The level(s) relate to value(s) of the QoS KPIs defined in TS 28.554 [10], for the relevant 5QI:

-	for a 5QI of GBR resource type, the Reporting Threshold(s) refer to the QoS flow Retainability KPI;

-	for a 5QI of non-GBR resource type, the Reporting Threshold(s) refer to the RAN UE Throughput KPI and/or delay in RAN KPI as defined in TS 28.554 [10].

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

To derive the QoS Sustainability analytics when the location information is an area of interest with coarse granularity (i.e. TAIs or Cell IDs):

-	The NWDAF collects the corresponding statistics information on the QoS KPI for the relevant 5QI of interests from the OAM, i.e. the QoS flow Retainability or the RAN UE Throughput or delay in RAN as defined in TS 28.554 [10] and average GTP metrics as defined in TS 28.552 [8].

To derive the QoS Sustainability analytics when the location information is an area of interest with fine granularity (i.e. geographical area that can be smaller than a cell):

-	The NWDAF derives the UE list for the area of interest with fine granularity by two steps, firstly based on an initial selection of the UE list in the corresponding coarse area from AMF, secondly based on finer granularity location data from LCS, e.g. NWDAF can collect finer granularity location data of a UE from LCS and identify whether this UE is inside of the area of interest with fine granularity area, input data as defined in Table 6.9.2-2.

-	The NWDAF can then collect the corresponding UE level information on the QoS KPI for the relevant 5QI of interests from the 5GC NF/OAM, i.e. input data as defined in Table 6.9.2-2.

-	NWDAF derives QoS sustainability statistics or predictions for the area of interest with fine granularity by averaging these input data for all the UEs that are in the UE list.

To improve QoS Sustainability analytics, the NWDAF may additionally collect GTP metrics defined in Table 6.9.2-3.

If the Analytics target period refers to the past:

-	The NWDAF verifies whether the triggering conditions for the notification of QoS change statistics are met and if so, generates for the consumer one or more notifications.

-	The analytics feedback contains the information on the location and the time for the QoS change statistics and the Reporting Threshold(s) that were crossed.

If the Analytics target period is in the future:

-	The NWDAF detects the need for notification about a potential QoS change based on comparing the expected values for the KPI of the target 5QI against the Reporting Threshold(s) provided by the consumer in any cell in the requested area for the requested Analytics target period. The expected KPI values are derived from the statistics for the 5QI obtained from OAM. OAM information may also include planned or unplanned outages detection and other information that is not in scope for 3GPP to discuss in detail.

-	The analytics feedback contains the information on the location and the time when a potential QoS change may occur and what Reporting Threshold(s) may be crossed.

* * * Next Change * * * *

6.10.1	General

Dispersion analytics identifies the location (i.e. areas of interest, TAs, cells) or network slice(s) where a UE, or a group of UEs, or any UE disperse most (if not all) of their data volume and sessions transactions (i.e. MM and SM messages).

The NWDAF shall be able to provide dispersion statistics or predictions and shall be able to collect UE dispersion related information from NFs.

Dispersion is the percentage of activity that a UE, or group of UEs, or any UE, generates at a location or a slice during a period of interest. This enables the operator to rank hot locations or slices by the various activities and to identify the top contributors (i.e. UEs) for that activity. When heavy UEs at a particular location or slice are detected/predicted and reported to a policy entity, their QoS attributes, or access to an area of interest or a slice can then be limited.

-	Data volume dispersion - The percentage of data traffic volume that a UE, or a group of UEs, or any UE, generated at a location or in a slice during the period of interest.

-	Transactions dispersion - The percentage of MM and SM messages that a UE, or a group of UEs, or any UE, generated at a location or in a slice during the period of interest.

The operator may classify and assign one of the three mobility classes per dispersion characteristics for a UE or a group of UEs, or any UE as fixed, camper or traveller. In addition, the operator may classify a UE as a Top-Heavy UE in case its dispersion percentile rating, at a location or a slice, is higher than a threshold value defined by the operator (e.g. 0.9 percentile), see percentile ranking in the output tables, clause 6.10.3. The classification is based on either Data volume dispersion or Transactions dispersion.

-	Data-Classification - fixed, camper, traveller data-classification per thresholds assigned by the operator.

EXAMPLE:	When a UE disperses, during the period of observation, most (threshold=95%) of its data at a location or a slice, the dispersion data-classification of the UE at that location or slice is "fixed".

-	Transaction-Classification - fixed, camper, traveller transactions-classifications per thresholds assigned by the operator.

EXAMPLE:	When a UE disperses, during the period of observation, (threshold=40%) of its session transactions at a location or a slice, the dispersion transaction-classification of the UE is camper at that location or slice.

Based on the dispersion analytics, the analytics consumer can determine that a data volume or transactions hot spot is formed when the volume of data or amount of transactions dispersed by most of the UEs at the area of interest or slice exceeds a certain data or transaction threshold established through trend analysis or operator policy.

The consumer of dispersion analytics may indicate in its request:

-	Analytics ID = "Dispersion Analytics";

-	Target of Analytics Reporting: a single UE (SUPI), or a group of UEs (an Internal Group ID), or any UE. "Any UE" is only supported in combination with Analytics Filter Information (S-NSSAI, Area(s) of Interest, Top-Heavy UEs indication, Fixed indication, Camper indication or Traveller indication) and Dispersion Analytics type "Data Volume Dispersion Analytics" and/or "Transaction Dispersion Analytics";

-	Analytics Filter Information:

-	optional list of TA(s), Area(s) of Interest, Cells, or S-NSSAI(s), Application ID(s), Top-Heavy UEs, Fixed UEs, Camper UEs, Dispersion Analytics type (Data Volume Dispersion Analytics (DVDA) or Transactions Dispersion Analytics (TDA) or both); and

-	optional list of analytics subsets that are requested (see clause 6.10.3);

-	Preferred level of accuracy of the analytics;

-	Preferred level of accuracy per analytics subset (see clause 6.10.3);

-	Preferred order of results for the list of UE Dispersion Analytics information:

-	ordering criterion:

-	For DDA, one of the following: "time slot start", "data dispersion", "data classification", "data ranking", or "data percentile ranking";

-	For TDA, one of the following: "time slot start", "transaction dispersion", "transaction classification", "transaction ranking", or "transaction percentile ranking"; and

-	order: ascending or descending;

-	Analytics target period indicating the time period over which the statistics or predictions are requested;

-	Optionally, maximum number of objects and maximum number of SUPIs;

-	Optionally, preferred granularity of location information: TA level or cell level;

-	Optionally, Spatial granularity size and Temporal granularity size; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

NOTE 1:	If one or multiple ID(s) are included in the filter information, the output analytics will be reported per Application ID, otherwise the output analytics is aggregated information per AOI or slice independent of the applications.

NOTE 2:	Care needs to be taken with regards to load when requesting analytics for "Any UE". This could be achieved by utilizing event filters (e.g. Area of Interest for AMF) and possible analytics filters including Top-Heavy UEs and/or Fixed and/or Camper UEs and/or Analytics Reporting Information (e.g. SUPImax), or sampling ratio as part of Event Reporting Information. The load of analytics for "Any UE" can be alleviated by limiting the number of S-NSSAI in the analytics filter.

* * * Next Change * * * *

6.11.1	General

The NWDAF provides WLAN performance analytics to a service consumer NF. The analytics results are generated based on the data from other 5GC NFs and OAM. The analytics results, provided in the form of statistics or predictions, contain quality and performance of WLAN connection of UE according to UE location and SSID. The consumer can request either one-time or continuous reporting.

The service consumer may be an NF (e.g. PCF, NEF or AF).

If a service consumer is PCF, the WLAN performance analytics can be used to update WLANSP as defined in TS 23.503 [4].

If the service consumer is NEF or AF, the WLAN performance analytics per UE granularity can be used to select candidate members for an application layer operation.

The consumer of these analytics may indicate in the request or subscription:

-	Analytics ID = "WLAN performance";

-	Target of Analytics Reporting: a single UE (SUPI), a group of UEs (an Internal Group ID), or any UE;

-	Analytics Filter Information:

-	Area of Interest (list of TA or Cells);

-	SSID(s);

-	BSSID(s); and

-	optional list of analytics subsets that are requested (see clause 6.11.3);

-	An Analytics target period indicates the time period over which the statistics or prediction are requested, either in the past or in the future;

-	Optionally, Temporal granularity size;

-	Optional maximum number of objects;

-	Preferred level of accuracy of the analytics;

-	Preferred level of accuracy per analytics subset (see clause 6.11.3);

-	Preferred order of results for the list of WLAN performance information:

-	ordering criterion: "time slot start", "number of UEs", "RSSI", "RTT" or "Traffic information"; and

-	order: ascending or descending; and

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

If the Target of Analytics Reporting is any UE, then the Analytics Filter should at least include Area of Interest or SSID(s) or BSSID(s).

* * * Next Change * * * *

6.12.1	General

According to the Session Management Congestion Control (SMCC) mechanisms, i.e. DNN based congestion control defined in clause 5.19.7.3 of TS 23.501 [2] and S-NSSAI based congestion control defined in clause 5.19.7.4 of TS 23.501 [2], the SMF that is applying or has applied the SMCC mechanism does not store any historical information related to which UEs is/was subject to backoff timer settings. Therefore, fairness to apply the SMCC cannot be considered nor guaranteed. For example, among UEs that use a PDU Session associated with S-NSSAI#1, some of them may have experienced the S-NSSAI based high level of congestion control (e.g. receiving NAS SM reject messages with a long backoff timer) while some of the UEs may have experienced the S-NSSAI based low level congestion control (e.g. receiving NAS SM reject messages with a short backoff timer), within a specific period. The backoff timer provided to each UE can vary, e.g. up to 70 hours.

The service consumer (e.g. SMF, PCF) may request the NWDAF to provide Session Management Congestion Control Experience (SMCCE) statistical analytics for a specific DNN and/or S-NSSAI. The SMF uses potential congestion condition as a trigger to request the SMCCE analytics from the NWDAF.

The request by the SMF includes mainly the following parameters:

-	Analytics ID = "Session Management Congestion Control Experience".

-	Target of Analytics Reporting: one or more SUPI(s).

NOTE 1:	The UE(s) contained in the Target of Analytics Reporting. These are UE(s) that have the PDU Session for the DNN and/or S-NSSAI indicated by Analytics Filter Information.

-	Analytics Filter Information:

-	DNN and/or S-NSSAI; and

-	optional list of analytics subsets that are requested (see clause 6.12.3);

-	Analytics target period: the time window for which the statistics are requested.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

NOTE 2:	Predictions are not provided as output for the Session Management Congestion Control Experience analytics.

* * * Next Change * * * *

6.13.1	General

This clause describes the Redundant Transmission Experience related analytics. These analytics may be used as follows:

-	by the SMF to determine whether redundant transmission on N3/N9 interfaces (see clause 5.33.2.2 of TS 23.501 [2]) shall be performed, or (if it had been activated) shall be stopped;

-	by the PCF for the calculation of the Route Selection Components in a URSP Rule for redundant PDU Sessions as described in clause 5.33.2.1 of TS 23.501 [2].

The service consumer may be a NF (e.g. SMF, PCF).

The consumer of these analytics may indicate in the request:

-	Analytics ID = "Redundant Transmission Experience".

-	Target of Analytics Reporting: a single UE (SUPI), any UE, or a group of UEs (an Internal Group ID).

-	Analytics Filter Information optionally containing:

-	Area of Interest;

-	S-NSSAI;

-	DNN;

-	optional list of analytics subsets that are requested (i.e. UL/DL packet drop rate GTP-U, UL/DL packet delay GTP-U, see clause 6.13.3).

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Optionally, Temporal granularity size;

-	Preferred level of accuracy of the analytics;

-	Preferred order of results for the list of Redundant Transmission Experience:

-	ordering criterion: "time slot start"; and

-	order: ascending or descending;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

* * * Next Change * * * *

6.14.1	General

This clause specifies how an NWDAF can provide DN Performance Analytics which provides analytics for user plane performance (i.e. average/maximum traffic rate, average/maximum packet delay, average packet loss rate) in the form of statistics or predictions to a service consumer.

The DN Performance Analytics may provide one or a combination of the following information:

-	User plane performance analytics for a specific Edge Computing application for a UE, group of UEs, or any UE over a specific serving anchor UPF.

-	User plane performance analytics for a specific Edge Computing application for a UE, group of UEs, or any UE over a specific DNAI.

-	User plane performance analytics for a specific Edge Computing application for a UE, group of UEs, or any UE over a specific Edge Application Server Instance.

The service consumer may be an NF (e.g. SMF) or an AF.

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "DN Performance";

-	Target of Analytics Reporting: one or more SUPI(s) or Internal Group Identifier(s), or "any UE";

-	Analytics Filter Information as defined in table 6.14.1-1 and optionally a list of analytics subsets that are requested (see clause 6.14.3); and

-	optionally, a preferred level of accuracy of the analytics;

-	optionally, preferred level of accuracy per analytics subset (see clause 6.14.3);

-	optionally, preferred order of results for the list of Network Performance information:

-	ordering criterion: one of the analytics subset (see clause 6.14.3);

-	order: ascending or descending;

-	optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for respective analytics subsets (see clause 6.14.3) in order to be notified by the NWDAF;

-	optionally, maximum number of objects and maximum number of SUPIs;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested; and

-	Optionally, Spatial granularity size and Temporal granularity size.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

Table 6.14.1-1: Analytics Filter Information related to DN Performance Analytics

| Information | Description |
| --- | --- |
| Application ID (0..max) | The identification of the application(s) for which the analytics information is subscribed or requested. |
| S-NSSAI | Identifies the Network Slice for which analytics information is subscribed or requested. |
| NSI ID(s) | Identifies the Network Slice instance(s) for which analytics information is subscribed or requested. |
| Area of Interest | Identifies the Area (i.e. set of TAIs), as defined in TS 23.501 [2] for which the analytics information is subscribed or requested. |
| Anchor UPF info | Identifies the UPF where a UE has an associated PDU session. |
| DNN | DNN to access the application. |
| DNAI | The UPF ID/address/FQDN information identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| Application Server Address(es) | List of IP address(s)/FQDN(s) of the Application Server(s) that a UE, group of UEs, or 'any UE' has a communication session with for which DN Performance Analytic information is requested. |
| List of analytics subsets | List of analytics subsets that are requested among those specified in clause 6.14.3. |
| NOTE: All parameters are optional. |  |

* * * Next Change * * * *

6.16.1	General

This clause specifies the procedure on how NWDAF can provide NWDAF-assisted PFD Determination analytics, in the form of statistics.

To assist determination of PFDs for known application identifiers, if the related Service Level Agreement does not preclude, an NWDAF may perform data analytics on existing PFD information and user plane traffic and provide analytics results in the form of new or updated PFDs, e.g. IP 3-tuple list in PFD is new or updated, to an analytics consumer in the 5GC. The NEF(PFDF) as the consumer may forward new or updated PFD information provided by the NWDAF to the UPF via the SMF to detect a known application, as defined in TS 23.502 [3].

The service consumer may be a NEF(PFDF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = " PFD Determination";

-	Target of Analytics Reporting: "any UE";

-	Application identifier;

-	Analytics Filter Information optionally containing:

-	S-NSSAI;

-	DNN;

-	An Analytics target period indicates the time period over which the statistics are requested;

-	Optionally, the preferred level of accuracy on the PFD statistics;

-	Optionally, maximum number of objects. This refers to the analytic output (i.e. maximum number of new or updated PFDs);

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

* * * Next Change * * * *

6.17.1	General

This clause specifies how an NWDAF can provide Location Accuracy Analytics which provides analytics for LCS QoS accuracy to an analytics consumer such as LMF.

With different input data to NWDAF, the Location Accuracy Analytics provides one or a combination of the following information:

-	when the input data is location:

-	location accuracy (i.e. Horizontal Accuracy (see clause 4.3.1 of TS 22.071 [35]) and optionally Vertical Accuracy (see clause 4.3.2 of TS 22.071 [35]) of applicable positioning method at indicated location;

-	Indication on whether a UE is indoor or outdoor when a location measurement is made (optional);

-	Indication on whether a UE location is measured with NLOS or LOS (optional);

-	when the input data is area:

-	average location accuracy (i.e. Horizontal Accuracy (see clause 4.3.1 of TS 22.071 [35]) and optionally Vertical Accuracy (see clause 4.3.2 of TS 22.071 [35]) of applicable positioning method at indicated area;

-	percentage of indoor/outdoor measurement for an applicable area (optional);

-	percentage of NLOS/LOS measurement for an applicable area (optional);

-	requested Confidence of location accuracy prediction (optional).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "Location Accuracy";

-	Target of Analytics Reporting: "any UE";

-	Analytics Filter Information:

-	Optionally, the Positioning methods used as defined in clause 4.2b of TS 23.273 [39];

-	Location information (horizontal and optional vertical); or

-	Area of Interest: restricts the scope of the LCS accuracy analytics to the provided area. The AOI is TA and/or cell list as defined in TS 23.502 [3]. Depends on the NWDAF consumer, there can be more than one AOIs in one request;

-	Optionally, the list of analytics subsets that are requested among those specified in clause 6.17.3;

-	optionally, a preferred level of accuracy of the analytics;

-	optionally, preferred level of accuracy per analytics subset (see clause 6.17.3);

-	optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the level to be reached for respective analytics subsets (see clause 6.17.3) in order to be notified by the NWDAF;

-	optionally, maximum number of objects; and

-	an Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

* * * Next Change * * * *

6.18.1	General

Clause 6.18 describes how NWDAF can provide E2E data volume transfer time analytics, in the form of statistics or predictions or both, to a service consumer. NWDAF collects E2E data volume transfer time related input data from 5GC NFs, OAM and AF. The consumer can either subscribe to analytics notifications (i.e. a Subscribe-Notify model) or request a single notification (i.e. a Request-Response model).

The E2E data volume transfer time refers to a time delay for completing the transmission of a specific data volume from UE to AF, or from AF to UE. If a target number of repeating data transmissions or a target time interval between data transmissions is given, the E2E data volume transfer time can be provided as an average value of every data volume transfer time within the Analytics target period. The E2E data volume transfer time analytics may be used to assist an AF hosting AI/ML-based services, e.g. for member selection of federated learning.

The E2E data volume transfer time analytics may be provided as defined in clause 6.18.3 for a UE individually or a list of UEs.

More than one E2E data volume transfer time classes might be assigned by operator or AF for a group of UEs. The UEs might be classified to high-, medium- and low-transfer time classes with respect to the threshold(s) of the corresponding class.

The service consumer may be an NF (e.g. AF, or NEF).

The consumer of these analytics may indicate in the request:

-	Analytics ID = "E2E data volume transfer time ".

-	Target of Analytics Reporting: a single UE (SUPI/GPSI) or a group of UEs (a list of SUPIs/GPSIs).

-	Analytics Filter Information optionally including:

-	DNN;

-	S-NSSAI;

-	Application ID;

-	Area of Interest (AOI(s)): restricts the scope of the E2E data volume transfer time analytics to the provided area;

-	An optional list of analytics subsets that are requested (see clause 6.18.3);

-	Data Volume UL/DL: indicates a specific data volume transmitted once from UE to AF and/or from AF to UE;

-	QoS requirements (e.g. 5QI, QoS Characteristics);

-	Optionally, either a target number of repeating data transmissions or a target time interval between data transmissions within the Analytics target period;

-	Optionally, a request for geographical distribution (i.e. the AoIs) of the UEs.

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, preferred level of accuracy of the analytics.

-	Optionally, preferred level of accuracy per analytics subset (see clause 6.18.3).

-	Optional preferred order of results for the list of E2E data volume transfer time:

-	ordering criterion: "E2E data volume transfer time",

-	order: ascending or descending.

-	Optionally, Reporting Thresholds, which indicate conditions on the level to be reached for respective analytics subsets (see clause 6.18.3)

-	Optionally, maximum number of UEs.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

* * * Next Change * * * *

6.19.1	General

Relative Proximity Analytics among UEs provided by NWDAF can be used to assist a consumer NF to more accurately localize a cluster (or a set) of UEs via provisioning statistics or prediction information related to their relative proximity. This analytics type may help the consumer improve the location estimation accuracy of a UE by using proximity information from nearby UEs, or it may help the consumer identify UEs in the vicinity of another UE. Relative proximity information can also be leveraged by NWDAF to provide location information with finer granularity than TA/cell.

The NWDAF provides relativity proximity analytics to a NF (e.g. NEF or AF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "Relative Proximity";

-	Target of Analytics Reporting: a UE, a group of UEs;

-	Analytics Filter Information:

-	optionally, S-NSSAI and/or DNN

-	an optional Area of Interest;

-	an optional individual or set of direction(s) of interest;

-	an optional number of UEs to be accounted for relative proximity (i.e. the number of UEs for which one UE may report proximity information);

-	optionally, one or several attributes to be accounted for relative proximity (i.e. additional information that can be provided in addition to distance between two UEs): velocity, average speed, orientation, mobility trajectory;

-	Optionally, the list of analytics subsets that are requested among those specified in clause 6.19.3;

-	Optionally, preferred level of accuracy of the analytics;

-	Optionally, preferred level of accuracy per analytics subset (see clause 6.19.3);

-	Optionally, "maximum number of objects" indicating the maximum number of UEs

-	An Analytics target period indicating the time period over which the statistics or predictions are requested.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

* * * Next Change * * * *

6.20.1	General

This clause specifies the procedure for an NWDAF to provide statistics on whether traffic of UEs via one or multiple PDU sessions is according to the information provide by the service consumer.

The NWDAF collects traffic flow information of UE traffic via PDU session(s) established for a specific S-NSSAI and/or DNN and provides statistics of UEs that route traffic according to the information provided by the service consumer (e.g., Traffic Descriptor, S-NSSAI, DNN) and UEs that route traffic which is not expected according to the information provided by the service consumer (e.g., Traffic Descriptor, S-NSSAI, DNN).

The assumption is that Traffic Descriptors are known (e.g. known flow description), so that there are associated Packet Detection Rule(s) for the known traffic configured in the UPF.

NOTE 1:	How traffic from different applications over the same S-NSSAI/DNN and/or PDU session is discerned is implementation-specific.

The service consumer may be a PCF.

The consumer of these analytics includes in the request:

-	Analytics ID = "PDU Session traffic".

-	Target of Analytics Reporting: a SUPI or a list of SUPIs or an Internal-Group-Id or any UE.

-	Traffic Descriptor: Application Identifier, IP Descriptions or Domain Descriptors are applicable.

NOTE 2:	The Application Id provided by the consumer is known by the network, i.e. corresponds to an Application Id known at the UPF.

-	Analytics Filter Information containing:

-	Area of Interest (i.e. the location of UEs where PDU Session traffic is monitored);

-	S-NSSAI;

-	DNN;

-	an optional list of analytics subsets that are requested (see clause 6.20.3);

-	Optionally, preferred level of accuracy of the analytics.

-	Optionally, preferred level of accuracy per analytics subset (see clause 6.20.3);

-	An Analytics target period indicates the time period over which the analytics are requested.

-	Optionally, maximum number of objects.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

* * * Next Change * * * *

6.21.1	General

The movement behaviour analytics provides analytics information (statistics or predictions) regarding the location, direction and velocity of UEs during an analytics target period in a target area. The statistics of movement behaviour collects a map of UE movements in a certain region in a certain period of time. In addition, the consumers can obtain the congestion situation in the target area through statistics.

The NWDAF provides movement behaviour analytics to a NF (e.g. NEF, AF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics ID = "movement behaviour";

-	Target of Analytics Reporting: any UE;

-	Analytics Filter Information:

-	Area of Interest (AOI): restricts the scope of the movement behaviour analytics to the provided area. The AOI may be described as shown in clause 5.5 of TS 23.273 [39];

-	Optionally, the list of analytics subsets that are requested among those specified in clause 6.21.3;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, preferred level of accuracy of the analytics;

-	Preferred level of accuracy per analytics subset (see clause 6.21.3);

-	Optionally, preferred granularity of location information: "longitude and latitude level";

-	Optionally, preferred orientation of location information: ("horizontal", "vertical", "both")"; and

-	Optionally, maximum number of objects.

-	Optionally, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

* * * Next Change * * * *

7.2.2	Nnwdaf_AnalyticsSubscription_Subscribe service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Subscribe.

Description: Subscribes to NWDAF analytics and optionally analytics accuracy information with specific parameters.

Inputs, Required:

-	(Set of) Analytics ID(s) as defined in Table 7.1-2;

-	Target of Analytics Reporting;

-	Notification Target Address (+ Notification Correlation ID);

-	Analytics Reporting Parameters (including Analytics target period, etc.).

NOTE 1:	Target of Analytics Reporting can be provided per individual Analytics ID.

Inputs, Optional:

-	Analytics Filter Information;

-	Time window for historical analytics;

-	Subscription Correlation ID (in the case of modification of the analytics subscription);

-	preferred level of accuracy of the analytics;

-	preferred level of accuracy per analytics subset;

-	Reporting Thresholds;

-	maximum number of objects requested (max);

-	preferred order of results, maximum number of SUPIs requested (SUPImax);

-	time when analytics information is needed;

-	Analytics Metadata Request;

-	(Set of) NWDAF identifiers used by the NWDAF service consumer when aggregating multiple analytic subscriptions;

-	Dataset Statistical Properties;

-	Output strategy;

-	Data time window;

-	consumer NF's serving area or NF ID;

-	information of previous analytics subscription, i.e. NWDAF identifier (i.e. Instance ID or Set ID), Analytics ID(s) (including SUPIs and Analytics Filter Information for UE-related Analytics) and Subscription Correlation ID;

-	Use case context.

-	Analytics Accuracy Request information.

-	Analytics feedback information.

-	NF Instance ID(s) or NF Set ID(s) of data source NF(s).

NOTE 2:	Analytics Filter Information, Reporting Thresholds, maximum number of objects requested (max), maximum number of SUPIs requested (SUPImax), Analytics Metadata Request, Dataset Statistical Properties, Output strategy, Data time window and time when analytics information is needed can be provided per individual Analytics ID.

NOTE 3:	Analytics feedback information only can be included in modification request for the existing analytics subscription.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: First corresponding Analytics report is included, if available and if analytics consumer requested immediate reporting (see clause 4.15.1 of TS 23.502 [3]).

NOTE 4:	When the Target of Analytics Reporting is a SUPI or a GPSI then the subscription may not be accepted, e.g. user consent is not granted and an error is sent to the consumer. When the Target of Analytics Reporting is an Internal Group Id, or a list of SUPIs/GPSI(s) or any UE, no error is sent, but a SUPI or GPSI is skipped if user consent is not granted.

* * * Next Change * * * *

7.3.2	Nnwdaf_AnalyticsInfo_Request service operation

Service operation name: Nnwdaf_AnalyticsInfo_Request.

Description: The consumer requests NWDAF operator specific analytics and optionally analytics accuracy information.

Inputs, Required: (Set of) Analytics ID(s) as defined in Table 7.1-2, Target of Analytics Reporting, Analytics Reporting Parameters (including Analytics target period, etc.).

NOTE 1:	Target of Analytics Reporting can be provided per individual Analytics ID.

Inputs, Optional: Analytics Filter Information, preferred level of accuracy of the analytics, preferred level of accuracy per analytics subset, time when analytics information is needed, maximum number of objects requested (max), preferred order of results, maximum number of SUPIs requested (SUPImax), Analytics Metadata Request, Dataset Statistical Properties, Output strategy, Data time window, Use case context, Time window for historical analytics and Analytics Accuracy Request information, NF Instance ID(s) or NF Set ID(s) of data source NF(s).

NOTE 2:	Analytics Filter Information, Maximum number of objects requested (max), Maximum number of SUPIs requested (SUPImax), Analytics Metadata Request, Dataset Statistical Properties, Output strategy, Data time window and time when analytics information is needed can be provided per individual Analytics ID.

Outputs, Required: If the request is accepted, then set of the tuple (Analytics ID, Analytics specific parameters). When the request is not accepted, an error response.

Outputs, Optional: Timestamp of analytics generation (required when ADRF is deployed), validity period, confidence, revised waiting time, Analytics Metadata Information, Analytics Accuracy information. See clause 6.1.3.

NOTE 3:	Validity period can also be provided as part of Analytics specific parameters for some NWDAF output analytics.

NOTE 4:	When the Target of Analytics Reporting is a SUPI or a GPSI then the request may not be accepted, e.g. user consent is not granted and an error is sent to the consumer. When the Target of Analytics Reporting is an Internal Group Id, or a list of SUPIs/GPSI(s) or any UE, no error is sent, but a SUPI or GPSI is skipped if user consent is not granted.

* * * End of Changes * * * *