3GPP TSG-WG SA2 Meeting #159 	S2-2310623

Xiamen, China, 9-13 October 2023

# Title:	[Draft] Reply LS on QMC support in RRC_IDLE and RRC_INACTIVE

# Response to:	LS on QMC support in RRC_IDLE and RRC_INACTIVE (R3-234745/S2-2310127)

# Release:	Release 18

# Work Item:	NR_QoE_enh-Core

Source:	SA2

To:	RAN3

Cc:	RAN2, SA5, SA3

Contact Person:

Name:	Juan Zhang

E-mail Address:	juanzhan@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-2310624, S2-2310625, S2-2310626 and S2-2310632

1. Overall Description:

SA2 thanks RAN3 about the LS on QMC support in RRC_IDLE and RRC_INACTIVE, In order to support the signalling-based and management-based QoE measurement continuity for MBS across RRC states, SA2 discussed the following principles:

- If RAN wants to support the QMC measurement continuity for MBS across RRC states, RAN sends a RAN-provided QMC configuration information to AMF during N2 release procedure. The RAN-provided QMC configuration information will only include the Management-based QMC configuration information.

- If AMF does not have a stored RAN-provided QMC configuration information, AMF will store the RAN-provided QMC configuration information in UE context. If AMF has a stored RAN-provided QMC configuration information, AMF will update the RAN-provided QMC configuration information based on the latest received information in N2 release message. The RAN-provided QMC configuration information will not impact the QMC configuration information that received from UDM.

- If AMF receives a N2 release without RAN-provided QMC configuration information, but AMF has a storage of RAN-provided QMC configuration information in UE context, AMF will remove the RAN-provided QMC configuration information from UE context. This will not impact the QMC configuration information that received from UDM.

- During the CM-IDLE to CM-CONNECTED service request procedure, AMF provides both QMC configuration information that received from UDM and the RAN provided QMC configuration information to RAN in NGAP. It depends on RAN3 that whether to define two separate IEs for these two different QMC configuration information or only use a single IE.

SA2 approved the attached CRs to support the Signalling Based Activation of QMC for Rel-17.

2. Actions:

To RAN3:

ACTION: 	SA2 requests RAN3 to consider the above information and provide any feedback to SA2 if the QoE measurement configuration information is decided to store in AMF.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #160	13-17 November 2023	Chicago, USA

TSG-SA2 Meeting #161	26 February-3 March 2024	EU