3GPP TSG-WG SA2 Meeting #159 	S2-2310834

Xiamen, China, 9-13 October 2023

# Title:	[Draft] Reply LS on PIN ID

# Response to:	Reply LS on PIN ID (C1-236503/ S2-2310093)

# Release:	Release 18

# Work Item:	PIN

Source:	SA2

To:	CT1

Cc:	CT3, CT4

Contact Person:

Name:	Kefeng Zhang

Tel. Number:

E-mail Address:	kefezhan AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

SA2 thanks CT1 for the LS (C1-236503).

SA2 has concluded that a PIN is managed at the PIN application layer, and the allocation of PIN ID is out of the scope of SA2. In 5GS, a PIN ID is assumed to be unique in a PLMN and only used in the traffic descriptor of URSP rules.

SA2 suggests CT1 to consider the following principles in defining the format of PIN ID:

- The PIN ID is only contained as a component of traffic descriptor (e.g., a string, with variable size and variable value) in the URSP rule.

- To ensure uniqueness of the PIN ID, the following is assumed: (a) If the PIN ID is assigned by an AF, the PCF or AF is assumed to include the AF ID in the PIN ID; (b) if the PIN ID is assigned by the network, the network is assumed to include a reserved unique value for each PIN ID..

- There is no need to specify the format of PIN ID for other protocols.

2. Actions:

To CT1:

ACTION: 	SA2 kindly asks CT1 to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #160	13-17 November 2023	Chicago, USA

TSG-SA2 Meeting #161	26 February-3 March 2024	EU