3GPP TSG-SA WG2 Meeting #159 	S2-2310844

Xiamen, China, 09 – 13 October, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0957 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on parameters for PFD Determination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This proposal is to correct and clarify the input parameters usages defined in Table 6.16.2-1. It is proposed to correct the input parameter list to support the SNSSAI and DNN in the output table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected parameters descriptions in Table 6.16.2-1 to clarify how to collect input data at the UPF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear how to produce output parameters based on input parameters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.16.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

6.16.2	Input Data

The NWDAF collects information on user data traffic from NF(s) for a specific S-NSSAI, DNN, Application ID, and retrieves the existing PFDs from the NEF(PFDF). The detailed input data are described in Table 6.16.2-1.

Table 6.16.2-1: Input data to detect known application from NFs

| Information | Source | Description |
| --- | --- | --- |
| Application ID (NOTE 1) | UPF | Identifies the application |
| IP Flow Information (1..max) | UPF | Per IP flow related information for the application |
| > IP 5-tuple | UPF | Identifies a service flow of the UE that uses the application. |
| > Start time | UPF | Start time of traffic detection for the flow. |
| > End time | UPF | End time of traffic detection for the flow. |
| > UL Data volume | UPF | Measured UL data traffic volume for the flow. |
| > DL Data volume | UPF | Measured DL data traffic volume for the flow. |
| > UL Data Rate | UPF | Measured UL data rate for the flow. |
| > DL Data Rate | UPF | Measured DL data rate for the flow. |
| > URL list | UPF | List of URLs extracted from the inspected user plane packets in the flow. |
| > SNSSAI | UPF | SNSSAI of the PDU session that the user packet is transmitted. |
| > DNN | UPF | DNN of the PDU session that the user packet is transmitted. |
| > Domain Name list | UPF | List of domain names extracted from the inspected user plane packets in the flow. |
| PFD Information | NEF(PFDF) | PFD Information stored in the UDR (as Application Data) and retrieved by NEF (PFDF), as defined in clause 6.1.2.3.2 of TS 23.503 [4]. |
| > Application ID | NEF(PFDF) | Identification of the application that refers to one or more application detection filters. |
| > IP 3-tuple list | NEF(PFDF) | Including protocol, server side IP address and port number. |
| > URL list | NEF(PFDF) | The significant parts of the URL to be matched, e.g. host name. |
| > Domain Name list | NEF(PFDF) | A Domain Name matching criteria and information about applicable protocol(s). |
| NOTE 1: The Application ID maps in the UPF to a detection algorithm that has a wider scope than the Application ID provided by the consumer. |  |  |

NOTE:	Extensive reporting of all traffic flows may conflict with the requirement to avoid extra UPF load. An NWDAF may subscribe only for reporting for some UEs to limit the load.

6.16.3	Output Analytics

The output analytics of NWDAF is defined in Table 6.16.3-1. The output analysis may be used to provision new or updated PFDs information for known applications. The NWDAF may assign a confidence to the PFD Determination statistics based on the input data provided by UPF.

Table 6.16.3-1: PFD Determination statistics

| Information | Description |
| --- | --- |
| Application ID | Application ID of the application that refers to the application detection filter (Flow descriptor, URL or Domain name information) stored in the UDR. |
| List of application traffic description |  |
| > S-NSSAI | Identifies the Network Slice for which analytics information is provided. |
| > DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. |
| > Flow descriptor | Flow descriptor containing 3-tuple server side IP address, server port number, and protocol of uplink/downlink application traffic as described in TS 23.503 [4]). |
| > URL list | the significant parts of the URL to be matched, e.g. host name. |
| > Domain name information | A Domain name matching criteria and information about applicable protocol(s). |
| Confidence | Confidence on the provided application traffic description for the Application ID. |

* * *End of Changes * * *