3GPP TSG SA WG 2 Meeting 159		S2-2310922

Xiamen, China, 9 – 13 October 2023

Title:	Reply LS on PDU Set Size and signaling aspects

Response to:	S4-231435

Release:	Release 18

Work Item:	XRM

Source:	SA WG 2

To:	SA WG 4

Cc:

Contact person:			Dimitrios Karampatsis

dkarampatsis AT lenovo DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR5028

# 1	Overall description

SA2 thanks SA4 for the LS on requesting clarification on the PDU set size and signalling aspects

Regarding the below question to SA2.

In Release 18, can the UPF always be aware by network configuration of possibly present NAT46/NAT64 within a CSP network?

SA2 answers:

It is up to UPF implementation and network deployment whether a UPF can detect or be aware of a presence of a NAT in the network path over N6 between the Application Server sending RTP packets and the UPF.

If the UPF/operator IP network implements NAT46/64 functionality, the UPF can use implementation-specific means to adapt the PDU set size in case there is an IP version mismatch.

If however the NAT46/64 is placed in the communication path between the UPF and AS (e.g., in a CSP/DN domain) the UPF may not always detect presence of NAT.

As such, SA2 has agreed the attached CR where an AF when detecting an IP version mismatch includes within the Protocol Description of the AF session request the "origin IP version" of the RTP packet which is forwarded via control plane signalling to the UPF. It would be then up to UPF implementation how to handle the RTP header extension information received from such packets (e.g. adapt the PDU set size accordingly to NAT configuration).

# 2	Actions

To SA4

ACTION: 	SA2 respectfully asks SA4 to take the above information into account and provide any feedback

# 3	Dates of next TSG SA WG 2 meetings

SA2#159	13th - 17th November 2023, 		Chicago, USA

SA2#160e	22nd January - 26th January 2024, 		e-meeting