3GPP TSG SA WG-2 Meeting #159		S2-2310924

Xiamen, China, October 09 – 13, 2023	(revision of S2-231xxxx)

# Source:	LG Electronics

# Title:	New WID: UE requested URSP provisioning (TEI19_URSPreq)

# Document for:	Approval

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	UE requested URSP provisioning

Acronym:	TEI19_URSPreq

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 740061 | Stage 2 of 5G System - Phase 1 | Rel-15 SA2 5GS_Ph1 |

3	Justification

For V2X Policy, ProSe Policy, A2X Policy and Ranging/SL Positioning Policy, UE requested Policy provisioning is supported as specified in TS 23.287, TS 23.304, TS 23.256, TS 23.586 and TS 23.503. For these policies, the UE can trigger policy provisioning procedure by sending a UL NAS TRANSPORT message carrying the UE Policy Container (UE Policy Provisioning Request) to the AMF. Then, the PCF that receives the UE Policy Provisioning Request can provide the UE requested policy to the UE.

On the other hand, such UE requested Policy provisioning is not supported for URSP. As the URSP continues to be enhanced and extended over multiple releases (e.g. Validation Criteria, ProSe aspect, URLLC aspect, PIN aspect, 5WWC aspect, etc), there may be the case that the UE is required to request URSP rules to the PCF instead of waiting for the updated URSP rules from the PCF (e.g., when URSP rules are missing or invalid).

Therefore, this work item proposes to specify UE requested URSP provisioning.

4	Objective

The objective of this work item is to:

-	Add description and procedures related to UE requested URSP provisioning.

This work item will require 0.5 TU(s) to discuss and agree on the corresponding CRs.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.502 | Add procedure for UE requested URSP provisioning | SA#106 (Dec. 2024) |  |
| TS 23.503 | Add description for UE requested URSP provisioning | SA#106 (Dec. 2024) |  |

6	Work item Rapporteur(s)

LaeYoung Kim (LG Electronics), laeyoung.kim@lge.com

7	Work item leadership

SA2

8	Aspects that involve other WGs

None

9	Supporting Individual Members

Supporting IM name

LG Electronics

♥♥♥

♥♥

♥