3GPP TSG SA WG2 Meeting #158	S2-2310951

# Goteburg, Sweden, 21-25 August

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1359 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.228 Editorial change for the DC procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current description in the procedure is not explicitly clear on when will IMS AS update the media resource on MF/MRF when receiving the SDP answer. This CR propose to make it more explicit. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the procedure to include when will IMS AS update the media resource on MF/MRF when receiving the SDP answer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC 7.1, AC 7.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## AC.7.1	Bootstrap Data Channel Setup Signalling procedure

Figure AC.7.1-1depicts a signalling flow diagram for establishing a bootstrap data channel in a person-to-person use case. The MF anchors the bootstrap data channel, and the originating network is offering a bootstrap data channel to the remote peer as well for application download.

In the call flow the two UEs have already established an IMS audio session, and the originating UE is updating the IMS audio/video session to an IMS data channel session.

NOTE 1:	Some SIP signalling not relevant for the procedure, is not shown in the call flow below.

Figure AC.7.1-1: Bootstrap Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

1.	UE#1 sends the SIP INVITE request with an initial SDP to the IMS AS, through P-CSCF and S-CSCF in the originating network. The initial SDP contains offers for the bootstrap data channel establishment request with bootstrap DC stream ID. In this example procedure, the SDP contains both bootstrap data channel offers for originating side and terminating side.

NOTE 2:	This SIP INVITE can also be a SIP re-INVITE performed after the initial IMS audio session is setup.

2.	IMS AS validates user subscription data to determine whether the data channel call request should be notified to DCSF.

If the IMS AS determined, based on the user profile, that data channel call request needs to be notified to DCSF, the IMS AS selects a DCSF for this user based on local configuration or discovery and selection of a DCSF instance via NRF.

If the IMS AS determined, based on the user profile, that the data channel call request need not to be notified to DCSF, or DCSF decides that DC request is not allowed, the IMS AS proceeds with normal IMS procedures to setup the MMTel session without performing Data Channel bootstrapping, by deleting DC related media information and sending the updated SIP INVITE to the originating S-CSCF.

3.	IMS AS notifies the DCSF of the DC call event by sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, Calling ID, Called ID, Session Case, Event initiator, Media InfoList, DC Stream ID) request to the DCSF.

4.	After receiving the DC control request, the DCSF determines the policy about how to process the bootstrap data channel establishment request based on the related parameters in the Data Channel control request (e.g. CallingID, CalledID, DC Stream ID) and/or DCSF service specific policy.

5.	Since the SessionEstablishmentRequestEvent indicates that served user is offered local bootstrap media, DCSF, based on its policies reserves originating side MDC1 media information, as well as the terminating side MDC1 remote bootstrap media (targeting remote UE), which are used to receive UE request for application downloading from MF or MRF.

6.	DCSF invokes the Nimsas_MediaControl_MediaInstruction (Session ID, Media Instruction Set) operation based on its policies instructing the IMS AS how to set up bootstrap data channel with MF both for originating and terminating side. The MediaInstructionSet provided by the DSCF, includes its MDC1 media endpoint addresses created in step 5, DC Stream ID, and the replacement HTTP URL representing the application list offered via the MDC1 interface.

In this scenario, the DCSF instructs the IMS AS to terminate bootstrap data channel establishment request on originating MF, and initiate remote bootstrap data channel establishment request(targeting remote UE) as well as forwarding remote bootstrap data channel establishment request of served user (targeting remote DCSF) towards terminating network.

7.	The IMS AS selects a MF based on local configuration or discovers and selects a MF instance or MRF supporting DC media function via NRF.

8.	IMS AS invokes Nmf_MRM_Create(List of Media Termination Descriptors) service operation to instruct MF to allocate required data channel media resources. IMS AS request creation of two different Media Terminations, one representing the local bootstrap media to be terminated and the other representing the remote bootstrap media to be offered to remote UE. Each Media Termination includes information required to allocate resources in both Mb and the MDC1 interfaces. The MF responds with the negotiated data channel media resource information to IMS AS. If MRF is used, IMS AS uses Mr'/Cr to the MRF to reserve data channel media resources.

NOTE 3:	The MF media resource allocation in step 8 could be done with one or multiple service invocations.

9.	IMS AS responds to the MediaInstruction request received in step 6. The response may include the atomic success result of operation and also includes negotiated data channel media resource information for MDC1.

10.	The DCSF stores the media resource information and responds to the Notify Request received in step 3.

11-13.	IMS AS sends the INVITE which includes the updated SDP offer adding media information of MF or MRF via the originating S-CSCF to remote network side and UE#2. In this scenario, the SDP offer for bootstrap data channel to UE#2 is included.

14.	UE#2 and terminating network returns an 18X response with the SDP answer to bootstrap data channel to originating network. According to the received SDP answer, IMS AS may instruct MF or MRF to update data channel media resource information for UE#2.

15-16:	UE#2 and terminating network returns a 200 OK response.

17.	The IMS AS notifies the successful session establishment event, Nimsas_SessionEventControl Notify (SessionEstablishmentSuccessEvent, Session ID, Media Info List) to DCSF.

18.	The DCSF responds to the Nimsas notification request.

19.	200 OK forwarded to UE#1 which indicates the bootstrap data channel has been established.

20-23:	The bootstrap data channels have been established between originating MF or MRF and UE#1/UE#2. The UEs send application request messages to MF or MRF to request a data channel application or an application list if multiple DC applications are available, via the established bootstrap data channel with its data channel capabilities. The MF or MRF replaces the root URL with the replacement URL received in steps 8 and forwards the message to received media point of DCSF. The DCSF provides the application list and proper data channel applications further to UE#1 and UE#2 based on their data channel capabilities and their choices through MF or MRF.

The bootstrap data channels have also been established between terminating MF or MRF and UE#1/UE#2. The data channel application is requested and downloaded to UE#1 and UE#2 from terminating DCSF

Steps 20-23 may be executed after step 14, if the SDP answer in 200 OK to the PRACK and UPDATE messages contain the information required to establish bootstrap data channels.

NOTE 4:	IMS-AGW needs to allow the establishment of bootstrap data channels based on the information in the 200 OK to PRACK and UPDATE messages.

24.	Subsequent procedures continue.

NOTE 5:	In the DC application list, the DCSF may provide DC applications supported by both UEs, or only supported by the UE who sends the application request message. The detail of how to provide the application list to the UE and how to use it by the UE are based on implementation and out of scope of 3GPP.

# * * * * Second change * * * *

### AC.7.2.3	Person-to-Application and Application-to-Person (P2A2P) Procedure

This procedure enables originating and terminating UE to establish application data channels for the same application to communicate with the same Data Channel Application Server.

The P2A2P procedure requires the establishment of application data channels from UE#1 to MF and from UE#2 to MF. The two application data channels are associated with the same application. This enables UE#1 and UE#2 to interact with the same DC Application Server simultaneously and the DC Application Server to correlate the data exchanged with both UEs. The P2A procedures as described in clause AC.7.2 are used to establish one application data channel between UE#1 and MF and, for A2P scenario, one application data channel between UE#2 and MF. In case of two involved UEs, this enables independent communication between UE#1 and UE#2 with the DC Application Server.

Figure AC.7.2.3-1: Symmetric Application Data Channel Establishment

0.	IMS session and bootstrap data channels are established. Selected data channel applications are downloaded to UE#1 and possibly UE#2.

1.	UE#1 sends SIP re-INVITE request with an updated SDP to IMS AS. The updated SDP contains the bootstrap data channel, the application data channel information and associated DC application binding information.

2.	IMS AS validates the data channel media description information and/or user subscription data to determine whether the DCSF needs to be notified.

3.	IMS AS selects and notifies the DCSF about the call event and data channel establishment request.

4.	The DCSF determines how to process the application data channel establishment request based on the parameters (i.e. associated DC application binding information) in the notification from IMS AS and/or DCSF service specific policy.

5.	DCSF determines that the added application data channel media in the SDP offer requires the DC Application Server is the endpoint for both originating and terminating UE and that the application DC must be anchored at the MF or MRF. DCSF communicates with the DC Application Server for DC resource control. Once the application data channel is established, the DC Application Server will send/receive traffic to/from UE#1 through the MDC2 interface.

NOTE 1:	Details on how the DCSF communicates with the DC Application Server are out of scope of this release.

6.	DCSF invokes Nimsas_MediaControl service to send data channel control request to IMS AS, including information how to relay data channel media via the MDC2 interface.

7.	IMS AS reserves data channel media resources at the MF via DC2 or at the MRF via Mr'/Cr based on the DC media information received from DCSF.

8.	IMS AS notifies the DCSF about MediaControl instruction control response.

9.	DCSF communicates with the DC Application Server for DC resource control and provides information on data channel media resources reserved at the MF.

10.	The DCSF stores the media resource information and replies to the Nimsas notification request.

11-12.	IMS AS sends re-INVITE which include the SDP offer from MF or MRF for the application data channel to the originating S-CSCF and then to the remote network and UE#2.

13-16.	UE#2 and terminating network return 200 OK with SDP answer for audio/video and for the application data channel. The terminating network P-CSCF executes QoS procedure for application data channel media based on the SDP in the 200 OK. Based on the received DC Application binding information in the re-INVITE, UE#2 may need to download the corresponding DC Application signalled in the SDP offer, if not done already and associate it with the requested application DC.

NOTE 2:	The UE at the terminating side is capable to determine if to use the DC application based on the received DC application binding information.

17.	IMS AS notifies the DCSF about the successful result of the MediaChangeRequest event.

18.	DCSF replies to the notification received in step 17.

19.	IMS AS may request MF/MRF to update the media resources based on the SDP answer.

20-21.	The IMS AS includes SDP answer for application data channel to UE#1 in 200 OK and sends 200 OK to S-CSCF and P-CSCF.

22.	The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP in the 200 OK.

23.	P-CSCF returns the 200 OK to UE#1.

24.	UE#1 sends ACK to the terminating network.

25.	The application data channel between UE#1 and DC Application Server is established via MF or MRF. MF or MRF forwards data channel traffic between UE#1 and DC Application Server via MDC2.

26.	The application data channel between UE#2 and DC Application Server is established via MF or MRF. MF or MRF forwards data channel traffic between UE#2 and DC Application Server via MDC2.

# * * * * End of changes * * * *