3GPP TSG-WG SA2 Meeting #159 	S2-2310954

Xiamen, October 9th – 13th, 2023	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Key issue on support IMS DC PS Data Off

Document for:	Approval

Agenda Item:	19.2

Work Item / Release:	FS_NG_RTC_Ph2 / Rel-19

Abstract: This paper proposes to add the key issue of how to enhance IMS architecture, interface and procedures to support multiplexing a SCTP connection for multiple DC applications in FS_NG_RTC_Ph2 TR.

# 1. Introduction

The FS_NG_RTC_Ph2 SID includes the following work task:

-	WT-4.2: how to support 3GPP PS Data Off for IMS data channel and applications over IMS data channel.

# 2. Discussion

The requirement for supporting 3GPP PS Data Off for IMS data channel and applications over IMS data channel was coming from R19 SA1, which TS 22.011 [1] defines that:

Each of the following operator services shall be configurable by the HPLMN operator to be part of the 3GPP PS Data Off Exempt Services:

-	MMTel Voice;

-	SMS over IMS;

-	USSD over IMS (USSI);

-	MMTel Video;

-	Services over IMS Data Channel;

-	Particular IMS services not defined by 3GPP, where each such IMS service is identified by an IMS communication service identifier;

-	Device Management over PS

-	Management of USIM files over PS (e.g. via Bearer Independent Protocol); and

-	IMS Supplementary Service configuration via the Ut interface using XCAP.

Observation 1: It is assumed that multiple IMS DC Services could be supported in one PLMN.

IMS data channel architecture is agnostic to a given IMS DC service, which means it could support various IMS DC services without specific enhancement to the network.

Observation 2: Operators may want to apply IMS PS Data Off policies to all or only to a subset of the IMS DC services that it provides.

Given the above requirement, the network should:

-	Support a mechanism that services over IMS DC that are not PS Data Off Exempted shall not establish IMS data channel when the PS Data Off is active.

# 3. Text Proposal

It is proposed to capture the following changes into TR 23.700-77.

# 4. References

[1] 3GPP TS 22.011, " Service accessibility"

# * * * * First change * * * *

## 5.X	Key Issue #X: Support 3GPP PS Data Off for IMS data channel and applications over IMS data channel

### 5.X.1	Description

The Key Issue is to study the IMS architecture and solution to support 3GPP PS Data Off for IMS data channel and applications over IMS data channel including:

-  Support a mechanism to selectively exempt services over IMS DC from PS Data Off. If a service over IMS DC is not PS Data Off exempted, the UE will not establish IMS data channel.

# * * * * End of changes * * * *