3GPP TSG-WG SA2 Meeting #159	S2-2310958

09 - 13 October, 2023, Xiamen, China

# Title:	[DRAFT] Reply LS on NSACF Role and NSAC Service Area

# Response to:	LS on NSACF Role and NSAC Service Area (S2-2310078/C4-232359)

# Release:	Rel-18

# Work Item:	eNS_Ph3

Source:	SA2

To:	CT4

Cc:

Contact Person:

Name:	Ashok Kumar Nayak

E-mail Address:	ashok.nayak@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

SA2 would like to thank CT4 regarding the LS on NSACF Role and NSAC Service Area. SA2 provides following answers to CT4 questions.

Q1. For hierarchical NSACF architecture, an NSACF may function as a primary NSACF or a subordinate NSACF. However, it is not clear to CT4 that whether the configuration of NSACF role (i.e. primary NSACF or subordinate NSACF) is per S-NSSAI basis, or for the whole PLMN?

Answer: The NSACF be it Primary NSACF or subordinate NSACF is always configured per S-NSSAI basis but if it is functioning as Primary NSACF then it is applicable for whole PLMN.

Q2. If the network is configured with multiple NSAC service areas, is it possible or valid deployment that, for a given S-NSSAI, an NSACF instance can act as the primary NSACF and at the same time for certain NSAC service area(s) also act as a distributed NSACF (with other NASC service areas served by other distributed NSACFs).

Answer: For a given S-NSSAI the NSACF service areas served by Primary NSACF is whole PLMN.

Q3. It is not clear to CT4, whether a NSAC service area may be identified by TAI(s) or should be instead identified by a new NSAC service area identifier that may take the form of an operator defined string?

Answer: The NSAC service areas can be identified by TAI(s) or some operator defined string like Area code or can be both.

2. Actions:

To CT4 group.

ACTION: 	SA2 kindly asks CT4 to take above information into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting#160	November 13 – 17, 2023	Chicago, USA