3GPP TSG-SA WG2 Meeting #159	S2-2311109

# Xiamen, China, October 09 - 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0962 |  | rev |  | - |  |  | Current version: |  |  | 18.3.0 |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  | UICC apps |  |  |  | ME |  |  | Radio Access Network |  |  | Core Network | x |  |

| Title: | Correction on usage of untrusted AF rating |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In clause 6.2.13.2, there's a proposal suggesting that AnLF can rate untrusted AF data sources; and if the rate is low, it might use alternative data sources (if available) to generate analytics (i.e., for making inferences). However, if the data source itself is of low quality, using it to train a model by MTLF will result in a less accurate machine learning model. Consequently, even if alternative data sources are used by AnLF for the corresponding analytics, they may not effectively address this issue. To address this concern, this CR proposes that, in addition to AnLF, the MTLF may also consider the data source rate during the (re)training phase of ML models. Moreover, there are some duplicated texts that this CR proposes to remove. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposing to use data source rating by MTLF for model training Removing the duplicated text |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.13.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

#### 6.2.13.2	Procedure for rating untrusted AF data sources

The process of rating untrusted AF data sources is depicted in Figure 6.2.13.2. For realizing potential issues, the NWDAF containing AnLF subscribes to the NWDAF containing MTLF, which performs an accuracy calculation based on the predicted and ground-truth data or alternatively the NWDAF containing AnLF can calculate the accuracy locally by comparing the predicted and ground-truth data.

Figure 6.2.13.2: NWDAF containing AnLF-based untrusted AF data source rating

1.	NWDAF containing AnLF subscribes to NWDAF containing MTLF for obtaining an ML model using the Nnwdaf_ModelProvision_Subscribe service operation. The NWDAF containing AnLF may include a threshold (as described in clause 6.2E.2) to indicate when the NWDAF containing MTLF needs to execute the accuracy monitoring operations.

Option 1: Accuracy report from NWDAF containing MTLF

2a.	NWDAF containing MTLF evaluates the ML model accuracy according to clause 6.2E.2.

2b.	An accuracy report is sent to the NWDAF containing AnLF, e.g. when the reporting threshold is met by invoking Nnwdaf_MLModelProvision_Notify service operation.

Option 2: NWDAF containing AnLF computes accuracy

2c.	NWDAF containing AnLF calculates the accuracy by comparing the predictions with ground truth data.

2d.	NWDAF containing AnLF is aware that the ML Model used has a low accuracy either by receiving the accuracy report in step 2b or monitoring the accuracy by itself in step 2c. NWDAF containing AnLF determines that it needs to check further the data sources and compute data source rating. The decision conditions upon which it needs to initiate data source rating for a data source is based on NWDAF containing AnLF implementation.

3a-3b.	NWDAF containing AnLF initiates rating of a data source by requesting and receiving supplementary data, i.e. via Nnwdaf_DataManagement_Fetch / Ndccf_DataManagement_Notify, from different data sources (if available) to verify the data source quality or correctness. Such data can be for example performance data from the OAM which are supplementary to the data from untrusted AFs, or data from UPF supplementary to the data from untrusted AFs.

4.	NWDAF containing AnLF updates the rating for the sources where untrusted data is deviated from the supplementary trusted data per Event ID.

NOTE 1:	An NWDAF containing AnLF determines the rating of an untrusted AF data source based on internal operations.

5a.	NWDAF containing AnLF stores the untrusted AF data source rating locally.

5b.	NWDAF containing AnLF may send the untrusted AF data source rating into an external storage, if available. In this release, only UDSF is considered as external storage for the data source rating results. NWDAF containing AnLF uses the Nudsf_ UnstructuredDataManagement_Create service operation.

NOTE 2:	To avoid an untrusted AF to be permanently excluded as a data source, the NWDAF containing AnLF can re-rate the untrusted AF based on its internal logic. For example, it can rate the untrusted AF after some timer expired.

6.	A NWDAF consumer subscribes to a certain Analytics ID = "xx", using Nnwdaf_AnalyticsSubscription_Subscribe service operation.

7a.	The NWDAF containing AnLF retrieves the untrusted AF rating of the data sources locally.

7b.	The NWDAF containing AnLF retrieves the untrusted AF rating of the data sources from the UDSF using the to use Nudsf_ UnstructuredDataManagement_Query service operation.

8.	If the rating of one or more untrusted AF is below a threshold (i.e. that is pre-configured), then the NWDAF containing AnLF can:

(i)	select an alternative untrusted AF (if available) with higher rating; or

(ii)	request supplementary data from other trusted data sources.

9.	The NWDAF containing AnLF subscribes to a new data source to receive alternative or supplementary data if a new data source is selected in step 8.

10.	The NWDAF containing AnLF may use the rate of untrusted AF data sources as a criterion to calculate the confidence level of the respective analytics output.

11.	The NWDAF containing AnLF provides the analytics output to the analytics consumer, using the Nnwdaf_AnalyticsSubscription_Notify service operation.

In the case of trigger for ML model (re)training, the NWDAF containing MTLF optionally performs steps 12-14.

12.	[Optional] The NWDAF containing MTLF retrieves the untrusted AF rating of the data sources of the Event IDs needed to (re)train the ML model from the UDSF using the to use Nudsf_ UnstructuredDataManagement_Query service operation.

13.	[Optional] If the rating of one or more untrusted AF is below a threshold (i.e. that is pre-configured), then the NWDAF containing MTLF can:

(i)	select an alternative untrusted AF (if available) with higher rating; or

(ii)	request supplementary data from other trusted data sources.

14.	[Optional] The NWDAF containing MTLF subscribes to a new data source to receive alternative or supplementary data if a new data source is selected in step 13.

15. The NWDAF containing MTLF (re)trains the ML model.

********** End of Changes **********