3GPP SA WG2 Meeting #159	S2-2311134

Xiamen, PRC, October 9-13th, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	KI description for WT#3.1

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract of the contribution: This paper proposes KI description for WT#3.1

# Discussion

SP-231198 defined FS_XRM Ph2 WT#3.1 for Rel. 19 as:

WT#3 Further enhancement to support XR based on non-3GPP access.

WT#3.1 Study how to support L4S for non-3GPP access networks and intermediate 5GS nodes (N3IWF, TNGF and W-AGF) to perform ECN marking for L4S.  ​

- Support L4S in untrusted/trusted access (e.g. N3IWF, TNGF).

- Support L4S in wireline access (e.g. W-AGF).

NOTE 1: It is limited to re-using existing control plane and user plane between 5GC and non-3GPP access networks. Assumptions on W-AGF functionality are to be verified with BBF and CableLabs.

A Rel. 19 KI is needed for this WT. The following is proposed.

# Proposal

It is proposed to define the following KI for Rel. 19 for XRM_Ph2 WT#3.1 for incorporation in the XRM Ph2 TR23.700-70.

*** Start of changes ***

X.1	Key Issue #X: L4S for non-3GPP access networks and intermediate 5GS nodes

X.1.1	Description

As the use-cases and applications of XRM are not limited to 3GPP access only, XRM devices and applications may use non-3GPP access as a means of communication. In such a scenario where the XRM device(s) use non-3GPP access path with 5GC backhaul, the intermediate nodes such as N3IWF, TNGF, W-AGF may experience UP traffic congestion (for example, an N3IWF/TNGF/W-AGF/5G-RG deployment supports multiple access connections and as a result may experience congestion).

L4S is a concept that allows for congestion reporting throughout the network (i.e., E2E) where any node managing traffic queues may support L4S-specific ECN marking (i.e., access points and intermediate routers such as N3IWF, TNGF, W-AGF, 5G-RGs).

The objective of this Key Issue is to study whether any enhancements are required to extend the L4S mechanism for the non-3GPP access and potential impacts of such extension on the non-3GPP access-specific intermediate nodes such as N3IWF, TNGF, W-AGF, 5G-RGs. The KI touches on several areas related to L4S with respect to non-3GPP access. The following aspects should be studied:

- Whether and how L4S congestion with ECN marking is handled in non-3GPP access-specific 5GS intermediate nodes such as N3IWF, TNGF, W-AGF, 5G-RGs, etc.

- Whether and how congestion information exchange happens between the non-3GPP access intermediate nodes and 5GC, and vice versa​.