SA WG2 Meeting #159	S2-2311176

October 9-13, 2023, Xiamen, China

Source:	CableLabs

Title:	pCR 23.700-70: Solution for PDU Set QoS support in wireline access

Document for:	Approval

Agenda Item:	19.3

Work Item / Release: 	FS_XRM_Ph2 / Rel-19

Abstract: This paper proposes a solution for PDU Set QoS support in wireline access.

# 1. Introduction

The new approved SID titled Study on Architecture enhancement for XRM Ph2 (SP-231198) includes the below non-3GPP aspects to be studied as part of Rel-19:

WT#3 Further enhancement to support XR based on non-3GPP access.

WT#3.2 Study how PDU Set QoS Control mechanisms can be extended to non-3GPP access networks.

-	Support PDU Set QoS in untrusted/trusted access (e.g. N3IWF, TNGF).

-	Support PDU Set QoS in wireline access (e.g. W-AGF).

This pCR proposes a solution for WT#3.2 on PDU Set based QoS handling support in wireline access.

# 2. Proposal

This contribution proposes to include Solution X in TR 23.700-70 v0.0.0.

Start of changes

6.X	Solution #X: PDU Set handling in wireline access

6.X.1	Key Issue mapping

This solution proposes a solution for the following issues in KI#x:

- The mechanisms needed to provide the PDU Set QoS parameters to the wireline access gateway nodes (W-AGF, RG) to support PDU Set based QoS handling in the wireline access.

- The mechanisms needed to provide the PDU Set Information to the wireline access gateway nodes (W-AGF, RG) to support PDU Set based QoS handling in the wireline access.

6.X.2	Description

6.X.2.1	General

To support PDU Set based QoS handling in the wireline access, PDU Set QoS parameters and PDU Set Information needs to be provided to the wireline nodes (W-AGF and RG).

The proposed solution has the following principles:

- Currently specified PDU Set QoS parameters would be supported in wireline access.

- Currently specified PDU Set Information would be supported in wireline access.

6.X.2.2	Supporting PDU Set based QoS handling in W-5GAN

The PDU Set based QoS handling by the W-5GAN is determined by PDU Set QoS parameters and PDU Set information provided by the PSA UPF.

For QoS Flows with PDU Set based QoS handling enabled, PDU Set QoS parameters are determined by the PCF (based on information provided by AF and/or local configuration) and provided by SMF to the W-AGF as part of the QoS profile. When the W-AGF receives N2 requests related to PDU Session resources, the W-AGF maps the QoS profile(s) received from the 5GC to W-UP level QoS.

The PSA UPF identifies PDUs that belong to PDU Sets and determines the below PDU Set Information which it sends to the W-AGF in the GTP-U header. The PDU Set Information comprises:

- PDU Set Sequence Number.

- Indication of End PDU of the PDU Set.

- PDU Sequence Number within a PDU Set.

- PDU Set Size in bytes.

- PDU Set Importance.

6.X.3	Procedures

TBD

6.X.4	Impacts on services, entities and interfaces

TBD

End of changes