3GPP TSG-SA WG2 Meeting #159	S2-2311200

# Xiamen, P.R. China,  09th Oct 2023 – 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.586 | CR | 0070 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | v | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SL positioning operation with proximity indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Client UE may send SL Positioning Service Request for positioning of target UE to a SL Reference UE when only SL Reference UE(s) are discovered. For successful SL positioning operation, the client UE shall be able to select a SL Reference UE which is in proximity to target UE. But, current mechanism does not provide such an information to client UE. For ranging, when UEs are not inproximity to each other, the service request from client UE shall be responded with reject which is not clear in current spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | After PC5 connection setup, client UE and SL reference UE communicates to indicate whether the SL reference UE is in proximity to the target UE. For ranging, failure case is clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The client UE cannot select proper SL reference UE so that SL positoining service request cannot be handled successfully. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | v | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | v | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | v | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.7.1	Procedures for service exposure to SL Positioning Client UE

#### 6.7.1.1	Procedures for Ranging/SL Positioning service exposure through PC5

Figure 6.7.1-1.	Service exposure to SL Positioning Client UE via PC5

1.	When a Ranging/SL Positioning service is triggered, SL Positioning Client UE may perform discovery procedure for the Ranging/SL Positioning service defined in clause 6.4 with following updates:

-	When SL positioning service is triggered, the client UE tries to discover the target UE and/or a SL Reference UE. When Model B discovery used, in the Solicitation message, target UE's user info is included and role of target UE and role of SL Reference UE are indicated in the RSPP metadata.

-	When Ranging service is triggered, the client UE tries to discover UE1 and/or UE2. When Model B discovery used, in the Solicitation message, UE1's user info and UE2's user info are included and role of target UE is indicated in the RSPP metadata.

During discovery for SL positioning service, the information of SL reference UE is acquired.

2.	If discovery procedure succeeds, the SL Positioning Client UE establishes a PC5 connection with the discovered UE.

3.	The SL Positioning Client UE is informed of UE's status information indicating that UE has NAS connection or no NAS connection from the discovered UE(s).

Editor's note:	Need to determine if such indication is provided during discovery or step 3, and whether or not it requires RAN WG2 involvement.

The SL Positioning Client UE may receive list of SL Reference UE(s) from target UE over the PC5 connection.

The SL Positioning Client UE may request the discovered SL Reference UE(s) the indication of proximity to the target UE.

SL Reference UE may inform SL Positioning Client UE of indication of proximity to the target UE as response to the request from client UE or unsolicitely.

4.	Based on UE's status information, the SL Positioning Client UE down selects UE(s) to perform SL Positioning operation and select a UE (here UE1) to receive SL Positioning Service Request. The SL Positioning Client UE selects a SL Reference UE which is in proximity to the target UE based on the indication received step 3. The SL Positioning Client UE sends a SL Positioning Service Request which includes info of SL Positioning Client UE, info of Target UE and info of SL Reference UE for SL positioning, and info of SL Positioning Client UE, info of UE1 and info of UE2 for Ranging.

For Ranging, if UE1 and UE2 are not in proximity, the SL Positioning Service Request is responded with reject by the receiving UE (either UE1 or UE2) and remaining steps are not performed.

5.	Authorization check of the client UE for the service request may be performed via SL Positioning Server UE or 5GC or internally by UE1 or target UE.

6.	The UE which received SL Positioning Service Request (here UE1) selects LMF or SL Positioning Server UE based on its status whether UE has NAS connection. Ranging/SL Positioning operation is performed as described in clause 6.8 among target UE, SL Reference UE and SL Positioning Server UE or LMF.

7.	The Ranging/SL Positioning result is provided to the SL Positioning client UE by UE1.

# * * * * End of change * * * *