SA WG2 Meeting #S2-159	S2-2311210

October 09 - 13, Xiamen, China	(revision of S2-230xxxx)

Source:	Google

Title:	Key Issue #X for WT2.1 to support QoS Flow mapping for different media types multiplexed data flows

Document for:	Approval

Agenda Item: 	19.3

Work Item / Release:	FS_XRM Ph2 /Rel-19

Abstract of the contribution: This paper proposes a new KI#X for WT#2.1 to support QoS flow(s) mapping for different media types multiplexed data flows for FS_XRM Ph 2.

# 1	Discussion

This PCR is proposed a new KI#X for WT#2.1 to study whether and what enhancements are needed for traffic detection and QoS flow mapping for different media types multiplexed data flows within a single end-to-end transport connection.

# 2	Proposal

It is proposed to adopt the following changes into TR 23.XXX-YY.

* * * * Start of Changes (all new) * * * *

## 5.x	Key Issue #x: Support QoS Flow mapping for different media types multiplexed data flows

### 5.X.1	Description

Editor’s note: This clause provides a description of the key issue. It's recommended to provide Use cases/scenarios here to support the key issue.

This KI#X is related to WT#2.1 to study whether and what enhancements are needed for traffic detection and QoS flow mapping for different media types multiplexed data flows within a single end-to-end transport connection.

For Rel-18 PDU Set based handling supports for XRM services, all the PDUs (at the application layer) for a PDU Set are transmitted within the same QoS flow of a service data flow. Since QoS flow is still the finest granularity of QoS differentiation in the PDU Session, each PDU Set in a QoS flow is treated according to the same PDU Set based QoS requirements for a specific media type. That is, the existing QoS model maintains one to one relationship among service data flow, QoS parameters/characteristics, and QFI. This implies that XRM application has to use different data flows for different media types for PDU Set based handling to work as defined in Rel-18 TS23.501 [2] and TS 23.502 [3]. However nowadays XRM applications are most commonly streaming with different media types in multiplexed data flows for a transport layer connection, e.g. QUIC.

For WT#1.2, this key issue proposes to enable one or more QoS flow mapping for different media types multiplexed data flows within a transport layer connection. The key issue includes the following aspects:

-	What information needs to be provided by the AF to 5GS is support of multiple QoS flows mapping

-	Whether and how to enhance the QoS model and policy control for mapping multiple QoS flows

-	What information needs to be provided to the RAN and/or UPF to support PDU Set based handling in both the downlink and uplink direction if mapping multiple QoS flows is enabled

NOTE 1: the solution needs to specify the assumption of the application layer protocol, e2e encryption protocol, and transport layer protocol used by the XRM application.

NOTE 2: the solution should avoid or mitigate potential out of order PDUs or PDU Sets delivery for different media types multiplexed data flows.

NOTE 3:	Coordination with SA4 WG and RAN2 WG are needed.

* * * * End of Changes * * * *