3GPP TSG SA WG2 Meeting #159	S2-2311224

Xiamen, 2023‑10-09 - 2023‑10-13	(revision of S2-220xxxx)

Source:	Apple

Title:	Issues with UE-to-UE Relay Discovery

Document for:	Discussion

Agenda Item:	9.7.2

Work Item / Release:	5G_Prose / Rel-18

Abstract: This discussion paper highlights issues with UE-to-UE relay discovery procedure and proposes ways to fix them. Revision of S2-2308867 from SA2#158. Newly added sections 1.1.1 - 1.1.3

# 1. Discussion

### 1.1 Use of relay_indication in ProSe Direct Discovery

5G ProSe Direct Discovery using Model A in clause 6.3.2.1 specifies that the announcing UE includes relay_indication if a 5G ProSe UE-to-UE relay can broadcast its User Info ID.

In clause 6.3.2.1 for Model B Direct Discovery, it is specified that a Discoverer UE can include relay_indication in Solicitation message and Disoveree UE can include relay_indication in the Response message.

The relay_indication informs the recipient UE that it can re-broadcast the User Info ID in a further UE-to-UE Relay Discovery procedure. However, this method of informing consent for usage of User Info ID during U2U Relay Discovery procedures raises a few questions. A ProSe UE-to-UE relay discovery and communication is based on Relay Service Code (RSC). RSC contains information to indicate the connectivity service the 5G ProSe UE-to-UE Relay provides to 5G ProSe End UEs. There is no RSC involved in the ProSe Direct Discovery procedure. In that case it is not clear how the recipient UE in a ProSe Direct Discovery should relate User Info ID to one or more of its authorized RSCs.

Moreover, in clause 6.3.2.4.2 it is clarified that a ProSe UE-to-UE Relay discovers other UEs in proximity through a Relay Discovery or Relay Communication procedures. It is not expected that UE-to-UE Relay discovers UEs in it is proximity using a legacy ProSe Direct Discovery procedure.

Observation 1: ProSe direct discovery lacks RSC information, so a UE-to-UE Relay UE cannot correlate ProSe Relay Service and relay_indication in ProSe direct discovery.

We think there are three possible ways to resolve this issue:

Option 1: Add RSC along with relay_indication to ProSe Direct Discovery.

In Model A discovery, Announcing UE includes the relay_Indication and RSC(s) if 5G ProSe UE-to-UE Relay(s) can broadcast its User Info ID during the 5G ProSe UE-to-UE Relay Discovery for these RSCs.

In Model B discovery, Discoverer UE includes the relay_Indication and RSC(s) if 5G ProSe UE-to-UE Relay(s) can broadcast its User Info ID during the 5G ProSe UE-to-UE Relay Discovery for the indicated RSC(s). Similarly, the Discoveree UE includes relay_Indication and RSC(s) if 5G ProSe UE-to-UE Relay(s) can broadcast its User Info ID during the 5G ProSe UE-to-UE Relay Discovery for the indicated RSC(s).

The User Info IDs may also be different per RSC and in that case the UE includes User Info ID applicable for each RSC it is including in Direct Discovery procedure.

Option 2: Use only RSC(s) in ProSe Direct Discovery

If the ProSe Direct Discovery also includes the RSC(s), it can already be an indication that the UE is indicating its consent to use its User Info ID for a ProSe UE-to-UE Relay Discovery procedure for that RSC(s). In this case, we propose to remove the relay_indication from ProSe Direct Discovery and instead use list of RSC(s) and User Info ID(s) associated per RSC.

Option 3: Remove relay_indication from ProSe Direct Discovery

With this method, we clearly differentiate ProSe Direct Discovery from ProSe UE-to-UE Relay Discovery procedure. This seems to be aligned with the initial intention, since the CR0226 in S2-2303365 had removed references to Direct Discovery procedure from 3GPP TS 23.304 clause 6.3.2.4.2 step 1.

We propose to adopt the third option. A CR matching this proposal is provided in S2-2310158.

#### 1.1.1 Discussion in CT1#143

CT1 has also discussed UE-to-UE Relay Discovery procedures and has asked a question in their LS to SA2 (S2-2310091)

- According to clause 6.3.2.1 of TS 23.304, during 5G ProSe direct discovery procedures, the announcing UE/ discoverer UE/ discoveree UE "includes the relay_Indication if 5G ProSe UE-to-UE Relay(s) can broadcast its User Info ID during the 5G ProSe UE-to-UE Discovery". However, the user info ID, used in the 5G ProSe UE-to-UE relay discovery, cannot be obtained by the 5G ProSe UE-to-UE relay UE during the 5G ProSe direct discovery. So the 5G ProSe UE-to-UE relay UE cannot identify the same 5G ProSe end UE who sent the relay indication by using this user info ID in the 5G ProSe UE-to-UE relay discovery. This makes the relay indication mechanism cannot work. Then: Question 1: How can this issue be resolved?

CT1 has also raised the aspect that ProSe Direct Discovery is missing essential parameter to enable a UE-to-UE Relay UE to add a previously discovered UE to the list of End UEs to be broadcasted in a subsequent UE-to-UE Relay Discovery Announcement message. This is reiterating the fact that relay_indication in ProSe Direct Discovery is not sufficient to allow the Relay UE to fulfil the following yellow highlighted text.

- The 5G ProSe UE-to-UE Relay sends a UE-to-UE Relay Discovery Announcement message. The UE-to-UE Relay Discovery Announcement message contains the Type of Discovery Message, User Info ID of the 5G ProSe UE-to-UE Relay, RSC and list of User Info ID of the 5G ProSe End UEs supporting the RSC. The UE-to- UE Relay Discovery Announcement message is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.4. The 5G ProSe UE-to-UE Relay shall only announce User Info IDs of other UEs in proximity which provided relay_indication when they were previously discovered.

Observation 2: ProSe Direct Discovery lacks User Info ID which is needed by a Relay UE to broadcast in UE-to-UE Relay Discovery Announcement message.

#### 1.1.2 Discussions in SA2#158

In SA2#158 meeting, a comment was raised that “relay_indication” is to be used as a consent mechanism.  A ProSe UE including relay_indication in ProSe Direct Discovery messages is an indication to the Relay UE know that it is willing to be included in the list of End UEs in a subsequent UE-to-UE Relay Discovery Announcement (Model A). In the absence of “relay_indication”, a UE-to-UE Relay UE might collect User Info IDs of all ProSe UEs, that has availed its service for a UE-to-UE Relay Discovery or UE-to-UE Relay Communication and use them for subsequent UE-to-UE Relay discovery announcements.

In our analysis, inclusion of User Info ID in UE-to-UE Relay Discovery Announcement message in itself cannot be termed as a security risk. End UE’s User Info ID is also included in a UE-to-UE Relay Discovery Solicitation message from the Relay UE. Broadcasting in a Model A Discovery Announcement message also exposes User Info ID to the same extent. Other UEs in proximity listen to UE-to-UE Relay Discovery Announcement or UE-to-UE Relay Discovery Solicitation message and learns the End UE-1 is in 1-hop or 2-hop proximity to itself and this cannot be prevented.

Also, User Info IDs are not unique identifiers for the UE. It is derived within the ProSe service/application space. This may be understood only by other ProSe UEs that is also interested in the same ProSe service. Anonymity can be ensured by application design that does not allow a User Info ID to be mapped back to a UE/user identity.

Observation 3: Relay_indication alone cannot ensure privacy/user consent for an End UE taking part in a UE-to-UE Relay Discovery procedure.

#### 1.1.3 Summary

To summarize,

Observation 1: ProSe direct discovery lacks RSC information, so a UE-to-UE Relay UE cannot correlate ProSe Relay Service and relay_indication in ProSe direct discovery.

Observation 2: ProSe Direct Discovery lacks User Info ID which is needed by a Relay UE to broadcast in UE-to-UE Relay Discovery Announcement message.

Observation 3: Relay_indication alone cannot ensure privacy/user consent for an End UE taking part in a UE-to-UE Relay Discovery procedure.

Based on these observations, this discussion paper proposes the following:

Proposal 1: Correct ProSe Direct Discovery clause by removing relay_indication from ProSe Direct Discovery procedure.

### 1.2 Inconsistency with Stage 3 on U2U Relay Model B Discovery Response

3GPP TS 23.304 clause 6.3.2.4.3 describes the procedure for ProSe UE-to-UE Relay Discovery with Model B. In Step 4, it is stated that the 5G ProSe UE-to-UE Relay sends a 5G ProSe UE-to-UE Relay Discovery Response message. The 5G ProSe UE-to-UE Relay Discovery Response message contains the Type of Discovery Message, User Info ID of UE-to-UE Relay, RSC, User Info ID of the discoverer 5G ProSe End UE (UE-1) and User Info ID of the Discoveree 5G ProSe End UE (UE-2). However as per the Stage 3 description in Table 10.2.1.14 of 3GPP TS 24.554, the User Info ID of the Source Discoverer UE is mandatory to be included. It is proposed that TS 23.304 description is fixed to include the UE-1 User Info ID as one of the parameters for the Discovery Response in Step 4.

Proposal 2: Include the Discoverer UE User Info ID in the Discovery Response message sent from the U2U Relay

A CR with this proposal is provided in S2-2310158.

2. Proposal

Proposal 1: Correct ProSe Direct Discovery clause by removing relay_indication from ProSe Direct Discovery procedure.

Proposal 2: Include the Discoverer UE User Info ID in the Discovery Response message sent from the U2U Relay