3GPP TSG|WG-2 Meeting #159 	S2-2311237

Xiamen, China, October 9-13, 2023

# Source:	China Telecom

# Title:	New WID on 5GC support for IoT NTN

# Document for:	Approval

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New WI on 5GC support for IoT NTN

Acronym:	TEI19_ 5GC_ IoT NTN

Unique identifier:

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 80048 | Stage 1 of 5GSAT | Service requirements of satellite access in 5G |
| 920069 | NB-IoT/eMTC support for Non-Terrestrial Networks (LTE_NBIOT_eMTC_NTN) | E-UTRAN support for IoT NTN |
| 860005 | Integration of satellite systems in the 5G architecture (5GSAT_ARCH) | 5GS NTN functionality to be used as baseline for IoT NTN in EPS |
| 930019 | Architecture support for NB-IoT/eMTC Non-Terrestrial Networks in EPS (IoT_SAT_ARCH_EPS) | WID of E-UTRAN support for IoT NTN |
| 800026 | Study on architecture aspects for using satellite access in 5G | Unresolved key issue leftover from R17 |
| 940074 | 5GC enhancement for satellite access Phase 2 | SA2 Rel-18 SID for 5G System enhancement for satellite access |
| 980014 | 5GC/EPC enhancement for satellite access Phase 2 | SA2 Rel-18 WID for 5G System enhancement for satellite access |

3	Justification

IoT NTN plays a crucial role in enhancing the IoT market by providing efficient, cost-effective, and scalable connectivity solutions, especially when considering the interoperability with terrestrial networks. It has been proven that IoT NTN is the first commercially deployed 3GPP NTN technology based on observation on recent market actions and business plans from various operators, satellite companies and chipmakers.

First available IoT NTN (IoT_SAT_ARCH_EPS) specification is standardised in 3GPP in Release-17, with support of Cellular IoT over satellite access into EPS. For a traditional mobile network operator with legacy EPS deployed, it can upgrade the existing EPC to support IoT NTN.

But for some operators especially new satellite operators, with no existing EPC deployed, from evolution point of view 5GC may be more suitable for future commercial deployment of IoT NTN. Furthermore if an operator need joint NR NTN and IoT NTN deployment to meet various requirements from market, then a single-core solution both for NR NTN and IoT NTN has advantages over two-core sulution, from the perspective of reducing CAPEX & OPEX.

Based on above requirement, 5GC support for IoT NTN is very urgent issue for the successfulness of IoT NTN commercial launch. Moreover the 5GC is on evolution roadmap, with constantly developed new features which can also apply to IoT NTN access, as contrast 3GPP has almost stopped developping new feature on EPC. So it is necessary to standardize the 5GC support for IoT NTN in R19 to meet commercial deployment and evolution requirements.

3GPP has already standardized the 5GC support of CIoT including NB-IoT, involving 5GC and NG-RAN enhancement. So the standardization for 5GC support for IoT NTN could mainly focus on specific aspects for CIoT over satellite access. In fact the similiar specific mechanisms designed for 5GC support of NR NTN are already reused for EPC support of IoT NTN. These specific features includes:

- Support for identification and restriction and new RAT types in 5GC of using satellite access to support Cellular IoT

- Verification of UE location for satellite access to support Cellular IoT

- Network selection for satellite access to support Cellular IoT

- Mobility registration update for satellite access to support Cellular IoT

- Tracking area handling for satellite access to support Cellular IoT

- Mobility forbidden area and Service Area Restrictions for satellite access to support Cellular IoT

- Access restriction for Cellular IoT

- Discontinuous network coverage for satellite access to support Cellular IoT

4	Objective

The objective is to specify enhancement on interfaces, protocols and features of 5GC in order to support IoT NTN.

Based on above justification, the following enhancements are needed:

- Identification and distinguish the RAT types in 5GC based on new CIoT NTN satellite access types provided on N2 interface, for the purposes of QoS treatment, charging and mobility restriction.

- Alignment work on both UE side and network side to support the new added satellite access type

- Mobolity restriction for IoT-NTN related RAT types.

- Policy control for IoT NTN.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | A new clause to state the general guidence on support for Cellular IoT over satellite access into 5GS. New RAT Types values relevant to Cellular IoT over satellite access are introduced in 5GC, for identification and access restriction of the different RATs. | TBD | This will be handled as “TEI19”. |
| 23.502 | A new clause to state the general guidence on the procedures and Network Function services for E-UTRA satellite access to 5GC | TBD |  |
| 23.503 | 5GC policy control for IoT NTN | TBD |  |

This TEI19_WID requires 1 TU

6	Work item Rapporteur(s)

Nie Heng, China Telecom, nieheng@chinatelecom.cn

7	Work item leadership

SA2

8	Aspects that involve other WGs

NGAP enhancement by RAN3, e.g. identification, restriction for RATs of IoT NTN, location reporting for IoT NTN

9	Supporting Individual Members

Supporting IM name

China Telecom

VIVO?

CATT?

Xiaomi?

Novamint?

Thales?

Sateliot?