3GPP TSG-WG SA2 Meeting #159	S2-2311301

Xiamen, China, October 9 – 13, 2023	(revision of S2-2310474)

Source:	Huawei, HiSilicon

Title:	New KI: Information collection for energy saving and/or energy efficiency

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract: This document proposes a new Key Issue description regarding the information collection for energy saving and/or energy efficiency.

1. Introduction

This document proposes new Key Issues description regarding the following aspect:

- Information provisioned by Radio Access Network for energy saving and energy efficiency. Following part copies WT#1

- WT #1. Study potential framework for network energy consumption exposure. This will include whether and what information is exposed, how it is exposed (e.g., charging) and at what granularity, e.g., at RAN level, Core Network level, network slice level, UE level, PDU session level, and/or QoS flow level. Additionally, whether and how renewable energy or carbon emission information for such granularities can be exposed by an MNO will be studied.

Energy used by the RAN nodes contributes the major part of the energy consumption in the PLMN network, while the energy consumption at 5GC NFs needs to be considered as well. Therefore, the network might be able to derive the information related to energy consumption to further determine the subsequent behaviours with respect to energy saving /efficiency (e.g. policy control, NF energy state) by identifying the necessary/relevant information items from this study, and we add such part as a new KI.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-66

# * * * * First change * * * *

X.C	Key Issue #C: Collection of energy related information

X.C.1	Description

Energy used by the RAN nodes contributes the major part of the energy consumption in the PLMN network, while the energy consumption at 5GC NFs needs to be considered as well. Therefore, the network might be able to derive the information related to energy for the subsequent behaviours, i.e. exposure. The existing mechanism(s)/information elements are the starting point.

This key issue will study the following aspects:

-	Whether, how and what energy related information (including the information from OAM) of the RAN nodes can be collected by the 5GC, if the existing mechanism/information elements are not sufficient, to enable the subsequent exposure.

-	Whether, how and what energy related information (including the information from OAM) of the 5GC NF(s) can be collected, to enable the subsequent exposure. The existing mechanism(s)/information elements are the starting point.

NOTE:	Existing information related to Energy Efficiency from SA5 will be considered, and coordination with SA5 is needed for the study of this KI.

Editor's note: The Key Issue may be updated if other use cases are identified and there are common aspects regarding collection of energy related information.

# * * * * End of changes * * * *