3GPP TSG-WG SA2 Meeting #159	S2-2311328

# Xiamen, P.R. China, 09 - 13 October, 2023		(revision of S2-2310907)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4749 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Slice usage control clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Clarify that the UE keep slice usage policy stored associated the configured S-NSSAI. - Clarify that if no timer value received from UDM, then the timer can be set per local configuration or from PCF. - When the slice deregistration inactivity time value is updated, besides the notification of updated time value to UE, it is unclear how to handle the existing registered S-NSSAI or established PDU session. There are two mechanisms: - Option 1: the AMF/SMF applies the updated time value to the registered S-NSSAI or established PDU session directly. By doing this, it may cause the registered S-NSSAI and/or established PDU session be deregisterd or released directly due to the shorten value. This may be not the expection of the UE. To resolve this issue, it is proposed that the UE restart the time value when it received the upated time value and AMF/SMF restart the time value directly. - Option 2: the AMF/SMF directly deregister the already registered S-NSSAI and/or release the existing PDU session. Hence when the UE register the S-NSSAI next time or establish a new PDU session, the updated value applies. Above two mechanisms are all posible mechanism. Comparing two mechanisms, option 1 can avoid unnecessary session broken case. It is proposed to adopt opton 1. - There is a clause referecne 4.15.6.2 of TS23.502. Indeed it should be clause 4.15.6.3g. - For the slice usage control of the PDU session, it seems the existing PDU session inactivity timer is reused. However the existing timer is only used for PDU session UP deactivation. It is beneficial to have two independent timers, one for UP deactivation, the other for PDU session release. For example if the two timer use the same term, it means either selective UP deactivation or PDU session slice usage control is triggered when the timer is expired , i.e. they can not be used simulatnously or cause some complexity/error. One example, UP deactivation timer== 10 minutes. PDU session release==30 minutes. After 10 minutes no data tranmission, UPF notifies the SMF to deactivate the PDU session. Then SMF notifies UPF to set the next notification time is 20 minutes. However during that time, the data path may be activated again. So the SMF need remember the previous time value set at the UPF is 20 minutes and need be adjusted back to 10 minutes again. If the SMF forget to adjust that timer, then next UP deactivation timer==20 minutes. This is incorrect value. Also these two timer are set for different purpose. If we mix these two term it may cause confusion. For example the selective UP deactivation timer does not apply to the always-on PDU session. However the slice usage control can still apply to the always-on PDU session. If we mix two term, we need further clarification that the time for slice usage control is still applied to always-on PDU session. Considering above, it is propsed to distinguish these two timers, i.e. slice usage control timer is renamed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the slice usage policy at the UE side is kep stored same as the Configured S-NSSAI. - Clairfy that the only if time value from UDM not received, then it can be from local configuration or PCF. - Clarify how to handle the case if the deregistration inactivity time or PDU session slice inactivity time value is updated. - Update the incorrect reference. - Update the PDU session inactivity timer to PDU session slice usage inactivity timer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Unclear how long the slice usage policy is kept stored at the UE side. - Unclearhow the timer value is set. - Unclear how to handle if the time value is updated. - Incorrect TS reference. - Mixed usage of the UP deactivation timer for PDU session release. Selective UP deactivatoin and PDU session slice usage control can not be applied to the same PDU session simultanously or complexity logic at the SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.15.2, 5.15.15.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.15.15.2	UE Configuration of network-controlled Slice Usage Policy

The UE during the Registration procedure may indicate in UE MM Core Network Capability that it supports UE configuration of network-controlled Slice Usage Policy. If so, the AMF determines Slice Usage Policy for one (or more) Network Slice(s) for the UE and configures the UE with this information together with Configured NSSAI to control the usage of  this (or these) Network Slice(s).  The AMF may be locally configured with network Slice Usage Policy, or receive the policy from the AM-PCF, or per the information received from UDM for AF managed timer values (see clause 5.15.15.3 for more details).

The network-controlled Slice Usage Policy is provided to the UE in the Registration Accept or the UE Configuration Update Command and may include:

-	An indication, for one or more of S-NSSAI(s) of the HPLMN in the Configured NSSAI, whether the UE only registers with the Network Slice with the network when applications in the UE require data transmission in the Network Slice (i.e. the UE can only register the Network Slice only on demand and consider the Network Slice as on demand S-NSSAI).

NOTE:	All Other Network Slices in the Configured NSSAI are handled by the UE using UE specific policies (e.g. they may be registered irrespective of applications need).

-	For all on demand S-NSSAI(s) of the HPLMN in the Configured NSSAI, a deregistration inactivity timer that causes the UE to deregister the Network Slice after the last PDU Session associated with the S-NSSAI is released. This deregistration inactivity timer is started at the UE and AMF per access type when the last PDU Session associated with the S-NSSAI is released, or the Network Slice is included in the Allowed NSSAI and no PDU session is established. The deregistration inactivity timer is stopped and reset when the first PDU session is established or the S-NSSAI is removed from the Allowed NSSAI. The AMF and UE may locally remove the S-NSSAI from the Allowed NSSAI when the timer expires. The AMF may also send a UE Configuration Update Command to remove the slice from the Allowed NSSAI.

If the UE and network state became misaligned, the UE may, for example, request connectivity in a Network Slice which is no longer allowed. In this case, the AMF shall provide the updated Allowed NSSAI in a UE Configuration Update Command after rejecting the PDU Session establishment. The UE may then re-register with the Network Slice if needed.

The UE stores the Received Slice Usage Policy with the Configured NSSAI for the serving PLMN and this is kept stored for as long as a Configured NSSAI remains stored for the PLMN. When the Configured NSSAI is updated, the AMF may also provide a new Slice Usage policy to the UE.

The AMF receives deregistration inactivity timer values as described in clause 5.15.15.3. If the deregistration inactivity time value is updated, the AMF provides the updated value to the UE in the Registration Accept (if the registration procedure is ongoing) or the UE Configuration Update Command message. The AMF update the corresponding time value, and restarts the deregistration inactivity timer if it is ongoing. When the UE receives the updated deregistration inactivity time value and UE supports UE configuration of network-controlled Slice Usage Policy, it updates the corresponding time value, and restart the slice deregistration inactivity timer if it is ongoing.

5.15.15.3	Network-based per UE Network Slice usage behaviour control

The 5GC performs Network Slice usage monitoring to be able to enforce the release of inactive PDU Sessions, and deregistering of UEs from Network Slices with no PDU Sessions on them according to its own policies. In order to support usage monitoring for a Network Slice:

-	the AMF runs a slice deregistration inactivity timer per S-NSSAI and access type to deregister the Network Slice which is started when the Network Slice is not used by any PDU Session over the corresponding access type. The slice deregistration inactivity timer is stopped and reset when at least a PDU Session associated with the Network Slice is successfully established or the Network Slice is removed form the Allowed NSSAI. When the slice deregistration inactivity timer for a Network Slice over an access type expires, the AMF removes the Network Slice from the Allowed NSSAI over the access type by sending the UE Configuration Update Command to impacted UE(s).

-	the SMFs provide to UPFs that handle the PDU sessions in the Network Slice a PDU Session slice inactivity timer. The PDU Session slice inactivity timer is started after no data packet is transmitted or received and runs until the next data packet is transmitted or received which restarts the timer again. If the PDU Session slice inactivity timer expires before any packet is received or transmitted, the UPF reports this PDU Session slice inactivity event to the SMF to cause the SMF to release the PDU Session. While releasing the PDU session the SMF may indicate the release cause because of slice inactivity. When the AMF receives the notification of PDU Session release and it includes the release cause of slice inactivity and if the Network Slice of the released PDU Session is not used by other PDU Sessions (i.e. the last PDU Session using the Network Slice is released) over the corresponding access type, the AMF may trigger the UE Configuration Update procedure to remove the Network Slice from the Allowed NSSAI over that corresponding access type or start slice deregistration inactivity timer for the Network Slice.

If the PDU Session slice inactivity time value is updated and the related timer is already configured at the UPF before, the SMF provides the updated PDU Session slice inactivity time value to UPFs. The UPF updates the corresponding time value and restarts the PDU Session slice inactivity timer if it is ongoing.

If an S-NSSAI is dedicated for a single AF, and if authorized by operator policy to provide deregistration inactivity/PDU Session slice inactivity timer values for the S-NSSAI, the AF uses external parameter provisioning procedure to provide deregistration inactivity and PDU session slice inactivity timer values as described in clause 4.15.6.3g of TS 23.502 [3]. In this case, the AF provided timer values are stored in the UDM and provided to the AMF/SMF as part of subscription data for the corresponding S-NSSAI. If no AF is authorized to provide deregistration inactivity/PDU Session slice inactivity timer values for the S-NSSAI, i.e. no timer value received from UDM, the slice deregistration inactivity timer value and PDU Session slice inactivity timer value are either pre-configured in the AMF/SMF or received by the AMF/SMF during the AM Policy Association / SM Policy Association procedure respectively.

To enable a serving network to direct UEs to a preferred Network Slice, the AMF may request the UE to transfer a PDU Session from one S-NSSAI to another S-NSSAI as described in clause 5.15.19.

# * * * * End of changes * * * *