3GPP TSG-WG SA2 Meeting #159	S2-2311358

# Xiamen, China, Oct 09 – 13, 2023		(revision of S2-2304437, 10934, 11332)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4524 | rev | 2 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on support of CN based MT communication handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RedCAP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In "UE Triggered Connection Resume in RRC_Inactive procedure", the N2 Path Switch procedure may be performed when the UE is accessing a gNB which is different from the one that configured RRC_INACTIVE. However, when UE in RRC Inactive is configured with eDRX cycle longer than 10.24s and the gNB indicates that to the AMF before, how the signaling and/or data handled in CN (if available) are delivered to the UE is still missing. Hence, clarifications is necessary. - Align with RAN3 regarding the renamed N2 message to trigger RAN paging. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarification on N2 Path Switch procedure during the UE triggered Connection Resume in RRC_INACTIVE with long eDRX. - Alignments with RAN3 on the name of the N2 message for RAN paging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.2.2, 4.8.2.2b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.8.2.2	UE Triggered Connection Resume in RRC_INACTIVE procedure

The Connection Resume procedure is used by the UE in RRC_INACTIVE state, e.g. to transition to RRC_CONNECTED state or for Small Data Transmission while in RRC_INACTIVE as specified in TS 38.300 [9]. Triggers for the UE to initiate this procedure are defined in clause 5.3.3.2.5 of TS 23.501 [2].

Figure 4.8.2.2-1: Connection Resume in RRC_INACTIVE

1.	UE to NG-RAN: RRC message (Resume ID).

The UE initiates connection resume from RRC_INACTIVE state, see TS 38.300 [9]. The UE provides its Resume ID needed by the NG-RAN to access the UE's stored Context.

2.	[Conditional] NG-RAN performs UE Context Retrieval.

UE Context Retrieval is performed when the UE Context associated with the UE attempting to resume its connection is not locally available at the accessed NG-RAN. The UE Context Retrieval procedure via NG-RAN is specified in TS 38.300 [9].

3.	NG-RAN to UE: RRC messages.

NG-RAN determines whether the UE shall be transitioned to RRC_CONNECTED state or kept in RRC_INACTIVE (e.g. the latter in the case of Small Data Transmission as defined in TS 38.300 [9]).

4a.	[Conditional] N2 Path switch procedure.

If the accessed NG-RAN is able to retrieve the UE Context, the accessed NG-RAN node initiates N2 Path Switch procedure, i.e. steps 1 to 8 of clause 4.9.1.2.2 and including Xn data forwarding.

If the Connection Resume procedure is a response to RAN paging which is triggered by 5GC due to an N2 interface procedure, NG-RAN and 5GC handle the N2 interface procedure as a collision described in clause 4.9.1.2.

If Connection Inactive procedure with CN based MT communication handling (see clause 4.8.1.1a) has been performed previously then when the path switch procedure is performed downlink data or signalling delivery is triggered, if there is any.

4b.	[Conditional] N2 Notification,

4b.1	If the accessed NG-RAN is the same as the NG-RAN that configured RRC_INACTIVE and still has the UE context, NG-RAN sends:

-	an N2 Notification to the AMF indicating the UE is in RRC_CONNECTED, if an AMF requested N2 Notification (see clause 4.8.3); or

-	an MT Communication Handling request to the AMF indicating the UE is now reachable for downlink data and/or signalling if Connection Inactive procedure with CN based MT communication handling (see clause 4.8.1.1a) has been performed previously.

4b.2	The AMF invokes Nsmf_PDUSession_UpdateSMContext Request towards SMF indicating the Downlink data delivery for each PDU session with active user plane, if the AMF has requested data buffering as described in clause 4.8.1.1a.

4b.3	N4 session modification procedure is triggered by the SMF. If data buffering is handled in the UPF, the SMF updates the UPF with appropriate rules to trigger data delivery.

4b.4	The SMF sends the Nsmf_PDUSession_UpdateSMContext response.

4b.5	The AMF sends the N2 MT Communication Handling response message to NG-RAN.

If NG-RAN determines that the connection resume is for Small Data Transmission as defined in TS 38.300 [9] and step 4a or steps 4b.1 to 4b.5 have been performed, then NG-RAN keeps the UE in RRC_INACTIVE state and the UL/DL Small Data are transferred via the NG-RAN. Based on the procedures defined in TS 38.300 [9], if the UE is re-configured with RRC Inactive with eDRX>10.24s, the NG-RAN may send an N2 message to 5GC as described in step 2 in clause 4.8.1.1a so the CN can then handle mobile terminated (MT) communication.

# * * * * Second change * * * *

#### 4.8.2.2b	Network Triggered Connection Resume in RRC_INACTIVE with CN based MT communication handling

When the UE is in CM-CONNECTED with RRC_INACTIVE state with CN based mobile terminating (MT) communication handling, high latency communication as described in clause 5.31.8 of TS 23.501 [2] is applied.

This procedure may be triggered by MT data, or a N1 procedure from SMF and UPF as shown in Figure 4.8.2.2b-1. When the procedure is triggered by other NFs (e.g. SMSF, LMF, GMLC), the UPF (or SMF) in the following figure should be replaced by the respective NF (the corresponding service operations used by other NFs when they communicate with AMF may also be different from the service operations used by SMF/UPF).

During the procedure, the NG-RAN (i.e. gNB) performs RAN paging towards the UE based on the N2 message from the AMF in order to trigger the UE triggered Connection Resume procedure in clause 4.8.2.2.

Figure 4.8.2.2b-1: Network Triggered Connection Resume for UE in RRC_INACTIVE with CN based MT communication handling

1a.	When downlink data is received and the SMF/UPF is requested to perform buffering as specified in clause 4.8.1.1a, the UPF/SMF checks with AMF for the possibility of data delivery, similar to step 2 of clause 4.24.2 with the following differences:

-	In the Namf_MT_EnableUEReachability the SMF may also send the PPI, the ARP and the 5QI, and/or QFI of the QoS Flow of the PDU Session which triggered the request for paging policy differentiation as defined in clause 5.4.3.2 of TS 23.501 [2].

If the SMF, while waiting for UE triggered Connection Resume indication or a reject response (with Estimated Maximum Wait time) from the AMF, receives any additional Data Notification message due to additional data packets for another QoS Flow associated with a higher priority (i.e. ARP priority level) than the priority indicated to the AMF in the previous Namf_MT_EnableUEReachability, or the SMF derive a different Paging Policy Indicator according to the additional Data Notification, the SMF invokes a new Namf_MT_EnableUEReachability indicating the higher priority or different Paging Policy Indicator to the AMF. The information contained in the new Namf_MT_EnableUEReachability request overrides the information from the previous Namf_MT_EnableUEReachability request that is stored in the AMF. If the SMF receives any additional Data Notification messages due to additional data packets for another QoS Flow associated with same or lower priority than the priority indicated to the AMF in the previous Namf_MT_EnableUEReachability or if the SMF has sent the second Namf_MT_EnableUEReachability message indicating the higher priority and receives additional downlink data packets for this UE, the SMF buffers these downlink data packets and does not send a new Namf_MT_EnableUEReachability meassage.

-	The AMF determines if the UE is reachable based on the stored eDRX values for RRC_INACTIVE state provided by NG-RAN in clause 4.8.1.1a. If the UE is unreachable, the AMF stores the information received in the Namf_MT_EnableUEReachability request and provides the Estimated Maximum Wait time in the response message based on the eDRX values for RRC_INACTIVE in AMF (steps 2-5 are postponed until the UE becomes reachable). If the UE is considered reachable, step 2 is executed immediately.

NOTE:	This handling is similar to CM-IDLE with eDRX. When the AMF provides the Estimated Maximum Wait time, it can consider the time needed for RRC level procedures (e.g. RRC RNA update procedure) when UE wakes up from the eDRX cycle.

2.	When the AMF determines that the UE is reachable, the AMF sends an RAN Paging Request message to NG-RAN with the request for the UE's RRC connection to be resumed. The AMF may include the following parameter(s) the PPI, the ARP and the 5QI, and/or QFI of the QoS Flow of the PDU Session ID in the RAN Paging Request message to trigger and enable RAN paging.

3.	NG-RAN performs RAN paging towards the UE considering the parameters provided by the AMF.

4.	When the UE receives RAN paging, it initiates the UE triggered Connection Resume procedure and NG-RAN notifies CN as specified in clause 4.8.2.2 including the N2 Notification in step 3b.

5.	The UPF triggers downlink data delivery if there is any. The AMF sends downlink NAS messages if there is any.

# * * * * End of changes* * * *