3GPP TSG-SA WG2 Meeting #159	S2-2311387

# Xiamen, China, 09 – 13 October 2023

Title:	[Draft] Reply LS on 5G ProSe UE-to-UE relay discovery

Response to:	LS on 5G ProSe UE-to-UE relay discovery (S2-2310091/C1-236448)

Release:	Rel-18

Work Item:	5G_ProSe_Ph2

Source:	SA2

To:	CT1

Cc:

Contact Person:

Name:	Qiang Deng

Tel. Number:

E-mail Address:	dengqiang1 AT catt DOT cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.304 CRxxx1, CRxxx2, CRxxx3

1. Overall Description:

SA2 would like to thank CT1 on the LS on 5G ProSe UE-to-UE relay discovery (S2-2310091/C1-236448).

SA2 has discussed the questions and would like to provide the answers as followings.

- According to clause 6.3.2.1 of TS 23.304, during 5G ProSe direct discovery procedures, the announcing UE/ discoverer UE/ discoveree UE "includes the relay_Indication if 5G ProSe UE-to-UE Relay(s) can broadcast its User Info ID during the 5G ProSe UE-to-UE Discovery".

However, the user info ID, used in the 5G ProSe UE-to-UE relay discovery, cannot be obtained by the 5G ProSe UE-to-UE relay UE during the 5G ProSe direct discovery. So the 5G ProSe UE-to-UE relay UE cannot identify the same 5G ProSe end UE who sent the relay indication by using this user info ID in the 5G ProSe UE-to-UE relay discovery. This makes the relay indication mechanism cannot work. Then:

Question 1: How can this issue be resolved?

SA2 answer to Q1: SA2 has agreed the attached CRxxxx. The "relay_indication" was removed from the description in clause 6.3.2.1 of TS 23.304.

- According to clause 6.3.2.4.4 of TS 23.304, for candidate 5G ProSe UE-to-UE Relay Discovery to support the negotiated Relay reselection, the discoverer End UE discovers a candidate 5G ProSe UE-to-UE Relay without sending the RSC and the User Info ID of the discoveree 5G ProSe End UE. Then:

Question 2: How can the 5G ProSe UE-to-UE relay UE tell that the 5G ProSe UE-to-UE Relay Discovery Solicitation message is for the relay reselection so that it can skip finding the target end UE and respond to the source end UE directly?

Question 3: How can the 5G ProSe UE-to-UE relay UE know which service the discoverer end UE wants to use before accepting the 5G ProSe UE-to-UE Relay Discovery Solicitation message?

SA2 answer to Q2 and Q3: SA2 has agreed the attached CRxxxx. SA2 would like to clarify that the procedure defined in clasue 6.3.2.4.4 of TS 23.304 was meant to be used by the "responding 5G ProSe End UE", as in TS 23.304 clause 6.7.4.2 step 4 or clause 6.7.4.3 step 7, and should terminate at the candidate relay UE.

Therefore, for Question 2, based on the attached CRxxxx, the relay UE determines to skip the step 2 and 3 in clasue 6.3.2.4.4 of TS 23.304, based on the absence of discoveree 5G ProSe End UE User Info ID, and the matching of its own User Info ID with the candidate 5G ProSe UE-to-UE Relay User Info ID in the received message.

For Question 3, the updated procedure has the RSC included in the solicitation message.

- According to clause 5.8.4.2, 6.4.3.7.1 and 6.4.3.7.4 of TS 23.304, the user info of the target 5G ProSe end UE (i.e. discoveree end UE) is mandatory in 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) and Direct Communication Request message for 5G ProSe UE-to-UE Relay Communication; while it is optional in the Direct Communication Request message for 5G ProSe UE-to-UE relay communication with integrated discovery. Then:

Question 4: Why are different approaches applied to the user info of the target 5G ProSe end UE in the above procedures?

SA2 answer to Q4: The procedure in clasue 6.4.3.7.4 of TS 23.304 was meant for the use with Integrated Discovery, i.e. there was no prior standalone 5G ProSe UE-to-UE Relay discovery. Therefore, it was model like the procedure in clause 6.4.3.1 of TS 23.304, with both the UE oriented option and ProSe Service oriented option supported. That was the reason for an optional User Info for the target 5G ProSe End UE.

On the other hand, the procedure in clause 5.8.4.2 and 6.4.3.7.1 of TS 23.304 were for the operation with a separate 5G ProSe UE-to-UE Relay discovery before the link establishment. In such case, the target 5G ProSe End UE is always known. Therefore, the User Info would be mandatory for these procedures.

2. Actions:

To CT1 group.

ACTION: 	SA2 kindly asks CT1 to take the above answers into account and develop the corresponding stage 3 specifications.

3. Date of Next SA2 Meetings:

TSG-SA2 Meeting #160	13-17 November 2023	Chicago, USA

TSG-SA2 Meeting #Ad Hoc	21-26 January 2024 	Electronic