3GPP TSG-SA WG2 Meeting #159	S2-2311399

# Xiamen, China, 09 – 13 October 2023								  	    (Revision of S2-2310410)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0364 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Issue on U2U relay reselection triggered by PC5 RLF or PC5 link release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the agreed running CR for TS38.300 (R2-2309339), the U2U remote UE can trigger the U2U relay reselection based on the cases as quoted below. The U2U Remote UE may triggers U2U Relay reselection in the following cases: - The PC5 signal strength of the current U2U Relay UE is below a (pre)configured signal strength threshold; (Case 1) - When U2U Remote UE detects PC5 RLF; (Case 2) - When U2U Remote UE receives the PC5 RLF indication from the U2U Relay UE; (Case 3) - When U2U Remote UE receives a PC5-S link release message from U2U Relay UE; (Case 4) - Indicated by the upper layer of the U2U Remote UE. (Case 5) This CR is to take TS38.300 into account for 5G ProSe UE-to-UE Relay reselection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 6.7.4.1, the statement of “the 5G ProSe End UEs keep performing the measurement of the signal strength of PC5 unicast link with the 5G ProSe UE-to-UE Relay for relay reselection” is modified as “the 5G ProSe End UEs may trigger the 5G ProSe UE-to-UE Relay reselection based on conditions (e.g. the measured signal strength of PC5 unicast link with the 5G ProSe UE-to-UE Relay) as specified in TS 38.300 [12]”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Triggering of the 5G ProSe UE-to-UE Relay reselection may not be aligned with TS38.300 and thus leading to failure on the U2U relay communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

6.7.4.1	General

After being connected to the 5G ProSe UE-to-UE Relay, the 5G ProSe End UEs may trigger the 5G ProSe UE-to-UE Relay reselection based on conditions (e.g. the measured signal strength of PC5 unicast link with the 5G ProSe UE-to-UE Relay) as specified in TS 38.300 [12].

For 5G ProSe UE-to-UE Relay reselection, a 5G ProSe UE-to-UE Relay may be discovered by either the discovery procedures defined in clause 6.3.2.4 or by the negotiated 5G ProSe UE-to-UE Relay reselection procedure defined in clause 6.7.4.2 or clause 6.7.4.3.

In the negotiated UE-to-UE Relay reselection defined in clause 6.7.4.2 or clause 6.7.4.3, one 5G ProSe End UE initiates the UE-to-UE Relay reselection procedure, the 5G ProSe End UEs can negotiate a new 5G ProSe UE-to-UE Relay using the existing connection and to establish the communication via the reselected 5G ProSe UE-to-UE Relay prior to releasing the communication via the current 5G ProSe UE-to-UE Relay.

END OF CHANGES