3GPP TSG-WG SA2 Meeting #159	S2-2311474

Xiamen, P.R. China, 09 - 13 October, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4950 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Providing both ULI and Additional ULI to other NFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is proposed the following changes in clause 5.35A.6: - Clarificaitons on how the IAB-donor-CU determines a UE is served by an authorized MBSR. Currently, the IAB-donor-CU’s determination that a UE is served by an authorized MBSR is missing. The IAB-donor-CU knows an MBSR operates as an IAB node with mobility based on the MBSR indication included in the RRC connection. When the the MBSR is authorized based on the MBSR authorization indication provided by the AMF and a UE is served by an MBSR cell of this MBSR, the IAB-donor-CU shall provide 'Additional ULI'. - Adressing the following Editor's note: Editor's note: If there is a need to provide both ULI and Additional ULI to other NFs will be specified. Based on the descriptions of clause 5.9 of TS 23.273, the AMF serving the UE also provides LMF with the additional ULI Information received from NG-RAN, so that the LMF can initiate the positioning procedure to obtain the location information of the MBSR.Therefore, it is proposed to update clause 5.35A.6 to align with TS 23.273. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarificaitons on how the IAB-donor-CU determines a UE is served by an authorized MBSR. - Addressing Editor's note about providing both ULI and Additional ULI to other NFs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current descriptions are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.35A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS23.502 CR4195 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.35A.6	Providing cell ID/TAC of MBSR for services

The TAC and cell ID broadcasted by the MBSR cell(s) are configured as specified in TS 38.470 [165].

After the MBSR is authorized as defined in 5.35A.4, when a UE is served by a cell of this MBSR, the IAB-donor-CU may provide 'Additional ULI' in addition to User Location Information, in N2 messages. The 'Additional ULI' is the ULI of the IAB-UE. The AMF may consider the 'Additional ULI' when it determines UE location and manages the UE location related functions (e.g. Mobility Restrictions).

When the AMF provides user location information to other NFs (e.g. LMF as specified in clause 5.9 of TS 23.273 [87]) for a UE connected via MBSR, the AMF may also send the Additional ULI received via N2 messages.

# * * * * End of changes * * * *