3GPP TSG-WG SA2 Meeting #159	S2-2311500

# Xiamen, China, October 9 – 13, 2023		(revision of S2-2310478)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2114 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on deregistration procedure of 5G-RG serving AUN3 devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedure of 5G-RG deregistration via W-5GAN when it is also serving AUN3 devices needs to update to clarify how the AUN3 device behind 5G-RG to be deregistered when 5G-RG’s is deregistered. Currently, 5G-RG’s context is released before W-AGF trigger AUN3 device’s deregistration. In this case, W-AGF may have no information to trigger the deregistration of the AUN3 devices behind the 5G-RG. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that the W-AGF needs to temporarily maintain the mapping of AUN3 devices and 5G-RG to perform step 5 and 6 when the 5G-RG’s context is released. However, how to temporarily maintain that information is based on implementation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AUN3 devices cannot be deregistered when the 5G-RG is deregistered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

7.2.8.3	5G-RG Deregistration via W-5GAN when it is also serving AUN3 devices

Figure 7.2.8.3-1: 5G-RG Deregistration procedure via W-5GAN when serving AUN3 devices

1-3.	The same as steps 1-3 for Figure 7.2.1-1 with the following modification:

In the case of 5G-RG initiated deregistration, 5G-RG shall first deregister each of the registered AUN3 devices connected to it (if any) before initiating the deregistration of itself.

4.	W-AGF controls if there exist any AUN3 devices connected to the 5G-RG that are registered to the 5GC through the W-AGF. If there are no AUN3 devices, then steps 5 and 6 shall be ignored.

NOTE1:	How the W-AGF temporarily maintains the mapping of AUN3 devices and 5G-RG to perform step 5 and 6 when the 5G-RG’s context is released is based on implementation.

5.	[conditional] For each AUN3 device identified in step 4, the W-AGF shall send AN Release request as specified in clause 4.2.6 of TS 23.502 [3]. Here, the cause should indicate the disconnection of 5G-RG.

6.	[conditional] Upon receiving the AN Release request with the cause specified in step 5, the AMF shall initiate deregistration of the AUN3 device as specified in clause 4.2.2.3.3 of TS 23.502 [3].

7.	The W-AGF may initiate the release of the signalling connection between 5G-RG and W-AGF.

NOTE2:	Whether this step is needed, and if so, the details of this step are defined by BBF and/or CableLabs.

8.	The RG disconnects the AUN3 device. How this is done is outside of 3GPP scope.

# * * * * End of changes * * * *