3GPP TSG-SA WG2 Meeting #159	S2-2311518

(revision of S2-2311170)

# Xiamen, China, Oct 9-13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2124 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SUPI and SUCI for 5G-BRG support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2 |  |  |  |  |  | Date: |  |  | Sep 28, 2023 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 4.7.3 of TS 23.316 mentions, “The SUPI for an 5G-BRG shall contain an IMSI”. However, the SUPI for an 5G-BRG may contain either an IMSI or a GLI. In the context of non-3GPP access to SNPN using the 5G system, there is the option to use additional EAP methods and NAI-based SUPI for primary authentication. Concequently, a 5G-BRG with a SUPI that contains a GLI in NAI format might use 3GPP access to establish a conneciton to 5G. Furthermore, as specified in Clause 4.3.3.2 of TS 23.316, “for subscriptions for 5G-BRG, GLI is used to describe the Forbidden Area”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In PLMN, the SUPI for an 5G-BRG shall contain an IMSI, as described in clause 5.9.2 of TS 23.501 [2]. The SUPI for accessing SNPN is defined in Clause 4.16.1. The SUPI for accessing PNI-NPN is defined in Clause 4.16.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current wording of the statement might lead to misunderstandings regarding the authentication capabilities of SNPNs. It limits the flexibility and diverse use-cases of SNPNs to meet 5G requirements, such as utilizing additional EAP methods and NAI-based SUPI for primary authentication. Furthermore, the stipulation would be inadequate when a 5G-BRG subscription uses GLI to define Forbidden Area. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 4.7	Identifiers

### 4.7.1	General

As described in TS 23.501 [2], each subscriber in the 5G System shall be allocated one 5G Subscription Permanent Identifier (SUPI) for use within the 3GPP system. As described in TS 23.501 [2], each FN-RG or 5G-RG accessing the 5G System shall be assigned a Permanent Equipment Identifier (PEI).

The clauses below describe specific aspects for supporting 5G-RG and FN-RG.

### 4.7.2	SUPI and SUCI for 5G-BRG support

In PLMN, the SUPI for an 5G-BRG shall contain an IMSI, as described in clause 5.9.2 of TS 23.501 [2]. The SUPI for accessing SNPN is defined in Clause 4.16.1. The SUPI for accessing PNI-NPN is defined in Clause 4.16.2.

The SUCI provided by the 5G-BRG to the network contains the concealed SUPI, as described in TS 33.501 [11].

### 4.7.3	SUPI and SUCI for FN-BRG support

The SUPI for an FN-BRG subscription shall, based on operator configuration, either contain an IMSI or a GLI as defined in clause 4.7.8. A SUPI containing a GLI takes the form of a NAI whose user part is the GLI and whose realm part is an identifier of the operator owning the subscription.

The SUCI provided by the W-AGF to the 5GC for FN-BRG always corresponds to a SUPI containing a GLI. This SUCI acts as pseudonym of the SUPI and the UDM performs a mapping to the actual SUPI that, depending on operator configuration, contains either an IMSI or the same GLI that was provided in the SUCI.

As described in TS 23.003 [14], the SUCI also contains an identifier of the Home network, i.e. the identifier of the operator owning the subscription.

### 4.7.4	SUPI and SUCI for 5G-CRG and FN-CRG support

The SUPI for a FN-CRG subscription shall, based on operator configuration, contain either an IMSI, as described in clause 5.9.2 of TS 23.501 [2], or a GCI (Global Cable identifier defined in clause 4.7.9).

The SUPI for a 5G-CRG subscription shall, based on operator configuration, contain either an IMSI, as described in clause 5.9.2 of TS 23.501 [2], or a GCI (Global Cable identifier defined in clause 4.7.9).

Only 5G-CRG whose SUPI corresponds to an IMSI may use 3GPP access to connect to 5GC.

A SUPI containing a GCI takes the form of a NAI where the user part is the GCI and the realm part is an identifier of the operator managing the subscription.

NOTE 1:	The realm part used to identify the operator managing the subscription can differ depending on whether the wireline access network belongs to a PLMN or SNPN. The NAI format for SUPI containing GCI for PLMN and SNPN is defined in TS 23.003 [14].

The SUCI provided by the 5G-CRG to the network contains the concealed SUPI, as described in TS 33.501 [11].

The SUCI provided to the network for FN-CRG support always corresponds to a SUPI containing a GCI. This SUCI acts as pseudonym of the SUPI and the UDM performs a mapping to the SUPI that, depending on operator configuration, contains either an IMSI or the same GCI than in the SUCI.

As described in TS 23.003 [14], for both cases where the SUCI contains an IMSI or contains a GCI, the SUCI contains an identifier of the Home network i.e. an identifier of the operator managing the subscription.

NOTE 2:	If the SUCI contains an IMSI, the identifier of the operator managing the subscription is carried in the MCC/MNC part of the IMSI as defined in TS 23.003 [14].

# * * * * End of changes * * * *