3GPP TSG-SA WG2 Meeting #159	S2-2311428

# October 09 – 13, 2023, Xiamen, China										 (revision of S2-2310251)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0109 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New PQI values for A2X communication over PC5 reference point |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Reply LS to RAN2 on BRID and DAA broadcast over LTE and NR PC5 (S2-2307781) includes the following SA2 answer. SA2 answer: SA2 discussed that defining new PQI(PC5 5QI) value(s) is beneficial for BRID and DAA over NR PC5 so that the AS layer can distinguish A2X services from V2X services if differentiated handling is needed. The related CR work will be progressed in Rel-18. For BRID and DAA over LTE PC5, it is considered that the existing QoS framework can be used. TS 22.125 includes the following service and performance requirements that can be considered for A2X communication over PC5 reference point. [R-5.2.2-011] The 3GPP system shall support a direct UAV to UAV local broadcast communication transport service which can transmit messages with an end-to-end latency of at most 100ms. There are four control modes considered in the C2 communication for the UAV operation that are with different requirements, e.g. on message intervals, sizes, and end to end latencies, etc., including steer to waypoints, direct stick steering, automatic flight by UTM and approaching autonomous navigation infrastructure. - Steer to waypoints: the control message contains flight declaration, e.g. waypoints, sent from the UAV controller or UTM to the UAV. The control mode is used in both of direct C2 communication and network-assisted C2 communication. - Direct stick steering: the control message contains direction instructions sending from the UAV controller to the UAV while optionally video traffic is provided as feedback from the UAV to the UAV controller. The control mode is used in both of direct C2 communication and network-assisted C2 communication. - … Table 7.2-1 KPIs for command and control of UAV operation Control Mode; Function; Typical Message Interval; Max UAV ground speed; Typical message Size (note 1); End to end Latency; Reliability (note 2); Positive ACK (note 8). Steer to waypoints (note 3); UAV terminated C2 message; >=1 s; 300 km/h; 100 byte; 1 s; 99.9%; Required. UAV originated C2 message (note 4); 1 s; 84-140 byte; 1 s; 99.9%; Not Required. Direct stick steering (note 5); UAV terminated C2 message; 40 ms (note 6); 60km/h; 24 byte; 40 ms; 99.9%; Required. UAV originated C2 message (note 7); 40 ms; 84-140 byte; 40 ms; 99.9%; Not Required. …. NOTE 1: Message size is at the application layer and excludes any headers and security related load. The numbers shown are typical as message size depends on the commands sent and is implementation specific. NOTE 2: Message reliability is defined as the probability of successful transmission within the required latency at the application layer while under network coverage. NOTE 3: Video is neither required nor expected to be used for steering in this mode. NOTE 4: It may be possible to transmit this message on an event driven basis (e.g. approaching a geo fence). A status message may, but is not required to, be sent as a response to a control message. NOTE 5: A video feedback is required for this mode. The KPIs for video are defined in table 7.2-2. NOTE 6: UAVs on-board controllers typically update at either 50Hz (20ms) or 25Hz (40ms). NOTE 7: A status message may, but is not required to, be sent as a response to a control message A 1Hz slow mode also exists. NOTE 8: Positive ACK is sent to the originator of the message (i.e. UAV controller and / or the UTM). The 5G system makes no assumption whether an appropriate ACK is sent by the application layer. …. Table 7.2-2 KPIs for video used to aid UAV control. Scenario (note 2); Data rate; End to end Latency; Reliability (note 1); Direction; Positive ACK required. VLOS (visual line of sight); 2 Mpbs at 480 p, 30 fps; 1 s; 99.9%; Sent by UAV; Not Required. Non-VLOS; 4 Mbps at 720 p, 30 fps; 140 ms; 99.99%; Sent by UAV; Not Required. NOTE 1: Message reliability is defined as the probability of successful transmission within the required latency. NOTE 2: Maximum UAV speed is same as control mode of direct stick steering in 7.2-1. Therefore, new PQI values need to be defined for UAS. Please note that the following PQI values are already defined in TS 23.287 and TS 23.304. • GBR: 21~23 (TS 23.287); 24~26, 32~34 (TS 23.304) • Non-GBR: 55~59 (TS 23.287); 60, 61, 70, 71 (TS 23.304) • Delay Critical GBR: 90, 91 (TS 23.287); 92, 93 (TS 23.304) |
| Summary of change: |  | §3.2 - Add PQI. §6.2.4.1 - Fix the clause title, i.e. from V2X to A2X. - Extend the referred clauses of TS 23.287 from 5.4.1 to 5.4.3. - Define new PQI values. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PQI values for A2X communication over PC5 may be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

A2X	Aircraft-to-Everything

AAM	Area Airspace Manager

BRID	Broadcast Remote Identification

BVLOS	Beyond Visual Line of Sight

C2	Command and Control

DAA	Detect And Avoid

DDAA	Direct Detect And Avoid

MBS	Multicast/Broadcast Service

NRID	Networked Remote Identification

PQI	PC5 5QI

RID	Remote Identification

TPAE	Third Party Authorized Entity

UAS	Uncrewed Aerial System

UAV	Uncrewed Aerial Vehicle

USS	UAS Service Supplier

UTM	Uncrewed Aerial System Traffic Management

UUAA	USS UAV Authorization/Authentication

UUID	Universal Unique Identifier

* * * * Start of Next Change * * * *

6.2.4.1	QoS handling for A2X communication over PC5 reference point

A2X leverages what is defined for V2X in clauses 5.4.1 to 5.4.3 of TS 23.287 [11] with the following differences:

-	V2X is replaced by A2X.

-	Groupcast mode communication is not supported.

-	Definitions for PC5 QoS handling are described in clause 6.2.4.

-	PC5 QoS configuration parameters are described in clause 6.2.1.2.1.

-	Definitions for A2X communication over PC5 Reference point are described in clause 6.2.2.1.

-	Procedures for A2X unicast mode communication over PC5 Reference point are described in clause 6.3.3.2.

-	Procedures for A2X Delivery of PC5 QoS parameters to NG-RAN are described in clause 6.3.5.7.

-	PQI values are defined for A2X communication over PC5 reference point. The one-to-one mapping of standardized PQI values that are defined to PC5 QoS characteristics is specified in table 6.2.4.1-1.

Table 6.2.4.1-1: Standardized PQI values to QoS characteristics mapping

| PQI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | GBR (NOTE 1) | 4 | 50 ms | 10-2 | N/A | 2000 ms | Unicast mode Detect and Avoid with lower latency requirement |
| 41 |  | 4 | 100 ms | 10-2 | N/A | 2000 ms | Unicast mode Detect and Avoid |
| 42 |  | 3 | 1000 ms | 10-3 | N/A | 2000 ms | C2 Communication – Steer to waypoints; C2 Communication – video used to aid UAV control (VLOS) |
| 43 |  | 3 | 40 ms | 10-3 | N/A | 2000 ms | C2 Communication – Direct stick steering |
| 44 |  | 3 | 140 ms | 10-4 | N/A | 2000 ms | C2 Communication – video used to aid UAV control (Non-VLOS) |
| 62 | Non-GBR | 4 | 100 ms | 10-2 | N/A | N/A | Broadcast Remote ID with lower latency requirement |
| 63 |  | 4 | 500 ms | 10-2 | N/A | N/A | Broadcast Remote ID |
| 64 |  | 4 | 50 ms | 10-2 | N/A | N/A | Broadcast mode Detect and Avoid with lower latency requirement |
| 65 |  | 4 | 100 ms | 10-2 | N/A | N/A | Broadcast mode Detect and Avoid |
| NOTE 1: GBR PQIs can only be used for unicast PC5 communications. |  |  |  |  |  |  |  |

* * * * End of Changes * * * *