3GPP TSG-WG SA2 Meeting #159	S2-2311692

09 - 13 October, 2023, Xiamen, China

# Title:	LS to Clarify the LADN Service Area Configuration in UDM Subscription information

# Response to:

# Release:	Rel-18

# Work Item:	GMEC

Source:	SA2

To:	CT4

Cc:

Contact Person:

Name:	Ashok Kumar Nayak

E-mail Address:	ashok.nayak@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.502 CR4527

1. Overall Description:

Proposal 1:

SA2 observed that CT4 has defined LADN service area in SMF Selection Subscription data but SA2 would like to clarify that the LADN service area is meant for AMF also for other functionalities than SMF selection. AMF as consumer NF receives this LADN service area information from UDM and provision to UE if the UE has indicated the capability to support the feature for LADN per DNN and S-NSSAI. SA2 has specified that any further update to LADN service area information will result in AMF sending the updated LADN service area information to UE using UE Configuration Update procedure and hence it is best if this information is configured under subscription data type “Access and Mobility Subscription data” instead of “SMF selection Subscription data”.

SA2 suggest CT4 to refer to the attached CR for provisioning of LADN Service Area information and share the view.

Proposal 2:

SA2 observed that CT4 (C4-232603) has defined LADN service area in DnnInfo, which is used in SMF Selection Subscription data. SA2 understands that the LADN service area in SMF Selection Subscription data defined by CT4 is only applicable for 5G VN group member according to the cited TS23.501CR 4313 (S2-2305613). But SA2 would like to clarify that

-	LADN service area provisioned for a DNN and S-NSSAI for a group of UEs: The UDM stores LADN service area for a group of UEs as part of the DNN and S-NSSAI specific Group Parameters.

In case of SMF selection for cross-SMFs 5G VN group communication, the AMF may obtain the LADN service area for the 5G VN group from the DNN and S-NSSAI specific Group Parameters linked to a 5G VN group. The LADN service area in SMF Selection Subscription data defined by CT4 for 5G VN group member will be duplicated in this case.

In case of LADN Information provisioning for each UE in the group, the AMF will consider the LADN service area within the DNN and S-NSSAI specific Group Parameters for this group. If LADN service area in SMF Selection Subscription data is extended to non-5G VN group member UEs, then this will cause duplications.

-	LADN service area provisioned for a DNN and S-NSSAI for an individual UE: SA2 believes this information is configured under subscription data type “Access and Mobility Subscription data”  at UDM. AMF as consumer NF receives this LADN service area information from UDM and provision to UE if the UE has indicated the capability to support the feature for LADN per DNN and S-NSSAI. However, if SA2 specified LADN service area per DNN and S-NSSAI in Access and Mobility Subscription data in this case and LADN service area in SMF Selection Subscription data is extended to non-5G VN group member UEs, then this will cause duplications.

SA2 observes existing and potential duplications with LADN Service Area information in SMF Selection Subscription data, thus suggests CT4 to refer to the attached CR for provisioning of LADN Service Area information and remove the LADN Service Area information from SMF Selection Subscription data. Please share the view.

2. Actions:

To CT4 group.

ACTION: 	SA2 kindly asks CT4 to look at the attached CR regarding provisioning of LADN service area information to UDM and share their view and if needed to update the stage 3 specification.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting#160	November 13 – 17, 2023	Chicago, USA

TSG-SA2 Ad-hoc                            22 January – 29 January 2023                Electronic meeting