3GPP SA WG2 Meeting #159	S2-2311903

09 - 13 October, 2023, Xiamen, P.R. China

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5001 | rev | 3 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to UPF Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2023-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Nupf_GetPrivateUEIPAddr API naming does not match all the supported parameters e.g. SUPI may be reported if available in UPF while SUPI is not private UE IP address. The Nupf_GetPrivateUEIPaddr API definition is not extensible. May require another new OpenAPI(s) servicing the similar information retrieval (request/response model) functionality with different parameters. Port number is needed for NAPT (Network Address Port Translation), while optional for NAT (Network Address Translation), related description in clause 5.8.2.17 could be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the Nupf_GetPrivateUEIPaddr service as Nupf_GetUEPrivateIPaddrAndIdentifiers service to support UE information retrieval in procedure and UPF service description. Update NAT to NAPT in clause 5.8.2.17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not matched service naming of Nupf_GetPrivateUEIPaddr API and not extendable for UE information retrieval. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.17, 6.2.3, 7.2.29 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 5.8.2.17	Data exposure via Service Based interface

The UPF may expose information by means of UPF event exposure service as described in TS 23.502 [3] clause 5.2.26.2, via a service-based interface directly. The NF consumer, which may receive UPF event notifications, are AF/NEF, TSNAF/TSCTSF and NWDAF/DCCF/MFAF.

When the UPF supports the data exposure via the SBI interface, it may register its NF profile to the NRF including the UPF Event Exposure services. For the UPF Event Exposure services, the NF profile includes the related event ID(s).

For data collection from UPF (see clause 4.15.4.5 of TS 23.502 [3]), NF consumer do the subscription to the UPF directly or indirectly via SMF. A consumer may subscribe to the UPF event exposure service directly only for data collected for "any UE" e.g. to collect user data usage information for NWDAF NF Load analytic (see clause 6.5 of TS 23.288 [86]), and if the subscription is not including any of the following parameters: AoI, traffic filtering, BSSID/SSID, Application ID.

UPF selection is further described in clause 6.3.3.1.

To minimize the impact of event notification to UPF data processing, the event subscription may include Reporting suggestion information. The Reporting suggestion information includes Report urgency and Reporting window information. Reporting urgency information represents whether this event report can be delay tolerant, i.e. the event report can be delayed. If the Reporting urgency information indicates "delay tolerant", the Reporting time is also provided, which defines the last valid reporting time, and UPF shall report the detected event before the last valid time. Per Reporting suggestion information UPF can concatenate several notification messages to the same notification endpoint in one notification message.

The UPF may also expose UE information by means of the Nupf_GetUEPrivateIPaddrAndIdentifiers service as described in TS 23.502 [3] clause 5.2.26.3. An UPF which is deployed with NAPT (Network Address Port Translation) functionality may support to provide the 5GC UE IP address to NEF based on NEF request containing public IP address and port number using the Nupf_GetUEPrivateIPaddrAndIdentifiers service as described in clause 4.15.10 of TS 23.502 [3] for AF specific UE ID retrieval.

# *** 2nd Change ***

### 6.2.3	UPF

The User plane function (UPF) includes the following functionality. Some or all of the UPF functionalities may be supported in a single instance of a UPF:

-	Anchor point for Intra-/Inter-RAT mobility (when applicable).

-	Allocation of UE IP address/prefix (if supported) in response to SMF request.

-	External PDU Session point of interconnect to Data Network.

-	Packet routing & forwarding (e.g. support of Uplink classifier to route traffic flows to an instance of a data network, support of Branching point to support multi-homed PDU Session, support of traffic forwarding within a 5G VN group (UPF local switching, via N6, via N19)).

-	Packet inspection (e.g. Application detection based on service data flow template and the optional PFDs received from the SMF in addition).

-	User Plane part of policy rule enforcement, e.g. Gating, Redirection, Traffic steering).

-	Lawful intercept (UP collection).

-	Traffic usage reporting.

-	QoS handling for user plane, e.g. UL/DL rate enforcement, Reflective QoS marking in DL.

-	Uplink Traffic verification (SDF to QoS Flow mapping).

-	Transport level packet marking in the uplink and downlink.

-	Downlink packet buffering and downlink data notification triggering.

-	Sending and forwarding of one or more "end marker" to the source NG-RAN node.

-	Functionality to respond to Address Resolution Protocol (ARP) requests and / or IPv6 Neighbour Solicitation requests based on local cache information for the Ethernet PDUs. The UPF responds to the ARP and / or the IPv6 Neighbour Solicitation Request by providing the MAC address corresponding to the IP address sent in the request.

-	Packet duplication in downlink direction and elimination in uplink direction in GTP-U layer.

-	NW-TT functionality.

-	High latency communication, see clause 5.31.8.

-	ATSSS Steering functionality to steer the MA PDU Session traffic, refer to clause 5.32.6.

NOTE:	Not all of the UPF functionalities are required to be supported in an instance of user plane function of a Network Slice.

-	Inter PLMN UP Security (IPUPS) functionality, specified in clause 5.8.2.14.

-	Event exposure, including exposure of network information, i.e. the QoS monitoring information, as specified in clause 5.8.2.18, events as specified in clause 5.2.26.2 of TS 23.502 [3], exposure of data collected for analytics, as specified in clause 5.2.26.2 of TS 23.502 [3] and exposure of the TSC management information as specified in clause 5.8.5.14.

-	Exposure of the UE information, e.g UE IP address translation information as specified in clause 5.2.26.3 of TS 23.502 [3] and clause 4.15.10 of TS 23.502 [3] if Network address translation (i.e. NAT) functionality of the UE IP address is deployed within UPF.

-	Support PDU Set Handling as defined in clause 5.37.5.

# *** 3rd Change ***

### 7.2.29	UPF Services

The following NF service is specified for UPF.

Table 7.2.29-1: NF Services provided by UPF

| Service Name | Description | Reference in TS 23.502 [3] |
| --- | --- | --- |
| Nupf_EventExposure | This service exposes UPF related information to the consumer NFs. | 5.2.26.2 |
| Nupf_GetUEPrivateIPaddrAndIdentifiers | This service exposes UPF information related to NAT information and SUPI. | 5.2.26.3 |

*** End of Changes ***