3GPP TSG-WG SA2 Meeting #160ad	S2-2401280

January 22 – 26, 2024	(revision of S2-24xxxx)

Source:	Samsung

Title:	New Sol: S-NSSAI being subject to energy control

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys/Rel-19

Abstract: This document proposes a new Solution for KI#3.

1. Introduction

This document proposes new solution for KI#3.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-66

*** 1st change (all new text) ***

6.x	Solution #x: S-NSSAI being subject to energy control

6.x.1	Key Issue mapping

Editor's note:	This clause lists the key issue(s) addressed by this solution.

6.x.2	Functional Description

Editor's note:	This clause will describe the solution principles and assumptions for corresponding key issue(s). Sub-clause(s) may be added to capture details.

This solution assumes that an S-NSSAI can be subject to energy control.

6.x.3	Procedures

Editor's note:	This clause describes high-level procedures and information flows for the solution.

6.x.3.1	Registration procedure

Figure 6.x.3.1-1 illustrates registration procedure considering energy states.

1. UE sends Registration Request message including S-NSSAI as Requested NSSAI.

2. S-NSSAI is subject to energy control. AMF checks energy state of the S-NSSAI.

Editor’s note: it is FFS how the AMF knows the S-NSSAI is subject to energy control and how the AMF check energy states of the S-NSSAI, e.g. via interaction with other NF.

3. If the requested S-NSSAI can’t be provided e.g. due to the lack of energy, AMF determines to reject the requested S-NSSAI. Otherwise AMF allows the requested S-NSSAI.

6.x.3.2	Session establishment procedure

Figure 6.x.3.2-1 illustrates session establishment procedure considering energy states.

1. UE sends PDU session establishment request including S-NSSAI.

2. If the S-NSSAI is subject to energy control, AMF discovers SMF via NRF.

3. AMF sends NF discovery request to NRF.

4. NRF checks energy states per NF and selects SMF.

5. NRF sends NF discovery response. If SMF is selected at step 4, the selected SMF information is included in NF discovery response.

6. AMF sends PDU session establishment request to the discovered SMF.

7. If the S-NSSAI is subject to energy control, AMF sends Admission control request message to ECNF.

8. ECNF checks energy states per S-NSSAI and determine whether the S-NSSAI can be provided.

9. ECNF sends Admission control response message to inform the SMF of whether the S-NSSAI can be provided.

Editor’s note: It is FFS what information is included in Admission control response message.

10. If the S-NSSAI can be provided, the SMF proceeds the session establishment procedure and sends response message to the AMF.

11. AMF sends PDU session establishment response to the UE.

6.x.3.3	Session modification/release procedure

Figure 6.x.3.3-1 illustrates session modification/release procedure considering energy states.

1. PDU session has been established as described in Figure 6.x.3.2-1.

2. ECNF monitors energy consumption and detects that energy states is below the threshold.

3. ECNF sends notification message to the SMF.

Editor’s note: It is FFS what information is included in the notification message.

4. SMF determines whether to modify or release the PDU session associated with the S-NSSAI, and sends message to the AMF.

5. AMF sends PDU session modification command message to the UE.

6.x.4	Impacts on existing services, entities and interfaces

Editor's note:	This clause captures impacts on existing services, entities and interfaces.

- New functionality called ECNF(Energy Control Network Function) in this solution is proposed. ECNF monitors energy states per S-NSSAI in a PLMN. ECNF can be co-located in an existing NF or can be defined as a new NF.

- AMF checks energy state per S-NSSAI.

- SMF checks energy state per S-NSSAI via ECNF during PDU session establishment procedure. SMF triggers PDU session modification/release procedure if receiving notification from ECNF.

*** End of the changes ***