SA WG2 Meeting #S2-160	S2-23xxxxx

13 - 17 November, 2023, Chicago, USA

- Title:	New Key issue for 5GS enhancements with energy saving analytics

- Source:	KDDI

- Document for:	Approval

Agenda Item:	19.4

Abstract: It is proposed a new Key Issue for 5GS enhancements with energy saving analytics.

# 1 Discussion

According to the FS_EnergySys (SP-231192), the following WT need to be studied:

WT #3. Study 5GS enhancements (e.g., energy usage adjustment for NF from CN aspect, energy saving related decision making, NF selection leveraging NF energy states) for network energy saving including 5GC(NFs) and NG-RAN interactions, analytics, etc. Impacts on the UE are not ruled out e. g., for scenarios specified in TR 22.882 by SA1 EnergyServ.

Based on the WT, a new key issue for 5GS enhancements with energy saving analytics is proposed.

# 2. Proposal

It is proposed to add the following key issue to TR 23.700-66 " Study on Energy Efficiency and Energy Saving".

- *** Start of Change  (all new text)***

## 5.X	Key Issue #X: 5GS enhancements with energy saving analytics

### 5.X.1	Description

This key issue proposes to study 5GS enhancements with analytics information to assist network energy saving. If a NF consumer acquires analytics statistc/prediction information for the network energy saving, it can utilize an appropriate decision making (e,g, NF selection, policy changes).to reduce energy consumption in 5GS.

In this key issue, the following aspects will be studied:

-	How a NF consumer requests/subscribes the analytics for network energy saving.and what are the necessary inputs.

-	What kind of input data needs to be collected by a NWDAF to deliver the analytics for network energy saving and from which provided NFs.

-	What kind of output data of the analytics for network energy saving needs to provide to a NF consumer (e.g., statistics and/or prediction).

-	Whether new type of analytics for network energy saving is needed.

-	How a NF consumer can make use of the analytics for network energy saving for its actions (e.g., NF selection, policy changes).

- *** End of Changes ***