3GPP TSG-WG SA2 Meeting #161 	S2-2401935

Athens, Greece, February 26 – March 1, 2024	(revision of S2-2400595)

Source:	China Mobile

Title:	KI #1, New Sol: Alternative PDU Set QoS to support differentiated QoS handling for PDU Set

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: Propose a solution for alternative PDU Set QoS enhancement to support PDU Set level differentiated QoS handling related to KI #1 and KI #9.

1. Introduction/Discussion

This solution is for the key issue#1 of PDU Set based QoS handling enhancement and key issue #9 of enhancement for XR related network information exposure.

In the 5GS, each frame/video slice of XR services is transmitted via multiple PDUs i.e. PDU Sets, where the frame or slice should be handled in an integrated manner. The PDU sets have different importance and service characteristics, e.g. the I-frame, which is used as basic frame, is irreplaceable and has higher importance than the P-frame, or a discardable frame has low importance. However the existing PDU Set based QoS handling binds to a single QoS flow with only one set of PDU Set QoS parameters, which is unable to meet the requirements of differentiated QoS handling for various types of PDU Sets.

The alternative QoS mechanism can be enhanced to support flexible QoS adjustment at PDU Set level with combination of PDU Set based QoS parameters, effectively adapting to the differentiated data rate or PDU Set Importance (PSI) in XR services. The requirements of QoS guarantees varies by importance level of PDU Set, e.g. I-frame requires stringent PDU Set based QoS handling to guarantee the data transmission.

To achieve dynamic QoS adjustment at PDU Set level, AF could provide the 5G network with the alternative PDU Set QoS Parameter sets in a prioritized order or the PDU Set QoS references, as well as the importance level for different PDU Sets from the application server. Each importance level of PDU set can be mapped to one set of PDU Set QoS parameters with specific priority or reference. When 5GS acquires the alternative PDU Set QoS Parameter sets and the mapping to the importance level of the PDU Set, 5GS could choose an appropriate alternative PDU Set QoS reference regarding corresponding importance level of each PDU set, which can promote the network resource scheduling based on XR service characteristics and improve the resource utilization efficiency of RAN side.

Step 1: AF provides 5GS with alternative PDU Set QoS parameters and the corresponding reference for each service flow, as well as the mapping relationship between alternative PDU Set QoS parameter sets and PDU Set Importance. The mapping relationship indicates the appropriate PDU Set QoS parameters for different PDU Sets with differentiated importance.

Step 2：5GS binds service flow to QoS flow and creates Alternative PDU Set QoS Profile with one-to-one mapping between one PDU Set QoS parameter set and one PDU Set Importance level, according to alternative PDU Set QoS parameters and the mapping relationship provided by AF.

Step 3：For different PDU Sets comprising the QoS flow, RAN chooses the PDU Set QoS Parameter set that corresponds to the specific PDU Set Importance to ensure the PDU Set based handling and schedules the radio resources based on the differentiated QoS requirements, when receiving the PDU Set with PSI marking from UPF.

This contribution proposes a solution for the enhanced alternative PDU Set QoS mechanism between AF and the 5GS and also QoS policy enhancement to support PDU Set level differentiated QoS handling. The solution applies to KI #1 and KI #9.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-70.

# * * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issue # |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | <KI #1> | <KI#2> | <KI#3> | <KI#4> | <KI#5> | <KI#6> | <KI#7> | <KI#8> | <KI#9> |
| Solution #X: Alternative PDU Set QoS to support differentiated QoS handling | x |  |  |  |  |  |  |  | x |

# * * * *Second change(all new) * * * *

## 6.X	Solution #X: Alternative PDU Set QoS parameters to support differentiated QoS handling

6.X.1	Key Issue mapping

This solution is for KI #1 PDU Set based QoS handling enhancement for XR/media services, and KI #9 enhancement for XR related network information exposure.

6.X.2	Description

This solution enhanced alternative QoS mechanism to support PDU Set QoS parameters between AF and the 5GS. The existing Alternative QoS profile could be enhanced to support alternative PDU Set QoS parameter Set(s) including PSDB, PSER and PSIHI. To achieve fine-granularity PDU Set based QoS handling in 5GS, the alternative PDU Set QoS parameters can be flexibly adjusted by RAN.

In Rel-18 clause 5.37.5.1, when the SMF receives the PCC rule, the SMF performs binding of the PCC rule to one QoS Flow as described in clause 6.1.3.2.4 of TS 23.503. If the PCC rule contains one or more PDU Set QoS Parameters (PSER, PSDB and PSIHI), the SMF adds these PDU Set QoS parameters to the QoS Profile of the QoS Flow as described in clause 6.2.2.4 of TS 23.503 [45].

In this solution, AF can additionally provide the PCF with one or more Requested Alternative PDU Set QoS Parameter Sets (including the PSDB, PSER, PSIHI), the corresponding reference of each alternative PDU Set QoS Parameter Set in a prioritized order, and the mapping relationship between PDU Set QoS reference and importance level of PDU Set (i.e. PSI value). The PCF generates the PCC rule containing alternative PDU set QoS parameter Set(s) and the mapping relationship. The SMF performs binding of the PCC rule to one QoS Flow, with generating the the QoS Profile, and Alternative PDU Set QoS Profiles, as well as the mapping relationship indication of PDU Set QoS references and the importance level of PDU Sets. At least one kind of the PDU Set QoS parameters (PSDB, PSER and PSIHI) should be in the Alternative PDU Set QoS Profiles.

The Alternative PDU Set QoS mechanism also support PDU Set QoS Notification Control (PQNC), which is notified from NG-RAN to SMF/PCF with including Alternative PDU set QoS reference or Alternative PDU set QoS parameters. Considering XR services, the application server can timely adjust the codec/traffic rate based on QoS information exposed by network. In order to support timely network information exposure, the PQNC mechanism can be enhanced to support expose the QoS related information (e.g. QoS profile/Alternative QoS profile/Alternative PDU set QoS reference or parameters) via user plane.

6.X.3	Procedures

6.X.3.1	Procedures of alternative PDU Set QoS handling

The procedure of alternative PDU Set QoS handling is described in the Figure 6.X.3.1-1.

Figure 6.X.3.1-1: High level procedure of alternative PDU Set QoS handling

1.	The AF provides Alternative Service Requirements to the NEF using Nnef_AFsessionWithQoS_Create request, including one or more Requested Alternative PDU Set QoS Parameter sets in a prioritized order, or the PDU Set QoS reference, and the mapping relationship of PDU Set QoS Parameters Sets and the PDU Set importance (PSI) values (e.g. one-to-one or many-to-one mapping). The mapping relationship includes importance levels of PDU Sets, each of which is mapped to one or more corresponding PDU Set QoS references or PDU Set QoS Parameter sets.

2.	The NEF performs authorization of the AF request. If the request is authorized, the NEF provides the Alternative Service Requirements to PCF by invoking the Npcf_PolicyAuthorization_Create request. Otherwise, NEF responses to AF that the request is not authorized and the procedure stops.

3.	Based on the Alternative Service Requirements from AF, the PCF generates one or more alternative PDU Set QoS Parameter Sets for QoS flows. Besides, the mapping relationship of PDU Set QoS references and the importance level of PDU Sets is generated for the target XRM service, which may indicate one-to-one or many-to-one mapping relationship.

4.	The PCF sends the PCC rules to SMF together with the alternative PDU Set QoS parameters. SMF generates the Alternative PDU Set QoS Profiles and the mapping relationship of PDU Set QoS Parameter sets and PDU Set Importance values and then sends them to RAN.

- NG-RAN selects proper Alternative PDU Set QoS profile that corresponds to the specific PDU Set Importance based on the the mapping relationship, when receiving the PDU Set with PSI marking from UPF. If one PSI value is mapped to multiple PDU Set QoS references, NG-RAN will choose one suitable PDU Set QoS reference for that PSI based on radio status. When NG-RAN cannot fulfill Alternative PDU Set QoS requirement, the PDU Set QoS Notification Control indication is sent from NG-RAN. SMF shall also provide to the PCF the reference to the Alternative PDU set QoS parameter set corresponding to the Alternative PDU set QoS Profile referenced by NG-RAN.

6.X.3.2	Procedures of network information exposure via user plane

The procedure of network information exposure via user plane is described in the Figure 6.X.3.2-1.

Figure 6.X.3.2-1: Network information exposure via user plane

1.	The AF provides Alternative Service Requirements to the NEF using Nnef_AFsessionWithQoS_Create request, including one or more Requested Alternative QoS Parameter sets in a prioritized order which may additionally include PDU Set QoS parameters, or the QoS reference. The Alternative Service Requirements may also include the requested Alternative PDU Set QoS Parameter sets or the reference. And the AF may subscribe to receive QoS notifications from UPF via user plane when the QoS targets can no longer (or can again) be fulfilled.

2.	The NEF performs authorization of the AF request. If the request is authorized, the NEF provides the Alternative Service Requirements to PCF by invoking the Npcf_PolicyAuthorization_Create request. Otherwise, NEF responses to AF that the request is not authorized and the procedure stops.

3.	Based on the Alternative Service Requirements from AF, the PCF generates one or more alternative QoS Parameter Sets and/or alternative PDU Set QoS Parameter Sets for QoS flows and QoS notification control policy, which may configure the QoS information exposure via user plane.

4.	The PCF sends the PCC rules to SMF together with the alternative QoS requirements and QoS Notification Control parameter which may request to expose the QoS information via user plane. SMF generates the Alternative QoS Profiles and/or Alternative PDU Set QoS Profiles, and the corresponding Notification control parameter, and then sends them to the NG-RAN.

- Based on the received Alternative QoS Profiles, if Notification control is enabled in the NG-RAN, the NG-RAN will indicate the QoS related information, e.g. QoS parameters or PDU Set QoS parameter in QoS Profile, the reference of Alternative QoS profile or the reference of PDU Set QoS parameters that can no longer (or can again) be fulfilled, towards UPF via user plane if requested.

- UPF receives the QoS notification from NG-RAN, and send the information to AF/AS.

6.X.4	Impacts on services, entities and interfaces

AF

- Provides the QoS requirements which include additional one or more Requested Alternative PDU Set QoS Parameter Sets (e.g. PSDB, PSER, PSIHI), the corresponding reference to each PDU Set QoS Parameter Set, and the mapping relationship of PDU Set QoS Parameters Sets and the PDU Set importance (PSI) values of different PDU Sets (e.g. one-to-one or many-to-one mapping).

- Subscribes to receive QoS notifications via user plane when the QoS targets can no longer (or can again) be fulfilled.

PCF

- Generates a PCC, which derives one or more Alternative PDU Set QoS Parameter Sets for this PCC rule based on the PDU Set QoS Reference parameters or the Requested Alternative PDU Set QoS Parameter Sets in the Alternative Service Requirements. In addition, this PCC rule provides the mapping relationship of PDU Set QoS references and the PDU Set importance (PSI) values. For QoS notification control, The PCC rule also includes the QoS notification control parameter which may request network exposure via user plane.

- Send the PCC rules to SMF together with the Alternative PDU Set QoS requirements, the mapping relationship, and the QoS notification control parameter.

SMF

- Provides the Alternative PDU Set QoS Profiles and the mapping relationship of PDU Set QoS Parameter set and PDU Set Importance value to RAN.

- Enables the Notification control if requested, and sends the Notification control parameter to RAN.

- Sends the Notification control information to PCF.

RAN

- Chooses the proper PDU Set QoS profile for PDU Set received from UPF according to the PDU set information (i.e. PSI value) and radio status.

- PDU Set level differentiated QoS handling.

- Performs QoS monitoring and sends the notification of QoS information to the UPF via user plane.

UPF

- Enable the QoS related information report via user plane: detect QoS information from uplink data's header and report the network information to AF/AS.

# * * * * End of changes * * * *