3GPP TSG-TSA2 Meeting #161	S2-2401982

# Athens, Greece, February 20 – 24, 2024		(revision of S2-2401434)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0207 | rev | 3 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on the case AF interacts with HPLMN to influence HR-SBO traffic at VPLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Nokia], Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 6.7.3.2 on Network triggered EAS change in HR-SBO context has unclear procedure and missing details related to AF influenced traffic routing information provided to H-PLMN and corresponding policies in case of HR-SBO PDU session anchored by V-SMF. These details were also missing in clause 6.7.4. Moreover, for HR-SBO sessions, it is noted in 23.502 that External Group Identifier and/or External Subscriber Category cannot be used by AFs in VPLMN to influence traffic. Also, the details of how V-NEF identifies whether the session is HR-SBO are given in clauses 4.3.6.1 and 4.3.6.5, not in 4.3.6.4 of 23.502. The references to HR-SBO session identification in clause 6.7.4 are corrected accordingly. When the AF sends EAS rediscovery to H-SMF via H-NEF, the H-SMF cannot map APP ID into impacted field (i.e. FQDN/EAS IP) if it doesn’t have VPLMN EDI. Hence, AF should provide FQDN/EAS IP in this case. Same as EDI and traffic influence, External Group Identifier also cannot be used by AFs in VPLMN to provide EACI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the usage of External/Internal Group Identifier for HR-SBO session in the note given in Table 6.2.3.4-1. Updates procedure in 6.7.3.2-1, by requiring that H-SMF provides Traffic influence related policies to V-SMF. Updates to the text in clause 6.7.4 to consider the cases where AF interacting with VPLMN and HPLMN for HR-SBO session. Correcting the references on HR-SBO session identifications in clause 6.7.4. Clarify the AF should provide FQDN/EAS IP rather than APP ID when sending EAS rediscovery to H-SMF via H-NEF. Removing the usage of External/Internal Group Identifier for providing EACI in VPLMN. Some editorial changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification related to HR-SBO scenarios. AF triggered EAS rediscovery via HPLMN and AF providing EACI in VPLMN procedures cannot work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4.1, 6.7.3.2, 6.7.4, 6.5.2.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 4732 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 6.2.3.4.1	General

EAS Deployment Information management refers to the capability to create, update or remove EAS Deployment Information from AF and the distribution to the SMF. The NEF is in charge of the management of EAS Deployment Information which may be stored in UDR.

The EAS Deployment Information indicates how edge services are deployed in each Local part of the DN, the description of EAS Deployment Information is shown in Table 6.2.3.4-1.

Table 6.2.3.4-1 Description of EAS Deployment Information

| Parameters | Description |
| --- | --- |
| AF ID | Addressing information of Application Function responsible for the DNAI in the record. [Optional]. See NOTE 1. |
| DNN | DNN for the EAS Deployment Information. [optional] |
| S-NSSAI | S-NSSAI for the EAS Deployment Information. [optional] |
| External Group Identifier/Internal Group Identifier | Group ID for the EAS Deployment information. [optional]. See NOTE 2. |
| Application ID | Identifies the application for which the EAS Deployment Information corresponds to. [optional] |
| FQDN(s) | Supported FQDN(s) for application(s) deployed in the Local part of the DN. |
| DNAI(s) | DNAI(s) for the EAS Deployment information. [optional] |
| DNS Server Information | list of DNS server identifier (consisting of IP address and port) for each DNAI. [optional] |
| EAS IP address range Information | IP address(es) of the EASs in the Local part of the DN or the IP address ranges (IPv4 subnetwork(s) and/or IPv6 prefix(es) of the Local part of the DN where the EAS is deployed for each DNAI. [optional] |
| N6 traffic routing information | Information about how to forward edge traffic in the local part of DN corresponding to DNAI. [optional] |
| NOTE 1: When an AF ID is provided, all DNAI(s) correspond to the same EHE provider. NOTE 2: The AF may provide External Group Identifier, and NEF can map the External Group Identifier into Internal Group Identifier according to information received from UDM. For HR-SBO roaming scenario, External Group Identifier and Internal Group Identifier, cannot be used by AF in VPLMN. NOTE 3: AF ID can be used in case of AF(s) involving different EHE providers, and the source EHE is unaware of other/target EHE specific deployment details. |  |

The EAS Deployment Information management procedures are described in this clause, the procedures are independent of any PDU Session, including:

-	The procedure for EAS Deployment Information management from AF via the NEF.

-	The procedure for EAS Deployment Information management in the SMF.

-	The procedure for BaselineDNSPattern management in the EASDF.

NOTE:	In order to support EAS discovery when the Edge Hosting Environment is provided by a partner, an SLA is needed between current operator and the partner to provide e.g. the Address(es) and credentials for the DNS servers if the partner hosts the DNS server(s) for the related DNS resolution.

#### 6.7.3.2	Network triggered EAS change in HR-SBO context

Figure 6.7.3.2-1 shows the procedure of EAS re-discovery and edge relocation when HR-SBO is supported and allowed in the target serving PLMN.

Figure 6.7.3.2-1: Network triggered EAS rediscovery and edge relocation in HR-SBO context procedure

0.	The procedures described in clauses 6.7.2.6, 6.7.2.7 and 6.7.2.9 are performed: inter V-SMF inter-PLMN N2 handover or mobility registration (0a) in the HR-SBO case; or inter V-SMF intra-PLMN N2 handover or Xn handover or mobility registration update (0b) in the HR-SBO case.

1.	In the case of the procedures happened in step 0a, if the AF had subscribed to the corresponding event and a serving PLMN change occurred towards a PLMN where local traffic offload is possible for the PDU Session, the H-SMF notifies the AF, indicating the new serving PLMN ID as well as HPLMN DNN and S-NSSAI for HR-SBO session, and H-SMF may include capability of supporting EAS IP replacement in 5GC with PLMN ID of VPLMN. This may take place as soon as the H-SMF has received an indication of Handover Complete (see step 13 of Figure 6.7.2.6-1).

NOTE:	Via this mechanism, the AF is aware of the PLMN to contact to issue traffic influence requests for HR-SBO sessions, if available, with HPLMN DNN and S-NSSAI information. The AF is assumed to check whether it has an SLA with the new serving PLMN. If the AF has no SLA with the new serving VPLMN, the AF interacts with H-NEF to issue traffic influence requests.

This may trigger the AF triggered edge relocation / EAS rediscovery as defined in step 1b of Figure 6.2.3.3-1 and in step 4a of Figure 6.3.3.1.1-1.

2a.	For AF triggered EAS re-discovery and edge relocation via interacting with HPLMN, the AF may indicate the EAS rediscovery for the impacted applications, which are identified by FQDN(s)/EAS IP address(es), to the H-SMF via the H-PCF and optional H-NEF, H-UDR using the AF influence on traffic routing procedure as described in clause 4.3.6 of TS 23.502 [3]. The AF may also provide EAS IP replacement information and target DNAI together with an indication of the PLMN associated with this target DNAI, i.e. the serving PLMN ID in case VPLMN (V-SMF) supports EAS IP replacement capability and has already notified this capability information to AF (directly or via H-SMF/H-NEF).

NOTE X:	If the AF triggers EAS rediscovery via interacting with HPLMN, FQDN(s)/EAS IP address(es) will be performed instead of Application Identifier(s).

NOTE: 	V-SMF providing the capability of supporting EAS IP replacement to AF can happen either:

- during V-SMF insertion/change phase where the capability information is provided to H-SMF and then to AF via H-NEF as noted in step 1; or

- as part of PDU session update respond procedure that is initiated by H-SMF in step 3a.

2b.	For AF triggered EAS re-discovery and edge relocation via interacting with serving VPLMN, the AF may indicate the EAS rediscovery for the impacted applications via the V-NEF using the procedure described in clause 4.3.6 of TS 23.502 [3].

This may trigger step 2 of Figure 6.2.3.3-1 where the SMF that initiates PDU Session modification is the V-SMF that initiates Nsmf_PDUSession_Update request with the requested PCO.

The AF may also provide EAS IP replacement information and target DNAI to the VPLMN (i.e. V-SMF). In this case, steps 3-4 are skipped.

3a.	(For AF triggered EAS re-discovery and edge relocation via interacting with HPLMN case): The AF traffic influence request information is sent to H-SMF via PCC rule. This may trigger step 2 of Figure 6.2.3.3-1 where the SMF that initiates the PDU Session modification is the H-SMF. The H-SMF issues a Nsmf_PDUSession_Update. The Nsmf_PDUSession_Update request includes policies due to AF provided traffic influence information, which may also contain EAS IP replacement information and target DNAI provided by AF in step 2. If the V-SMF cannot serve the target DNAI, it invokes a Nsmf_PDUSession_SMContextStatusNotify service operation to send the target DNAI to AMF, and the AMF selects a target V-SMF based on the target DNAI as described in clause 4.23.5.4 of TS 23.502 [3] by replacing I-SMF with V-SMF. The target V-SMF retrieves SM context from the source V-SMF using Nsmf_PDUSession_Context Request/Response, containing Authorization Result for HR-SBO, EAS IP replacement information and target DNAI in the Request. The target V-SMF may select a new V-EASDF as described from steps 2 to 12 in Figure 6.7.2.6-1.

3b.	(For AF triggered EAS re-discovery and edge relocation via interacting with VPLMN case): The V-SMF initiates Nsmf_PDUSession_Update request with the EAS rediscovery indication and the impact field to the H-SMF, and the H-SMF initiates Nsmf_PDUSession_Update Response towards the (target) V-SMF including the PCO information to be sent to the UE as described in step 2 of Figure 6.2.3.3-1. In intra-PLMN V-SMF change, the target V-SMF may use the source and target DNAI to determine the Impact field to be sent to the UE. In inter-PLMN mobility, the target V-SMF provides EAS rediscovery information without an Impact field.

4.	The V-SMF may initiate PDU Session Modification command including the PCO to the UE.

The PCO may include EAS rediscovery indication (optional) and the impact field (optional).

5	The V-SMF may configure the V-UPF (UL CL and L-PSA) with EAS IP replacement information.

### 6.7.4	AF request on PDU Sessions supporting HR-SBO

For HR-SBO PDU Sessions, the AF may interact with VPLMN or HPLMN in order to send an AF request to influence traffic routing. The AF is assumed to check whether it has an SLA with the serving PLMN. If the AF has no SLA with the serving VPLMN, the AF interacts with HPLMN (H-NEF or H-PCF) to issue traffic influence requests.

In case the AF has an SLA with the serving VPLMN,

the AF in VPLMN may send to V-NEF an AF request to influence traffic routing (e.g. for the purpose of subscription to UP path management events on HR-SBO Sessions in VPLMN). The AF request for the HR-SBO PDU Session (which can be differentiated from the non-roaming and LBO PDU Session by the V-NEF as described in clauses 4.3.6.1 and 4.3.6.5 of TS 23.502 [3].) from the AF is stored as Application Data (Data Subset = AF traffic influence request information) in the UDR of VPLMN as described in clause 4.3.6 of TS 23.502 [3]. To obtain the AF traffic influence request information, the V-SMF managing the PDU Session supporting HR-SBO subscribes to the NEF in VPLMN for notification of Application Data modification as specified in clause 4.3.6 of TS 23.502 [3].

In case the AF has no SLA with the serving VPLMN, the AF in HPLMN may send to H-NEF (or H-PCF directly) an AF request to influence traffic routing (e.g. for the purpose of subscription to UP path management events on HR-SBO Sessions in VPLMN) for HR-SBO session in VPLMN. The AF request for the HR-SBO PDU Session (which can be differentiated from the non-roaming and LBO PDU Session by the H-NEF as described in clauses 4.3.6.3 1 and 4.3.6.4 5 of TS 23.502 [3].) from the AF is stored as Application Data (Data Subset = AF traffic influence request information) in the UDR of HPLMN as described in clause 4.3.6 of TS 23.502 [3]. When there is an update on the information to influence traffic for the HR-SBO session, the UDR sends a notification to H-PCF of the HR-SBO session. In response to receiving the traffic influence request, the H-PCF may generate or update one or more PCC rules based on the traffic influence request where the PCC rule may contain information about the VPLMN DNAI(s) towards which the traffic routing should apply, and H-PCF sends an update notification to H-SMF.

The H-SMF may send to the V-SMF the received Application Function influence on traffic routing Enforcement Control  (i.e., received in PCC rules from the H-PCF). The H-SMF may also determine Application Function influence on traffic routing Enforcement Control information related to the VPLMN and provides the VPLMN-related Application Function influence on traffic routing Enforcement Control information to the V-SMF as described in step 3a of Figure 6.7.3.2-1.

##### 6.5.2.6.2	ECS Address Configuration Information Provision from AF via NEF in VPLMN

The AF provides non-PDU Session specific ECS Address Configuration Information via NEF in VPLMN to 5GC is defined in this clause.

Figure 6.5.2.6.2-1 ECS Address Configuration Information provisioning to UDR via NEF in VPLMN

1.	The AF invokes the Nnef_ECSAddress_Create /Update/Delete service operation to provide ECS Address Configuration Information to the NEF in VPLMN.

2.	NEF checks whether the AF is authorized to perform the request based on the operator policies.

NOTE:	External Group Identifier and Internal Group Identifier cannot be used by AF in VPLMN for HR-SBO case.

3.	The NEF invokes the Nudr_DM_Create/Update/Delete to the UDR in VPLMN if it is authorized.

4.	The UDR stores/updates/removes the corresponding information (and responds a Nudr_DM_Create/Update/Delete Response to the NEF).

5.	The NEF sends Nnef_ECSAddress_Create/Update/Delete Response to the AF.

******************* END of Changes *******************