SA WG2 Meeting #161	S2-2402017

26 February - 1 March, 2024, Athens, Greece

Source:	vivo

Title:	New Sol: Association of registrations

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes a new Solution for subscription and registration.

1	Discussion

This paper proposes a new solution for KI#1.1 subscription and KI#1.2 registration of the FS_MASSS SID (SP-231802).

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V0.1.0.

* * * * Start of Changes * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of DualSteer Solutions to Key Issues

|  | Key Issues for DualSteer |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | <Key Issue #1.1> | <Key Issue #1.2> |  |  |
| #X | x | x |  |  |

Table 6.0-2: Mapping of ATSSS_Ph4 Solutions to Key Issues

|  | Key Issues for ATSSS_Ph4 |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | <Key Issue #2.1> | <Key Issue #2.2> |  |  |
| #Y |  |  |  |  |

* * * * Second Change (all new text) * * * *

### 6.1.X	Solution #X: Association of registrations

6.1.X.1	Description

This solution addresses the Key Issue#1.1 "Subscription aspects to support DualSteer" and Key Issue#1.2 "Registration and mobility management for DualSteer".

6.1.X.2	Procedures

6.1.X.2.1	Information flow

Figure 6.1.X.2.1-1: Association of registrations

1-5.	The DualSteer device initiates 5GS registration procedure with SUPI#1 towards serving network#1. The AMF#1 in serving network#1 registers itself into UDM or UDM+HSS for SUPI#1.

6.	The DualSteer device decides to perform dual registration with SUPI#2, it initiates 5GS initial registration procedure or EPS Attach procedure towards serving network#2.

7.	Authentication and NAS security setup procedures with SUPI#2 are performed to activate integrity protection and NAS ciphering.

8.	The AMF#2 in serving network#2 invokes Nudm_UECM_Registration Request service operation to register AMF ID#2 into UDM for SUPI#2, or the MME in serving network#2 sends Update_Location Request to register MME ID into UDM+HSS for SUPI#2.

9.	The UDM or UDM+HSS associates the two registration procedures as dual-registration from a DualSteer device according to subscription data, and marks "dual-registration activated" for the DualSteer device.

Editor’s note: how to use the mark of "dual-registration activated" for policy or session management is FFS.

10.	The AMF#2 or MME of the serving network#2 responds to the DualSteer device.

6.1.X.2.2	Subscription data

In order to support dual registration of a DualSteer device, a new IE of "DualSteer Authorization" is included in the Access and Mobility Subscription data to authorize whether a SUPI is allowed to be used for DualSteer. If authorized, a new subscription data type, i.e., DualSteer management data, is used to accommodate the management information for a DualSteer device. Following table 6.1.X.2.2-1 shows the subscription enhancement for DualSteer.

Table 6.1.X.2.2-1: UE Subscription data types

| Subscription data type | Field | Description |
| --- | --- | --- |
| Access and Mobility Subscription data (data needed for UE | … | … |
| Registration and Mobility Management) | DualSteer Authorization | Indicates whether the SUPI is allowed to be used for DualSteer. If allowed, it indicates the SUPI Pair ID. |
|  | … | … |
| DualSteer management data | SUPI Pair ID | Indicates the user identifier of a DualSteer device with two SUPIs/subscriptions. |
|  | DualSteer status | Indicates the status of the DualSteer device, including dual-registered, single-registered. |

Table 6.1.X.2.2-2: UE Subscription data types keys

| Subscription Data Types | Data Key | Data Sub Key |
| --- | --- | --- |
| DualSteer management data | SUPI Pair ID | - |

6.1.X.3	Impacts on services, entities and interfaces

Editor's note:	This clause captures impacts on existing 3GPP services, entities and interfaces.

UDM:

-	Subscription data is enhanced to support DualSteer.

* * * * End of Changes * * * *