3GPP TSG-WG SA2 Meeting #161	S2-2402021

26th Feb – 1st March 2024, Athens, Greece	(revision of S2-XXXXXX)

Source:	Lenovo

Title:	KI #1, Sol #2: Update to provide the FEC ratio and/or indication to NG-RAN

Document for:	Approval

Agenda Item: 	19.3

Work Item / Release:	FS_XRM Ph2 /Rel-19

Abstract of the contribution: This contribution proposes an update to address the unclear parts in the solution 2 regarding FEC ratio and activation at NG-RAN.

1	Discussion

In SA2#160E, solution 2 is agreed for discarding of redundant PDUs (FEC). Apart from the controversial issue regarding whether and how to detect the FEC PDUs, there are still two unclear parts in the current solution that need to be addressed.

The 1st is the activation of FEC operation at NG-RAN for discarding. As stated in the step 1 and the impacts part, the AF will indicate that the UE application allows discarding of FEC PDU(s) that are redundant. This indication is forwarded to the SMF via PCF by including in the PCC rules. It needs to be stated out clearly that either SMF will provide this indication to NG-RAN for allowing FEC operation, or when the FEC ratio is provided, it implies that the indication is provided (FEC operation is allowed).

The 2nd is the corresponding FEC ratio for discarding. No FEC ratio is mentioned in the solution to be provided to the NG-RAN, either from SMF, or from UPF. Without this FEC ratio, the NG-RAN will not be able to know how many PDU(s) of one PDU set can be discarded.

Based on the discussion above, the changes are made as below.

2	Proposal

It is proposed to adopt the following changes into TR 23.700-70.

* * * * 1st Change * * * *

6.7	Solution #2: Discarding of redundant PDUs (FEC) and reporting

6.2.1	Key Issue mapping

This solution is for Key Issue #1, which addresses the following aspects:

- 	How the AF can indicate to the 5GC that the UE can handle when FEC PDU(s) is missing when the UE has received sufficient of corresponding PDU(s) so that the FEC PDUs will not be used.

6.2.2	Description

In Rel-18 we support PSER (PDU Set Error Rate) and PSIHI (PDU Set Integrated Handling Information) to allow NG-RAN to identify if all PDU in the PDU Set must be delivered. If some PDUs will not be delivered, then NG-RAN knows the rate of successfully delivered PDUs in a PDU Set that is needed for the receiver to be able to use the PDU Set. This allows for example NG-RAN to discard some PDU(s) from the PDU Set during congestion. In general, the receiving XR application will detect missing PDU’s and as result of that request reduced bandwidth (quality / frame rate etc.). Some RTP Payload Formats (like RFC8627 [32]) supports easy separation between the media itself and the FEC data allowing the receiver to not process the FEC stream if there are no transmission errors. If the UE application only uses the payload (without the FEC data) to adjust the bandwidth, then the AF should be able to configure the 5GC to allow that FEC PDU’s is allowed to be discarded before congestion in the network. This optimization will not impact the UE application. NG-RAN should also be able to report the amount of discarded data allowing the operator to deduct the amount of user data that was delivered to the UE.

6.2.3	Procedures

The Figure 6.2.3-1 shows the procedure for how the AF indicate that NG-RAN is allowed to discard FEC PDU(s):

Figure 6.2.3-1: Procedures for AF to indicate that it is allowed to discard FEC PDU(s).

The steps of Figure 6.2.3-1 are described as follows:

- The AF informs the PCF that the UE application allows discarding of FEC PDU(s) that are redundant using the Nnef_AFsessionWithQoS_Create/Update request message, together with the FEC ratio.

- PCF initiates the PDU Session modification procedure as defined in clause 4.3.3.2 of TS 23.502 [3]. The PCF generates appropriate PCC rules influenced on the information from AF. The PCF sends the PCC rules to SMF. SMF generates the QoS profiles and N4 rules based on the PCC rules from PCF. SMF sends the N4 rules to UPF and sends the QoS profiles to the NG-RAN node via AMF, including the FEC ratio, and/or the optional indication that the UE application allows discarding of FEC PDU(s) that are redundant.

- NG-RAN deliver DL PDUs and track if UE has received them to determines if discarding of future (unneeded) FEC PDU(s) is possible, if the AF has indicated that it is allowed. This assumes that NG-RAN uses a delivery mode that allows NG-RAN to know if the UE has received it correctly. The NG-RAN can use the FEC ratio to determine how many FEC PDU(s) of one PDU set can be discarded.

NOTE: It is up to NG-RAN implementation to select appropriate FEC PDU for discarding.

Editor’s Note: If is FFS if additional PDU Set QoS parameter needs to be defined that the UPF provides to inform NG-RAN on FEC PDU’s.

Editor’s Note: The NG-RAN delivery mode required to determine successful delivery of PDUs, is FFS.

- NG-RAN reports the amount of user data that was discarded to the AMF.

- AMF reports the amount of user data that was discarded to the SMF.

Editor’s note: how the removal of FEC data affects subsequent hops in DL/UL and consequently the end user experience is FFS.

Editor’s note: how in this envisioned solution the e2e FEC relates to FEC introduced by the radio interfaces’ channel coding and HARQ is FFS.

Editor's note: SA2 will reach out to SA4 to get feedback on this solution.

Editor's note: SA2 will reach out to RAN2 to get feedback on this solution.

6.2.4	Impacts on services, entities and interfaces

AF impacts:

- Provisioning that the UE application handles discarding of redundant FEC PDU(s), i.e. the 5GS is allowed to discard FEC PDU(s) regardless of congestion, and the FEC ratio.

PCF impacts:

- PDU Session procedure with support of this indication, the FEC ratio and corresponding PCC rule.

NG-RAN impacts:

- NG RAN may perform discarding of PDU(s) before congestion using the FEC ratio. In addition, NG-RAN to introduce the signalling of the amount of discarded data similar to the Secondary RAT data usage report between RAN and the AMF.

AMF impacts:

- Support reporting of the amount of discarded data from NG-RAN to SMF.

SMF impacts:

- PDU Session procedure with support of this indication sent to the NG-RAN optionally, and the FEC ratio. Support for taking into account the amount of discarded data when reporting data usage from UPF to CHF.

UPF impacts:

- The UPF impact is FFS.

NEF impact:

## Support for AF to provision this indication that dropping of redundant PDUs is allowed, and the FEC ratio.

* * * * End of Changes * * * *