SA WG2 Meeting #161	S2-2402108

February 26th – March 1st, 2024; Athens

Source:	OPPO

Title:	TR 23.700-45: New Solution for KI#1 – UE capability enhanced to support UE moving between CAG cell of 5G Femto and CSG cell.

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract of the contribution: This contribution proposes the solution related to the UE capability enhanced to support UE moving between CAG cell of 5G Femto and CSG cell.

1. Introduction/Discussion

This solution addresses the KI#1 on whether and how control signalling procedures are enhanced to support the UE moving between CAG cell of 5G Femto and CSG cell. More specifically, the mobility scenarios in this solution is the UE moving from the 5G Femto CAG cell to the CSG cell. The UE in this solution is on the connected mode. In order to support the UE moving from the CAG cell to the CSG cell, the UE should have both the CAG and CSG access capability. When the UE performing the registration procedure to the network, the AMF should include the allowed CAG and CSG list in the registration accept message to the UE. Then based on the allowed CAG and CSG list, the UE can select the suitable target cell. UE report the selected target CSG cell to the source gNB. Then the source gNB can determine the target eNB to perform the interworking procedure.

- Text Proposal

It is proposed to agree the following changes to TR 23.700-45.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> |
| #X | X |  |
| #2 |  |  |

# * * * * Second change * * * *

6.X	Solution #X: UE capability enhanced to support UE moving between CAG cell of 5G Femto and CSG cell

6.X.1	Description

The mobility scenarios in this solution is the UE moving from the 5G Femto CAG cell to the CSG cell. The UE in this solution is on the connected mode. In order to support the UE moving from the 5G Femto CAG cell to the CSG cell, the UE should have both the CAG and CSG access capability. When the UE performing the registration procedure to the network, the AMF should include the allowed CAG and CSG list in the registration accept message to the UE.

In order to let AMF can send the allowed CAG and CSG list to the UE. A new CAG and CSG related subscription data is introduced and stored in the UDM. When the UE perform the registration procedure, AMF will retrieve the CAG and CSG related subscription data from the UDM and include the allowed CAG and CSG list in the registration accept message. The CAG and CSG related subscription data stored in the UDM is defined as following:

| Subscription data type | Field | Description |
| --- | --- | --- |
| Access and Mobility Subscription data (data needed for UE Registration and Mobility Management) | CAG and CSG information | The CAG and CSG information includes Allowed CAG and CSG list and optionally an indication whether the UE is only allowed to access 5GS via CAG cells and EPS via CSG cell. |

Then when the interworking is needed, the UE can select the suitable target cell based on the allowed CAG and CSG list. UE reports the selected target CSG cell to the gNB based on the allowed CAG and CSG list. Then the source gNB can determine the target eNB to perform the interworking procedure.

6.X.2	Procedures

The procedure will be built based on the procedures defined by the 3GPP TS 23.502 clause 4.11.1.2.1 for 5GS to EPS handover using N26 interface with no flow modification.

6.X.3	Impacts on services, entities and interfaces

UE: Enhanced to support both CAG and CSG access capability. Determine the suitable target CSG cell based on the allowed CAG and CSG list.

AMF: Enhanced to determine the allowed CAG and CSG list based on the UE CAG and CSG access capability.

UDM: Enhanced to store the CAG and CSG related subscription data.

# * * * * End of changes * * * *