3GPP TSG-WG SA2 Meeting #161	S2-2402228

# Athens, Feb 26 – Mar 1, 2024                 	                              (revision of S2-2401097r06)

# Source:	China Mobile

# Title:	5GS enhancement on QoS monitoring enhancement

# Document for:	Approval

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 5GS enhancement on QoS monitoring enhancement

Acronym: TEI19_QME

Unique identifier :

Potential target Release: Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No | X | X |  |  | X |
| Don’t know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

The QoS monitoring is quite useful for particular URLLC services, mandate QoS guarantees from the network. The current specifications have defined the details of QoS monitoring, including the monitored QoS parameters (e.g., UL packet delay, DL packet delay, round trip packet delay, congestion, Data Rate, Packet Delay Variation, round trip packet delay when UL and DL are on different QoS flows), and how to measure and report the QoS parameters.

But several enhancements are still needed.

- The QoS monitoring capability negotiation is missing. The measurement of QoS parameters and reports of the measurement result need to be supported by RAN and UPF, and we may not assume that all the RAN(s) and UPF(s) support the QoS monitoring. So the SMF need to know whether the RAN support Qos monitoring.

- In the existing per Qos Flow Qos monitoring mechanism, the RAN reports the RAN part delay according to the QoS Monitoring Reporting Frequency IE provided by SMF. The value of this IE is set from 1 second to 1800 seconds. The current report frequency of RAN part delay may not meet the SA1 requirement specified in TS 22.261 clause 6.23.2 (e.g. per data packet). It proposes to introduce per Qos monitoring packet reporting. When the RAN receives the DL Qos monitoring packet with time stamp, it reports immediately the current available RAN part delay to UPF at next UL packet or dummy UL packet.

This work item proposes to introduce QoS monitoring capability negotiation mechanism and per packet Qos monitoring

4	Objective

The objective of this WID is to study the enhancement of QoS monitoring:

- Introduce QoS monitoring capability negotiation mechanism between RAN and 5GC (e.g. UPF, SMF).

- Enhance QoS monitoring mechanism to activate/deactivate per packet QoS monitoring reporting.

This work will require 0.5 TU to discuss and agree the corresponding CR(s).

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | General statement about QoS monitoring capability negotiation; Per packet Qos monitoring report is introduced. | SA#104 (June 2024) |  |
| TS 23.502 | PDU Session Establishment procedure is updated to support QoS monitoring capability negotiation. | SA#104 (June 2024) |  |
| TS 23.503 | A new indicator is introduced in PCC rule information to support per packet Qos monitoring report. | SA#104 (June 2024) |  |

6	Work item Rapporteur(s)

Aihua Li, China Mobile <liaihua@chinamobile.com>

7	Work item leadership

SA2

8	Aspects that involve other WGs

None identified.

9	Supporting Individual Members

Supporting IM name

China Mobile

ZTE

Tencent

Tencent Cloud

CATT

Orange

EDF

China Telecom

vivo

SK Telecom