3GPP SA WG2 Meeting #161	S2-2402333

26 February – 01 March, 2024, Athens, Greece

Source:	NEC

Title:	KI#3: Updates to Solution #14 UE energy usage control

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: The contribution addresses the Editor’s Notes in Solution #14 UE energy usage control of KI#3.

1. Introduction

This contribution proposes updates to Solution #14: UE energy usage control of Key Issue #3: 5GS enhancements for network energy saving and efficiency.

2. Proposal

It is proposed to agree the following solution to 3GPP TR 23.700-66 v0.3.0.

* * * Start of 1st change * * * *

## 6.14	Solution #14: UE energy usage control

### 6.14.1	Key Issue mapping

The solution applies to Key Issue #3 5GS enhancements for network energy saving and efficiency.

### 6.14.2	Functional Description

Assumptions:

-	It is assumed that each UE will have an Energy Credit Profile as a subscription information in the UDR which contains the allowed Energy Credit units for the UE. The Energy Credit units availability is maintained in the UDR and may be reset periodically based on the UE subscription plan.

-	It is assumed that the UE's Energy Credit units balance is retrieved by the PCF from the UDR. The PCF checks the UE's Energy Credit balance before PDU Session is established and updates the balance at the end of the PDU Session or at certain periodic interval if the PDU Session takes long by deducting the used Energy Credit units used for the PDU Session. The remaining Energy Credit balance for the UE is updated in the UDR by the PCF.

-	It is assumed that if there is no Energy Credit units left in the UE's Energy Credit balance, the UE is either rejected PDU Session establishment or the PDU Session is established but charged at higher service rate based on the operator's policy.

-	It is assumed that when the UE is rejected PDU Session establishment for no Energy Credit left, the UE is provided a new PDU Session rejection cause, e.g. 'no energy credit' and potentially a back-off timer. For legacy UEs an existing rejection cause is used in order to maintain backward compatibility.

### 6.14.3	Procedures

Figure 6.14.3: UE energy usage control

1.	PDU Session establishment start according to clause 4.3.2.2.1 of TS 23.502 [3].

2.	SM Policy establishment or modification according to clauses 4.16.4 / 4.16.5.1 of TS 23.502 [3].

3.	The PCF may decide to retrieve the remaining Energy Credit for the UE from the UE's Energy Credit Profile within the subscription information in the UDR with the Nudr_DM_Query message in which the PCF may indicate the UE ID and the Energy Credit balance update required parameter.

Editor's note:	It is FFS which network entity maintains the Energy Credit balance for the UE.

4.	If the UE has an Energy Credit profile, the UDR provides the UE's Energy Credit balance to the PCF.

5.	The UDR updates the remaining Energy Credit for the UE in the PCF with Nudr_DM_Query response (UE ID, Energy Credit balance) message.

6.	The PCF stores the UE's Energy Credit balance. If there is no remaining Energy Credit, based on the operator policy the UE may be rejected (step 6a) or serviced with a higher service rate (step 7).

6a. PDU Session establishment rejected for lack of Energy Credit.

7.	Continue with the PDU Session establishment procedure according to clause 4.3.2.2.1 of TS 23.502 [3].

8.	At the end of the PDU Session and at certain periodic interval if the PDU Session takes long, the CHF is aware of how much Energy Credit is used by the UE and the CHF updates the PCF with the Energy Credit used for the PDU Session with the Nchf_SpendingLimitControl_Notify (UE ID, Energy Credit used) message.

9.	PCF updates the available Energy Credit for the UE in the UDR by removing the Energy Credit used for the PDU Session.

10.	PDU Session is released according to TS 23.502 [3].

### 6.14.4	Impacts on existing services, entities and interfaces

UDR:

-	UE's Energy Credit profile as part of the UE subscription information is maintained in the UDR.

PCF:

-	UE's Energy Credit retrieval from the UDR and update at te end of the PDU Session.

UE:

-	New PDU Session reject cause (e.g. 'no energy credit') handling. No impact to legacy UEs as an existing reject cause can be used.

* * * End of 1st change * * * *